CATFLOW

A Physically Based and

Distributed Hydrological Model for

Continuous Simulation

of

Catchment Water- and Solute Dynamics

by Thomas Maurer, Erwin Zehe

User Guide

and

Program Documentation

(VERSION CATSTAT)

Institute for Water Resources Planning, Hydraulics and Rural Engineering (IWK)

University of Karlsruhe (TH)

Copyright (C) Thomas Maurer & Erwin Zehe

October 2007

- modified September 2010 by Jan Wienhöfer

This program is free software; you can redistribute it and/or

modify it under the terms of the GNU General Public License

as published by the Free Software Foundation; either version 2

of the License, or (at your option) any later version.

This program is distributed in the hope that it will be useful,

but WITHOUT ANY WARRANTY; without even the implied warranty of

MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.  See the

GNU General Public License for more details.

You should have received a copy of the GNU General Public License

along with this program; if not, write to the Free Software

Foundation, Inc., 51 Franklin Street, Fifth Floor, Boston, MA  02110-1301, USA.

Institute for Hydrology and Water Resources Planning (IHW)

University of Karlsruhe, Kaiserstr. 12, 76128 Karlsruhe, Germany

Tel.: +49.721.608.43814 Fax: +49.721.608.45651

e-mail: erwin.zehe@kit.edu

1	Introduction	6

2	Overview	9

2.1	State variables	9

2.2	Geometry	9

2.3	Processes	9

2.4	Boundary conditions	10

3	Input structure	14

3.1	General	14

3.2	Main input file	14

3.3	Project control file	14

3.4	Global files	18

3.4.1	Specification of soil types	18

3.4.2	Specification of time series	21

3.4.2.1	Time series of boundary conditions	22

3.4.2.2	Time series of sink/source data	23

3.4.2.3	Time series of precipitation	24

3.4.2.4	Time series of climatic data	25

3.4.2.4.1	Type 1	25

3.4.2.4.2	Type 2	26

3.4.2.5	Time series of land-use patterns	26

3.4.2.5.1	Land-use patterns: Assigning land-use units to land-use ID	27

3.4.2.6	Solute input mass	27

3.4.2.7	Boundary conditions of the drainage network	27

3.4.3	Assigning land-use IDs to land-use parameter files	28

3.4.3.1	Specification of land use parameters	28

3.4.4	Assigning sectors to a wind direction identifier	30

3.4.5	Drainage network geometry and -discretisation	30

3.4.5.1	No interaction between subsurface and channel water	32

3.4.5.2	Interaction between subsurface and channel water	32

3.4.5.2.1	Initial state of the drainage network	33

3.4.5.2.2	Assigning time series of input fluxes to boundary nodes of the drainage channel	33

3.5	Hillslope specific files	33

3.5.1	Hillslope geometry and -discretisation	33

3.5.2	Assigning soil types	36

3.5.3	Assigning factors to the Ksat field of the hillslope	37

3.5.4	Assigning factors to the theta field of the hillslope	37

3.5.5	Assigning macropore parameters	37

3.5.6	Specification of control volumes	38

3.5.7	Assigning initial conditions	38

3.5.7.1	Initial state for soil water dynamics	38

3.5.7.1.1	Blockwise assignment of either absolute suction head or degree of saturation	38

3.5.7.1.2	Assigning individual absolute values at each point	39

3.5.7.1.3	Uniform potential in entire domain	40

3.5.7.2	Initial solute concentration	40

3.5.7.2.1	Blockwise assignment of residual total concentrations	40

3.5.7.2.2	Assigning individual concentration values at each point	40

3.5.8	Specification of printout times	41

3.5.9	Assigning surface attributes	41

3.5.10	Assigning boundary conditions and sinks/sources	42

4	Preparing input files	45

4.1	Prolog	45

4.2	General	47

4.3	Data organisation	47

4.4	Required information	48

4.4.1	Spatial information	48

4.4.2	Temporal information	49

4.4.3	Constant information	49

4.4.4	Domain decomposition in hillslopes and drainage network	49

4.4.4.1	T_4HIM	50

4.4.4.2	ARC/INFO	51

4.4.4.3	MATLAB	52

4.4.5	Ordered graph of drainage network	52

4.4.6	Discretisation of hillslopes	53

4.5	Assigning attributs/indices	54

4.5.1	Surface (2D)	54

4.5.2	Depth (3D)	55

4.6	Soil types	55

4.7	Wind directions	55

4.8	Land use parameters	55

4.9	Time series	55

4.10	Initial conditions	56

4.11	Control files	56

5	Simulation output	57

5.1	Log-file	57

5.2	Tables of soil-hydraulic functions	57

5.3	Water balance	58

5.4	State variables	59

5.5	Overland flow	59

5.6	Evapo-Transpiration	60

5.7	Channel flow	61

5.8	Co-ordinate files	61

6	Visualisation	62

6.1	General	62

6.2	Longitudinal sections using Matlab	62

6.2.1	2-d cross sections of state variables without water fluxes	62

6.2.2	2-d cross sections of state variables and water fluxes	63

6.3	Areal maps using ARC/VIEW	64

6.4	Time series	64

6.4.1	Hydrographs for specified channel nodes	64

6.4.2	Surface runoff for individual hillslopes	65

7	Program structure	67

8	INCLUDE-files and COMMON-blocks	71

8.1	Dim.inc	71

8.2	Zeit.inc	74

8.3	Soil.inc	75

8.4	Hgfest.inc	76

8.5	Hgvari.inc	79

8.6	Hgbdry.inc	83

8.7	Pfest.inc	86

8.8	Pvari.inc	87

8.9	pbdry.inc	89

8.10	Bach.inc	89

8.11	Erosion.inc	92

9	Alphabetical list of all variables in COMMON-blocks	93

10	Moving to other platforms	110

11	References	111

# Introduction

CATFLOW is a physically based, distributed model for simulating the dynamics of water and solutes in small rural catchments on the event and season time scale. The considered control volume consists of the upper soil layer including the vegetation canopy, with the atmosphere and possibly groundwater imposing boundary conditions onto the system. Keeping in mind the aim of modelling the transport processes of dissolved and suspended substances including contaminants, both at the surface (erosion and wash-off) and underground (transport towards groundwater and channel), a process-oriented approach is chosen to model the inner structure of catchments as thoroughly as possible. This permits the examination of the interaction between subsystems, and finally enables the derivation of simplifications.

The model is based on the idea of subdividing the 3D-landscape in patches of 2D-hillslopes of variable width (assuming water flowing normal to the topographical contours) connected to a detailed (partly ephemeral) drainage network. Fig. 2.1 gives an idea of the approach for the Weiherbach cachtment. Though there have been developments based on this or similar ideas in the past (Jayawardena & White, 1977; Jayawardena & White, 1979; Beven et al., 1987; Calver & Wood, 1995; Vertessy et al., 1993) a new effort has been undertaken to improve concept and structure of this type of model.

As has been shown (Mesa & Mifflin, 1986) and is well understood today (Singh, 1995a), with increasing size of a catchment the shape of a hydrograph is more and more influenced by the properties of the drainage network rather then by overland flow; the latter then mainly has to provide realistic estimates of event based runoff coefficients. However, increasing size is a relative measure. If we trace down the drainage network into every single swale of the landscape (which of course requires a detailed spatial resolution of elevation) above considerations are valid for even very small catchments. What we get is a partial decoupling of runoff production and runoff propagation allowing us to put to work all the well-established methods developed in open channel flow and flood routing. CATFLOW implements schemes based on the simplified St. Venant equations. Thus we have achieved somewhat a reduction of the dimension of the problem.

As has been pointed out e.g. by (Kuehnel et al., 1990a; Kuehnel et al., 1990b; Wang & Dooge, 1993) runoff coefficients are determined by proper evaluation of infiltration and evapo-transpiration, both at times controlled by either soil or atmospheric conditions. CATFLOW spends considerable efforts in a more precise description of these processes.

Special features of the system include:

Problem-oriented domain decomposition and time-step control: The subdivision of the catchment area is carried out based on the topography. Beginning with the detailed drainage network and delineated subcatchments, hillslope areas are identified and then idealised as vertical longitudinal sections of variable width along the steepest gradient of the topography. This decomposition supports a far-reaching decoupling of runoff production at the hillslope and runoff propagation in the drainage network, where flow can be described in a more hydrodynamicly substantiated sense. Then, the longitudinal cross-sections as well as the drainage network are discretised taking into account the zones expected to be hydrologically most active.

Based on geomorphologically motivated considerations an efficient and simple parameterisation scheme has been developed in order to describe the complex geometry of the detailed drainage network.

Process-based modelling: The modelling scale is close to the process scale. This property of physically based models permits the consideration of the interaction between all relevant processes based on field observations. These are: evapo-transpiration, interception, infiltration and two-dimensional water movement in the soil matrix and preferential pathways, transport, ad- and desorption and degradation of non conservative solutes in the saturated/unsaturated zone as well as surface runoff, subsurface flow and channel flow (including runoff from sealed surfaces) under the influence of temporally and spatially variable climatic boundary conditions and land use. Last not least the possible interaction between channel flow and the saturated zone, i.e. groundwater influence or effluence has to be taken into account.

Flexible spatial/temporal resolution of boundary conditions and parameters: A prerequisite for the examination of interactions between those various processes and properties is the possibility of handling all data at an arbitrary spatial/temporal resolution. This requirement is also important with respect to a problem-oriented time-step control and has been met by implementing a suitable data structure.

Relational data management: To ensure the user-friendliness of the modelling system, the large amounts of temporally and spatially variable input and output data of state variables and both system and environment parameters are organised in a relational data structure, i.e. as an indicated system of tables and flexible input formats.

Furthermore, special emphasis has been put on the following features:

- Hillslope geometry is described by use of orthogonal, curvilinear coordinates as described e.g. by Kiefer et al. (1990)to (a) flexible model the irregular boundaries and (b) to more easily conduct numerical solution of Richards equation for matrix flow on a mathematical rectangular grid.

- Potential based solution of Richards equation using Picard iteration and Taylorseries expansion of the moisture content with respect to the Potential as described by (Hornung & Messing, 1984; Celia et al., 1990) thus allowing for modelling of saturated conditions. Solute transport is described by means of a two dimensional transport equation of the Convection-Dispersion-type (Briggar & Nielsen, 1967; Roth et al., 1996), including first order decay and linear, instantaneous sorption. The transport equation is numerically solved by means of the particle tracking method (Zehe, 1999; Thompson, 1987; Kinzelbach & Uffink, 1991).

- A simplified method for transport of water and solutes in preferential pathways. If the relative saturation S at the soil surface of a macroporous soil layer exceeds a threshold value, the saturated hydraulic conductivity is increased by a macroporousity factor. An effective pore water velocity is computed by averaging the corresponding values in the soil matrix and in the macropores in a grid cell. The required model parameters, i.e. the macroporousity factor fmac and the macroporous cross section in a grid cell amac may easily be derived from field and laboratory data (Zehe, 1999).

- Soil properties i.e. their hydraulic functions and transport parameter are provided in tables, thus allowing for easy inclusion of different descriptions. Currently the Van Genuchten (1980) and Mualem (1976) model is used mostly.

- Channel flow within the drainage network is calculated based on the kinematic wave or the diffusion wave simplifications of the St. Venant equation. Moreover interaction between channel flow and shallow groundwater, i.e. ground water influence or effluence may be accounted for. In this case, the water table in the channel determines a time dependent Dirichlet (pressure) boundary condition at the boundary of the corresponding hill slopes.

Each hillslope has its individual time step control, thus reducing overall CPU time consumption.

Thus, CATFLOW is a flexible tool for detailed modelling of catchments water- and solute dynamics at the resolution desired by the user.

A thorough description of CATFLOWs' theory and applications is given in detail by (Maurer, 1997a; Zehe, 1999). In these studies, the modelling system and extensively collected data have been used to parameterise, calibrate, and verify a model of the research catchment “Weiherbach” near Karlsruhe. In that context, an important aim was the synthesis and integration of numerous experimental results and the detailed research of individual processes performed by other authors.

# Overview

CATFLOW is a physically based, distributed model for the dynamics of water and transport of solutes in the upper soil layer of small rural catchments (including channel network and vegetation cover) on the event and season time scale. The model is based on the idea of subdividing the 3D-landscape in patches of 2D-hillslopes of variable width (assuming water flowing normal to the topographical contours) connected to a detailed (partly ephemeral) drainage network. Fig. 2.1 shows an example. The approach relies on the assumption of deterministic behaviour, i.e. starting from some initial condition the change of state variables under changing boundary conditions can be calculated.

## State variables

The physically based, distributed model of high spatio-temporal resolution calculates the following state variables:

- soil water potential,

- surface water depth,

- interception storage

- residual concentration of various solutes in the soil .

## Geometry

The geometry of the system under consideration (upper soil layer) is described by the following data:

- digital elevation model,

- channel profiles and elevations,

- soil hydraulic functions,

- macroporosity,

- surface roughness,

- channel roughness,

- parameters determining interaction between channel flow and the saturated zone,

- plant parameters.

## Processes

The model simulates various processes of water transport, as

- fluxes in saturated/unsaturated soil matrix,

- return flow,

- surface runoff,

- subsurface storm flow

- water movement and solute transport in the soil matrix

- water movement and solute transport in preferential pathways,

- linear, instantaneous equilibrium adsorption

- first order decay of non conservative solutes

- ground water influence or effluence

- transpiration,

- evaporation,

- interception,

- channel flow,

- interactions near stream,

- runoff from sealed surfaces.

## Boundary conditions

Systems state is changed by these processes under the impact of (mostly climatic) boundary conditions, as

- precipitation,

- net radiation,

- temperature,

- humidity,

- wind speed and wind direction,

- ground water levels.

Due to its high spatio-temporal resolution and thus data needs, CATFLOW requires a considerable number of files to be prepared and specified in project control files. Files are organized in a hierarchical way (Fig. 2.2), reflecting the relational data structure used throughout the program.

All geometric data are geo-referenced to a global coordinate system as well as all time series data are time-referenced to the actual date which can be input in a format dd.mm.yyyy hh:mm:ss.hs (CHAR*22). Care is taken to synchronise all temporal data with reference to the date.

The program and its input are based on SI-units (meters, seconds, kilo grams and ampere) if not stated otherwise. However, often conversion factors to SI-units can be assigned to input files, thus allowing the use of more familiar units.

Figure 2.1: Topographic analysis of the “Weiherbach-catchment” up to the gauge Menzingen (3.5 km2)

Figure 2.2: Input files hierarchy in program CATFLOW

Part I

USER GUIDE

# Input structure

## General

All filenames in project control files are specified relative to the location within the path, where the main input file (CATFLOW.IN) resides (and from where the program has to be executed). However, filename length is restricted to CHAR*30 (including path).

## Main input file

The main input file (CATFLOW.IN) holds nothing but a list of control filenames (format a30) of individual projects, which for simulation are successively executed until a blank line is encountered. Starting from column 30, a global surface roughness has to be specified which is used as a default value for computation of overland flow. This value may be overwritten in the files characterising crop properties (section “Assigning land-use IDs to land-use parameter files”). An example:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|demo/catfl.in                    5. 	% ifile0(1), kstall(1) (Strickler coefficient)

|demo/catfl_w.in               10. 	% ifile0(2), kstall(1) (Strickler coefficient)

| .

## Project control file

The project control file holds top level information of an individual project, such as simulation times and timestep control parameters. File names of input and output data files as well as lower level control files are specified. An example:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|27.06.1994 18:00:00.00		% dstrs begin of simulation (a22)

|27.06.1994 24:00:00.00		 % dstre end of simulation (a22)

| 0.0 				% t_off time offset in [s]

|pic 				% meth method (always 'pic') (a3)

| 360. 				% dtbach max. timestep for drainage network [s]

| 1.e-7 				% qtol threshold for initiating of drainage network computation [cbm/s]

| 86400. 				% dt_max max. timestep in [s]

| 0.01 				% dt_min min. timestep in [s]

| 10.00 				% dt initial timestep in [s]

| 0.020 				% d_Th_opt timestep control parameter [-]

| 0.020 				% d_Phi_opt timestep control parameter [-]

| 4 				% n_gr treshold number of Picard iteration step

| 10 				% it_max max. number of Picard iteration steps

| 1.e-3 				% piceps desired convergence tolerance Picard [-]

| 5.e-6 				% cgeps desired convergence tolerance CG [m]

| 15. 				% rlongi longitude of time zone (i.e. CET)

| 8.75 				% longi longitude of catchment

| 49.13 				% lati latitude of catchment

| 2   1				% istact number of solute types; i_sorp switch for adsorption isotherm (see section 3.4.1.1; needed when istact  > 0)

| -80				% iseed seed for portable random number generator

|simact				 % q_meth switch (simact or noiact)

| 17				% io_act number of specified output files (should be fixed)

| 011 100 000 111 000 00	 % io_log(i) switches that control amount of output to output files

|demo/out/log.out 		% ofile(1) log-file

|demo/out/vg_tab.out 		% ofile(2) tables of soil-hydraulic functions

|demo/out/bilanz.out 		% ofile(3) water balance

|demo/out/theta.out 		% ofile(4) soil moisture

|demo/out/psi.out 		% ofile(5) suction head

|demo/out/psi.fin 			% ofile(6) final state

|demo/out/fl_xsi.out 		% ofile(7) xsi-flux

|demo/out/fl_eta.out		 % ofile(8) eta-flux

|demo/out/senken.out 		% ofile(9) sinks

|demo/out/qoben.out 		% ofile(10) overland flow

|demo/out/etinz.out 		% ofile(11) Evapo-Transpiration

|demo/out/gang.out 		% ofile(12) drainage network hydrographs

|demo/out/v_eta.out		% ofile(13) generalised pore water velocity (for particle tracking)

|demo/out/v_xsi.out		% ofile(14) generalised pore water velocity (for particle tracking)

|demo/out/c.out 			% ofile(15) residual solute concentration in the soil

|demo/out/hko.out		% ofile(16) z – coordinate of the computational grid

|demo/out/sko.out			% ofile(17) x-coordinate of the computational grid

| 7 	% iin_act number of specified global input files

|demo/in/vgsoil.art 		% ifile(1) specification of soil types

|demo/in/rbsenk.def 		% ifile(2) specification of time series

|demo/in/lusenam.def 		% ifile(3) assigning land-use ID to land-use parameter file

|demo/in/windrich.def 		% ifile(4) sectors of wind direction

|demo/in/vorflut.def 		% ifile(5) drainage network and sealed surface geometry

|demo/in/vorflut.ini		% ifile(6) initial state of the drainage network

|demo/in/vorflut.rb		% ifile(7) assigning boundary conditions to channel nodes

| 10 	% iacnh number of hillslopes, each hillslope requires 8 – 9 input files

|demo/in/hang.geo 		% hgfile hillslope geometry and -discretisation

|demo/in/hang.bod		 % stfile assigning soil types

|demo/in/ks_feld.dat		% ifile assigning factors to Ksat-fields of the hillslope (pref.flow)

|demo/in/th_feld.dat		% ifile assigning factors to theta-fields of the hillslope (leave as all 1’s)

|demo/in/hang.mak		% mafile assigning macropore parameters

|demo/in/hang.cv 			% cvfile specification of control volumes

|demo/in/hang.ini 		% inifil assigning initial conditions for soil water dynamics

|demo/in/conc.ini			% infile assigning initial concentration for solutes optional

|demo/in/hang.prt 		% prtfil specification of printout times

|demo/in/hang.pob 		% pobfil assigning surface attributes

|demo/in/hang.rb 			% rbsfil assigning boundary conditions

Please note that the sequence of variables and filenames in the project control files is fixed and that the specified pathnames are read as character strings (CH*30).

The switch for model output files 011 100 000 111 000 00 consists of 17 numbers (0 or 1) for up to 17 outputfiles. If a one (switch on) is specified step, each time step will be written to the corresponding output files, if zero (switch of) is specified, output is only dumped at the printout times specified (section “Specification of printout times”). Hydrographs for certain channel nodes will only be dumped into output file # 12, if the switch is on 1.

Time step is controlled by several partly competing conditions involving a desired optimal maximum change in water content respectively potential within the entire domain and an iteration threshold.

Control parameters are:

| dtbach | tbach | maximum time step for drainage network computations in [s] |
| --- | --- | --- |
| dt_max | tmax | maximum time step for hillslope computations in [s] |
| dt_min | t min | minimum time step for hillslope in [s] |
| d_Th_opt | opt | optimal change in water content among all computational nodes within the entire domain [-] |
| d_Phi_opt | opt | optimal change in hydraulic head among all computational nodes within the entire domain [-] |
| it_max | nit,max | the maximum allowed number of iterations of the Picard-Iteration (Celia et al. 1990) |
| n_gr | n it,treshold | a treshold number of iterations of the Picard-Iteration (desired) |

After each simulation time step the actual changes within the entire domain are determined and a relative measure is calculated:

|  | act | max. actual change in hydraulic head among all computational nodes |
| --- | --- | --- |
|  | act | max. actual change in water content among all computational nodes |

Further, considering nit,akt , the actual number of iterations of the Picard-Iteration, the following rules apply to determine a new time step:

If r < 0.5 or nit,akt >nit,max, repeat calculation step

If r  0.5, calculation is continued.

1. For 0.5  r < 1 the time step is decreased:

In addition, if nit,akt > nit,threshold, a further reduction by a factor

(1-nit,akt - nit,grenz )/nit,max applies.

2. If r  1 the time step will be reduced for nit,act > nit,threshold

and increased otherwise:

Finding optimal values for the parameters d_Th_opt and d_Phi_opt is a trial and error process, values of 0.02 to 0.03 have been found to be suitable.

The parameter piceps is a convergence criterion to stop the Picard iteration. Small values lead to a highly accurate mass balance but increase computational effort. Finding optimal values for an individual problem is also a question of trial and error. The above specified value of 10-3, i.e. 0.001 yielded a good compromise for simulations carried out for the Weiherbach catchment.

The parameter istact (INT) specifies for how many different types of solutes are accounted for during the simulation. If istact = 0, only catchment water dynamics is calculated and the module for solute dynamics is switched of. If istact > 0 it is necessary to specify a seed value (REAL*8) for the generation of random numbers (with a implemented random number generator) for the particle tracking.

## Global files

### Specification of soil types

Here all parameters to describe the hydraulic and transport properties of all soils are specified. At initialisation, CATFLOW produces look-up-tables from this data. An example:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| 3 %anisotrop. So albedo soil resistance erosion

|loess (VanGenuchten:1980)

| 1 800 1. 1. 5.e-5 0.5 0.34 0.11 20. 0.70 0.050 eross1 eross2 eross3

|2.1e-6 0.44 0.06 0.40 2.06 -3. 5. 1300 0.8

| 0. 0.

| 0. 0.

| 2.5 E-6 1.5 E-6

|fine river sand (Vauclin et al., 1979)

| 2 401 1. 1. 1.e-5 0.5 0.24 0.14 30. 0.60 0.030 eross1 eross2 eross3

|9.7222e-5 0.3 40000. 2.9 2.99e6 5. -2.1 1. 1300 0.8

| 0. 0.

| 0. 0.

| 2.5 E-6 1.5 E-6

|tabulated data

|99 200 1. 1. 1.e-4 0.5 0.24 0.14 30. 0.60 0.030 eross1 eross2 eross3

|soil.tab

| 0. 0.

| 0. 0.

| 2.5 1.5 E-6

Line 1 holds the number of soils. Subsequently, each soil is counted beginning from ityp=1 and requires a block of 5 lines of data (lines 2-6, 7-11, 12-16 above):

a |boden(ityp)

b |imod(ityp) iactab(ityp) anisot(1,ityp) anisot(2,ityp) snull(ityp)

+ satkni(ityp) balmax(ityp) balmin(ityp)

+ f_rs(ityp) wp_rs(ityp) zd_max(ityp)

+ eross(1,ityp) eross(2,ityp) eross(3,ityp)

c |(bodpar(ipbm,ityp), ipbm=1,npbm), sattgr

e |(abbau(ipbm,ityp), ist=1,istact)

f |(k_sorp(ipbm,ityp), ist=1,istact)

g |(d:koef(ipbm,ityp), ist=1,istact)

| boden(ityp) | CHAR*30 | descriptive string |
| --- | --- | --- |
| imod(ityp) | INT | soil model identifier |
| iactab(ityp) | INT | length of table to be generated |
| anisot(1,ityp) | REAL | factor on conductivity for the larger main direction of anisotropy |
| anisot(2,ityp) | REAL | factor on conductivity for the smaller main direction of anisotropy |
| snull(ityp) | REAL | specific storage coefficient |
| satkni(ityp) | REAL | albedo-soil moisture relation parameter |
| balmax(ityp) | REAL | albedo-soil moisture relation parameter |
| balmin(ityp) | REAL | albedo-soil moisture relation parameter |
| f_rs(ityp) | REAL | factor for soil moisture weighting of soil resistance |
| wp_rs(ityp) | REAL | inflexion point of soil moisture weighting of soil resistance |
| zd_max(ityp) | REAL | maximum thickness of top dry layer (soil resistance, experimentally determined) |
| eross(i,ityp) | REAL | 3 soil dependent erosion parameters (dummy values) |
| bodpar(ipbm,ityp) | REAL | parameters of soil model, number npbm+ depends on imod(ityp) |
| abbau(ist, ityp) | REAL | life times characterising first order decay of the specified solutes in the soil |
| k_sorp(ist, ityp) | REAL | adsorption coefficients of the specified solutes in the soil |
| d_koef(ist, ityp) | REAL | dispersion coefficients of specified solutes in the soil |

#### Solute transport parameters

Three different parameters (decay, adsorption, dispersion) have to be specified for each solute. On runtime the solutes will be referred by an integer number according to their position in the parameter list. Hence, the sequence of the parameters has to be consistent with the sequence of the solute input masses specified in the corresponding file (section: solute input masses). For example, for three solutes, three decay parameters abbau(ist, ityp) have to specified in the line below the soil parameters (line “e” in the example above), of which the first one is for the solute 1, etc.

Adsorption coefficients are specified in relation to the type of adsorption isotherme given by i_sorp in the project control file. Possible values for i_sorp are 1 (linear adsorption), 2 (Freundlich isotherm), and 3 (Langmuir isotherm). In case of linear adsorption, one coefficient per solute type has to be given (up to 3 values in total); otherwise, two values per solute type have to be specified (up to 6 in total, first two values are for first solute, etc).

#### Soil hydraulic Models

Currently, three models are implemented:

imod=1: Van Genuchten-Mualem model:

This model (VanGenuchten 1980, Mualem 1976, Carsel & Parrish 1988, Rawls & Brakensiek 1989) requires 5 parameters plus 2 parameters specifying the range of the table (example: line 4 above).

| bodpar(1,ityp) | ks | saturated hydraulic conductivity [m/s] |
| --- | --- | --- |
| bodpar(2,ityp) | s | saturated soil moisture [-] |
| bodpar(3,ityp) | r | residual soil moisture [-] |
| bodpar(4,ityp) |  | reciprocal air entry point [1/m] |
| bodpar(5,ityp) | n | quantity characterising the pore size distribution [-] |
| bodpar(6,ityp) | log10 (min) | logarithm of minimum suction head (min in [m]) |
| bodpar(7,ityp) | log10 (max) | logarithm of maximum suction head (max in [m]) |
| bodpar(8,ityp) |  | bulk density [kgm-3] |
| sattgr |  | Threshold: relative saturation for initiating macropore flow |

imod=2: Tang & Skaggs model:

This model (Tang & Skaggs 1977, Vauclin et al. 1979) requires 6 parameters plus 2 parameters specifying the range of the table (example: line 7 above).

| bodpar(1,ityp) | ks | saturated hydraulic conductivity [m/s] |
| --- | --- | --- |
| bodpar(2,ityp) | s | saturated soil moisture [-] |
| bodpar(3,ityp) |  | parameter [-] |
| bodpar(4,ityp) |  | parameter [-] |
| bodpar(5,ityp) | A | parameter [-] |
| bodpar(6,ityp) | B | parameter [-] |
| bodpar(7,ityp) | log10 (min) | logarithm of minimum suction head (min in [m]) |
| bodpar(8,ityp) | log10 (max) | logarithm of maximum suction head (max in [m]) |
| bodpar(9,ityp) |  | bulk density [kgm-3] |
| sattgr |  | Threshold: relative saturation for initiating macropore flow |

imod=99: tabulated data:

In this case only a filename (CHAR*30) of a file holding tabulated data of soil hydraulic functions has to be specified (example: line 10 above). Also, the only considered input from the preceding line (example: line 9 above) is imod(ityp)+ (though the following 4 inputs of that line have to be specified as dummy values). Data needed are read from file tabfil(ityp)+ which looks as follows (example):

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|tabulated soil 1.000 1.000 0.1000E-03

| 0.656065E-01 0.100000E+06 0.513940E-21 -0.156982E-07

| 0.656867E-01 0.950508E+05 0.589429E-21 -0.167519E-07

| 0.657681E-01 0.903466E+05 0.676006E-21 -0.178765E-07

| 0.658507E-01 0.858752E+05 0.775299E-21 -0.190765E-07

| 0.659344E-01 0.816251E+05 0.889178E-21 -0.203570E-07

| 0.660194E-01 0.775853E+05 0.101978E-20 -0.217236E-07

| 0.661056E-01 0.737455E+05 0.116957E-20 -0.231818E-07

The header line holds the following four entries:

| boden(ityp) | CHAR*30 | descriptive string |
| --- | --- | --- |
| anisot(1,ityp) | REAL | factor on conductivity for the larger main direction of anisotropy |
| anisot(2,ityp) | REAL | factor on conductivity for the smaller main direction of anisotropy |
| snull(ityp) | REAL | specific storage coefficient |

and subsequent lines specify four columns holding

|  | soil moisture [-] |
| --- | --- |
|  | suction head [m] |
| k | hydraulic conductivity [m/s] |
| C | water capacity (negative) [1/m] |

starting from dry conditions (1=r).

### Specification of time series

In this file the filenames of files holding time series data are specified. An example:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|BOUNDARY CONDITIONS 	%mathematical boundary conditions

|2				% number of files to be read

|demo/in/bc1.dat

|demo/in/bc2.dat

|

|SINK/SOURCE

|0				% number of files to be read

|

|PRECIPITATION

|2				% number of files to be read

|demo/in/rain1.dat

|demo/in/rain2.dat

|

|CLIMATE

|1				% number of files to be read

|demo/in/ climate.dat

|

|LANDUSE

|demo/in/luse.dat

|

|MASSES

|1				% number of files to be read

|demo/in/massin.dat

|

|V    Drainage network input fluxes

|1				% number of files to be read

|demo/in/gang1.dat

There are up to 7 input-blocks, which are identified by key-letters in the first column:

| B | boundary conditions |
| --- | --- |
| S | sink/source |
| P | precipitation |
| C | climate |
| L | land use |
| M | input Masses for particle tracking in case of Neumann boundary conditions |
| V | input into the drainage system e.g. via a spring |

Usually, either B and S is specified (imposing mathematical boundary conditions directly) or P, C and L is specified (called atmospheric boundary condition); however, each combination is correct, too. If P and/or C are/is used, specification of L is mandatory. If the number of solutes accounted for is nonzero (istact > 0) and in case of a Neumann (flux) boundary condition for solute input, M specifies files holding the input masses of the solutes. In this case also B has to be specified, because a flux of solute into a control volume is always related to a water flux. If interaction of channel flow and subsurface water is of interest, V specifies input times series of discharge, which has to be related to boundary nodes of the drainage network.

If B, M and S, P or C or V is specified, the line following a key-letter holds the number of lines to read for the block (ianz (INT)). The following lines contain a list of filenames (CHAR*30) of time series files. On runtime this files will be referred by an integer number according to their position in the list. This integer number is used in the assignment file for boundary conditions and/or surface attributes. The line following key-letter L holds the name of a file specifying time series of land-use patterns. How do the time series files look like?

#### Time series of boundary conditions

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|31.12.1990 18:30:00.00 18000.

|# reference time [5h] -> [s]

|# time	irbtyp 	[rbpar1 	[rbpar2]] 	c_inj (for i=1, istact)

| 0. 	0

| 2. 	1	9.3e-6			0.165	0.165

| 10. 	1	2.1e-5			0.165	0.165

| 22. 	11	0.5	2.1e-5		0.165	0.165

| . . . .

The first line holds two variables, namely a string (CHAR*22) which has to start from the first column in a format dd.mm.yyyy hh:mm:ss.hs and a conversion factor for the time unit used subsequently into seconds. There might be comment lines indicated by #.

After this header information, it follows a list of lines, where each line represents one point in time. Each line starts with a time offset from the reference date (in time units e.g. h) and is followed by a boundary type specification (irbtyp) which again determines the number and type of required parameters (rbpar = 0, 1 or 2) and will be applied from the specified time onwards. If solute transport is accounted for and a Neumann boundary condition has to be specified the solute input concentrations have to be given additionally. E.g. if istact = 2, i.e. transport of two different solute is simulated (for irbtyp =1flux or irbtyp = 11 unequality) two input concentration values have two be specified. On runtime the solutes will be referred by an integer number according to their position in the list. Hence, the sequence of the input concentrations has to be consistent with the sequence of the solute parameters specified in the soil specification file (section: specification of soil types) and the specification of input masses (section: solute input mass). Please note that the total solute input mass resulting from the specified input concentrations c_inj, the water fluxes q or qpot and the duration of the input, has to be equal to the values listed in the M files. Here is a complete list of all possible boundary conditions:

| irbtyp | rbpar1 | rbpar2 | c_inj for number of specified solutes (optional) | description |
| --- | --- | --- | --- | --- |
| 0 | - | - | - | zero flux |
| 1 | q [m/s] | - | c [kgm-3] | flux per unit boundary |
| 3 | - | - | - | free drainage (mixed) |
| 5 | k/d [1/s] | ref [m] | - | leakage (mixed) |
| -1 | [m] | - | - | suction head |
| -2 | [m] | - | - | potential (refers to input geometry) |
| -4 | - | - | - | keep potential fixed |
| +/-10 | - | - | - | seepage |
| +/-11 | pot [m] | q_ pot [m/s] | c [kgm-3] | unequality (e.g. rainfall) |

If possible, boundary conditions are interpolated between discrete times. This only is possible if either irbtyp=0 or 1 is followed by irbtyp=0 or 1 or if irbtyp=-1 or -2 is followed by irbtyp=-1 or -2.

#### Time series of sink/source data

This file format is very similar to that used for boundary conditions. An example:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|31.12.1990 18:30:00.00 18000.

|# reference time [5h] -> [s]

|# time	isktyp	[skpar1		[skpar2]]

| 0.	0

| 2.	1	9.3e-6

| 10.	1	2.1e-5

| 22.	11	0.5		2.1e-5

| . . . .

The first line holds two variables, namely a string (CHAR*22) which has to start from the first column in a format dd.mm.yyyy hh:mm:ss.hs and a conversion factor for the time unit used subsequently (to seconds).There might be comment lines indicated by #.

After this header information, it follows a list of lines, where each line represents one point in time. Each line starts with a time offset from the reference date(in [time units]) and is followed by a sink/source type specification (isktyp) which again determines the number and type of required parameters (0, 1 or 2) and will be applied from the specified time onwards. Here is a complete list of all possible sink/source types:

| isktyp | skpar1 | skpar2 | description |
| --- | --- | --- | --- |
| 0 | - | - | zero flux |
| 1 | Q [1/s] | - | flux per unit area |
| 2 | Q [sqm/s] | - | flux |
| -1 | [m] | - | suction head |
| -2 | [m] | - | potential (refers to input geometry) |
| -4 | - | - | keep potential fixed |
| +/-10 | - | - | seepage |
| +/-11 | pot [m] | qpot [m/s] | unequality |

If qpot > 0 (sink)  cannot become greater than pot

If qpot < 0 (source)  cannot become smaller than pot

If possible, sink/source data are interpolated between discrete times. This only is possible if either

irbtyp = 0, 1 or 2 is followed by irbtyp = 0, 1 or 2, or if irbtyp = -1 or -2 is followed by irbtyp = -1 or -2.

#### Time series of precipitation

The first line holds three variables, namely a string (CHAR*22) which has to start from the first column in a format dd.mm.yyyy hh:mm:ss.hs, a conversion factor for the time unit used subsequently (to seconds) and a conversion factor for the flux unit used subsequently (to [m/s]). There might be comment lines indicated by #.

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|31.12.1990 18:30:00.00 360. 2.7778e-6

|# reference time conversion factors [6min] -> [s] [mm/6min] -> [m/s]

|# time 		nied

| 0.		 0.

| 10. 		3.1

| 11. 		0.5

| . .

After this header information, a list of lines follows, where each line represents one point in time. Each line starts with a time offset from the reference date (in [time units]) and is followed by a rainfall intensity. The intensities are interpolated between discrete times.

#### Time series of climatic data

There are two possible different file types.

##### Type 1

Type 1 specifies climatic data for use with the evapo-transpiration model.

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|1 1	#ktyp (file type); rBilart (specifies wheter measured values of net radiation rBilm (2) are used or rBil is computed (1) )

|31.12.1990 18:30:00.00 86400. 8. -6. 0.7 0.28e-3 1.5 0.1

|# 1 reference date and time

|# 2 time conversion factor, e.g. [d] -> [s]

|# 3 reference height of wind speed measurement [m]

|# 4-6 sw0 sw1 sw2: factors for rBil= sw0 + sw1*swrBil + sw2*swrBil^2,

|# 	relation of short wave radiation balance and net radiation, to be calibrated

|# 7-8 trueb truebf: factors to account for atmospheric dimming under consideration of water vapour

|# 	pressure, to be calibrated using clear sky radiation data

|#

|# meaning of -999: no data

|#

|# time 	glob.rad. 	net rad. 	temp 	rel. hum. 	v_wind		d_wind*

|# 	[W/m*m] 	[W/m*m] 	[degree] [%] (0..100) 	[m/s] 		[degree]

|# 	rGlo 		rBil 	temp 	relhum 	uref 		udir

| . . . . . . . 								clockwise from North

The first line holds the file type (1) and a switch (1/2) for the computation of net radiation. The second line holds eight variables, namely a string (CHAR*22) which has to start from the first column in a format dd.mm.yyyy hh:mm:ss.hs, a conversion factor for the time unit used subsequently (to seconds), the reference height of wind speed measurements (in [m]), three calibration factors for radiation balance, and two calibration factors for atmospheric dimming. There might be comment lines indicated by #.

Following this header information, a list of lines, where each line represents one point in time has to be specified. Each line starts with a time offset from the reference date (in [time units]) and is followed by global radiation, net radiation, temperature, relative humidity, reference windspeed and wind direction. All these values are interpolated between discrete times, except wind direction which requires an additional file assigning sectors to a wind direction.

##### Type 2

Type 2 specifies evapo-transpiration data directly (measured or calculated externally).

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|2 1

|31.12.1990 18:30:00.00 86400. 1.157407408e-8

|# reference time [d] -> [sec] [mm/d] -> [m/s]

|# time evapo transp z_root [m]

| 0. 0.124 0.008 0.180

| 1. 0.066 0.004 0.189

| 2. 0.031 0.003 0.198

| 3. 0.027 0.003 0.207

| 4. 0.055 0.007 0.216

| 5. 0.092 0.013 0.225

| 6. 0.138 0.022 0.234

| . . . .

| . . . .

The first line holds the file type (2). The second line holds three variables, namely a string (CHAR*22) which has to start from the first column in a format dd.mm.yyyy hh:mm:ss.hs, a conversion factor for the time unit used subsequently (to seconds) and a conversion factor for the flux unit used subsequently (to [m/s]). There might be comment lines indicated by #.

After this header information, it follows a list of lines, where each line represents one point in time. Each line starts with a time offset from the reference date (in [time units]) and is followed by evaporation, transpiration and root zone depth in [m]. All these values are interpolated between discrete times.

#### Time series of land-use patterns

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|01.03.1993 00:00:00.00

|demo/in/luse93h.dat

|01.10.1993 00:00:00.00

|demo/in/luse93z.dat

|01.03.1994 00:00:00.00

|demo/in/luse94h.dat

|01.10.1994 00:00:00.00

This time series describes the temporal change of land-use patterns. It is a sequence of date strings (format dd.mm.yyyy hh:mm:ss.hs) and filenames (CHAR*30) (files specifying the relation between land-use units and land-use ID, as shown below). Each pattern is valid in the interval of the enclosing date strings.

##### Land-use patterns: Assigning land-use units to land-use ID

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| 1			% indicates that the first column, holding land use unit IDs, is related to the first

| 1	 5	 1	% column of land use IDs, a land use ID points on table, specifying the properties

| 2	 7	 1	% of a crop such as corn or wheat

| 3 	14 	2

| . .

| . .

This file just holds the assignment land-use unit to several land-use Ids as e.g. exported from a GIS. The number in the first line indicates the land-use-ID column to be related to the first land-use-unit column. These land use units point on a table, that specify time dependent properties of a crop such as corn or wheat (see section “ Assigning land-use ID to land-use parameter file ”).

#### Solute input mass

If solute transport is accounted for, i.e. istact > 0, the first step for specifying a Neumann (flux) boundary condition for solute input is to give the total input mass of each solute respectively. The file holds a number of lines equal to the number of specified soil types. In each line the total input mass of a solute and a temporal increment for the injection of solute particles is given. On runtime the solutes will be referred by an integer number according to their position in the list. Hence, the sequence of the input masses has to be consistent with the sequence of the solute parameters specified in the soil specification file (section: “Specification of soil types”) and the sequence of input concentrations c_inj in the time series of boundary conditions. There might be comment lines indicated by #. The particle mass is calculated by dividing the input mass by the default number of particles specified in the include file dim.inc.

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|# m_inj [kg]	t_inj [s]

|4.195e-3	120.		% Bromide (solute number one)

|1.e-03	120. 		% Isoproturon (solute number two)

|…

#### Boundary conditions of the drainage network

If interaction between subsurface water and channel water is simulated, i.e. q_meth = simact, it is possible to specify a time series of input flux into the drainage network for special nodes, marked as boundary nodes in the file describing the geometry of the drainage network. The first line holds two variables, namely a string (CHAR*22) which has to start from the first column in a format dd.mm.yyyy hh:mm:ss.hs and a conversion factor for the time unit used subsequently into seconds. There might be comment lines indicated by #. After this header information, it follows a list of lines, where each line represents one point in time. Each line starts with a time offset from the reference date (in time units e.g. h) and is followed by the input water flux [m3s-1].

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|01.01.2000 00:00:00.00		3600.

|# reference date		conversion factor[h]  [s]

|0.	0.

|0.1	0.01

|0.2	0.05

|5.9	0.08

|12.	0.1

|12.5	1.

|13.	1.

|14.	1.

|15.	0.5

|16.	0.01

|17.	0.

|18	0.

|96.	0.

### Assigning land-use IDs to land-use parameter files

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|5        rye                           in/luse/variety/rye.par  % land use ID 5 points of the given parameter file

|7        corn                          in/luse/variety/corn.par

|14       vegetables                    in/luse/variety/veg.par

|. . .

This file relates land use IDs, that are assigned to land use unites (section “Land-use patterns: Assigning land-use units to land-use ID”) to files that specify the time dependent properties of these crops (one line each). The first 9 columns hold the land-use identifier (INT). Columns 10-39 hold a descriptive string (CHAR*30). Columns 40-69 hold the filenames (CHAR*30) specifying land use parameters.

#### Specification of land use parameters

For each land use, a file with its parameters has to be specified, which covers all days of a year. From the first line only the number of parameters is read. The second line holds an offset at position 1 and as many factors as parameters specified. This numbers apply to the data read from subsequent lines, namely day number (of a year), surface roughness, macroporousity factor, leaf area index, coverage coefficient, rooting depth, plant height and several parameters characterising the evapo-transpiration model, as plant albedo, minimal stomata resistance, a factor and an inflexion point parameter of the soil moisture weighting function for stomata resistance computation. The last 3 columns hold land-use dependent erosion parameters not yet specified in more detail.

The offset allows shifting the time axis, which then is automatically extended to cover the range from 1 to 366. The factors are used to scale the associated land-use parameters by multiplication. The tables generated this way represent time series of the (changing) parameters, which are evaluated at runtime by linear interpolation.

| 13 | KST | fmac | BFI | BBG | TWU | PFH | PALB | RSTMIN | WPBFW | FBFW | EROS1 | EROS2 | EROS3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0. | 4. | 1. | 1. | 1.1 | 1. | 1. | 1. | 1. | 1. | 1. | 1. | 1. | 1. |
| 1. | 4. | 1. | 0.738 | 0.080 | 0.390 | 0.096 | 0.2 | 150. | 0.03 | 40. | ? | ? | ? |
| 25. | 4. | 1. | 0.574 | 0.080 | 0.390 | 0.096 | 0.2 | 150. | 0.03 | 40. | ? | ? | ? |
| 55. | 4. | 1. | 0.451 | 0.200 | 0.495 | 0.096 | 0.2 | 150. | 0.03 | 40. | ? | ? | ? |
| 85. | 3. | 1. | 1.599 | 0.448 | 0.600 | 0.204 | 0.2 | 150. | 0.03 | 40. | . | . | . |
| 115. | 2. | 1. | 3.649 | 0.800 | 0.795 | 0.504 | 0.2 | 150. | 0.03 | 40. | . | . | . |
| 145. | 2. | 1. | 4.100 | 0.800 | 1.005 | 1.104 | 0.2 | 150. | 0.03 | 40. | . | . | . |
| 175. | 2. | 1. | 3.321 | 0.800 | 1.500 | 1.200 | 0.2 | 150. | 0.03 | 40. | . | . | . |
| 195. | 3. | 1. | 2.050 | 0.720 | 1.500 | 1.200 | 0.2 | 150. | 0.03 | 40. | . | . | . |
| 196. | 4. | 1. | 0.000 | 0.000 | 0.000 | 0.000 | 0.2 | 150. | 0.03 | 40. | ? | ? | ? |
| 271. | 4. | 1. | 0.000 | 0.000 | 0.000 | 0.000 | 0.2 | 150. | 0.03 | 40. | ? | ? | ? |
| 300. | 4. | 1. | 0.123 | 0.080 | 0.195 | 0.096 | 0.2 | 150. | 0.03 | 40. | ? | ? | ? |
| 330. | 3. | 1. | 0.902 | 0.080 | 0.390 | 0.096 | 0.2 | 150. | 0.03 | 40. | ? | ? | ? |
| 360. | 4. | 1. | 0.779 | 0.080 | 0.390 | 0.096 | 0.2 | 150. | 0.03 | 40. | ? | ? | ? |
| 366. | 4. | 1. | 0.745 | 0.080 | 0.390 | 0.096 | 0.2 | 150. | 0.03 | 40. | ? | ? | ? |

with:

| Kst | Strickler coefficient |
| --- | --- |
| fmac | macroporousity factor - currently not used in calculations |
| BFI | Leaf area Index |
| BBG | plant coverage |
| TWU | root depth |
| PFH | plant height |
| PALB | plant albedo |
| RSTMIN | minimum stomatata resistance |
| WPBFW | factor for soil moisture weigthing function for stomata resistance, typically around 0.05 |
| FBFW | inflexion point of soil moisture weigthing function for stomata resistance, typically value around 30 |
| Eros1 -3 | dummy |

### Assigning sectors to a wind direction identifier

Computation of evapo-transpiration relies on wind velocity which is dependent from location and wind direction. Here, sectors are defined for which individual reduction factors are specified in the surface attributes file (section “Assigning surface attributes”). Angles are input in degrees clockwise from north. Furthermore, reduction factors for the location of the climate stations are specified. An example file that defines the first sector between 240 and 50 degrees, and the reduction factor of the second climate station for this sector as 0.70 would read:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| 240	 0.81	 0.70 . .

| 50 	0.78 	0.67 . .

| 80 	0.97 	0.88 . .

| 220 	0.94 	0.83 . .

### Drainage network geometry and -discretisation

The drainage network is represented by a graph made up from interconnected nodes derived by topographical analysis. Fig. 4.2 in section “4.4.4 Domain decomposition in hillslopes and drainage network” gives an idea of this discretisation. Several attributes and parameters are assigned to each node as shown in the example below:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| 3480100. 5442500.		% reference coordinates (x,y)

|: Comment 1

|: Comment 2

|: Comment 3

|: Comment 4

| ¦ | 1 | 2 | 1 | 13 | 0 | 993.750 | 3706.250 | 222.584 | 21.9888 | 2.5846 | 0.117540 | 13172.7 | (continued below) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ¦ | 2 | 3 | 1 | 0 | 0 | 978.202 | 3690.702 | 219.999 | 20.3959 | 3.0324 | 0.148678 | 0.0 |  |
| ¦ | 3 | 4 | 1 | 0 | 0 | 960.128 | 3681.250 | 216.967 | 21.0325 | 3.3692 | 0.160188 | 0.0 |  |
| ¦ | 4 | 5 | 1 | 0 | 0 | 943.444 | 3668.444 | 213.598 | 21.9888 | 4.0124 | 0.182475 | 0.0 |  |
| ¦ | 5 | 6 | 1 | 0 | 0 | 927.895 | 3652.895 | 209.585 | 21.9888 | 2.8795 | 0.130955 | 0.0 |  |
| ¦ | 6 | 7 | 1 | 0 | 0 | 912.347 | 3637.347 | 206.706 | 21.9888 | 2.0083 | 0.091333 | 0.0 |  |
| ¦ | 7 | 543 | 1 | 0 | 0 | 896.798 | 3621.798 | 204.697 | 21.9888 | 1.5420 | 0.070125 | 0.0 |  |
| ¦ | 8 | 9 | 1 | 135 | 0 | 1443.750 | 2793.750 | 184.244 | 21.9860 | 2.8529 | 0.129761 | 14143.8 |  |
| ¦ | 9 | 10 | 1 | 0 | 0 | 1428.204 | 2778.204 | 181.391 | 21.9860 | 1.4896 | 0.067753 | 0.0 |  |
| ¦ | 10 | 11 | 1 | 0 | 0 | 1412.657 | 2762.657 | 179.902 | 21.9860 | 1.1476 | 0.052195 | 0.0 |  |
| ¦ | 11 | 12 | 1 | 0 | 0 | 1397.111 | 2747.111 | 178.754 | 21.9860 | 1.2672 | 0.057636 | 0.0 |  |
| ¦ | 12 | 13 | 1 | 0 | 0 | 1381.564 | 2731.564 | 177.487 | 21.8582 | 1.0547 | 0.048251 | 0.0 |  |
| ¦ | 13 | 14 | 1 | 0 | 0 | 1381.250 | 2709.708 | 176.432 | 21.9860 | 0.7882 | 0.035850 | 0.0 |  |
| ¦ | 14 | 15 | 1 | 0 | 0 | 1381.250 | 2687.722 | 175.644 | 21.9860 | 0.6028 | 0.027417 | 0.0 |  |
| ¦ | 15 | 648 | 1 | 0 | 0 | 1381.250 | 2665.736 | 175.041 | 21.9860 | 0.5236 | 0.023814 | 0.0 |  |
| ¦ | -16 | 17 | 1 | 40 | 0 | 493.750 | 3081.250 | 202.063 | 20.2176 | 0.3782 | 0.018708 | 15038.5 |  |

| . . . . . . . . . . . .

|---.---------|----.----|----.--|----.------|-----.-----|----.----|----.----|-----.-------|-----|-----|-----|----|

| (continued from above) | 13172.7 | 3 | 8.0 | 222.44 | 0.147 | 0.015 | 8.00 | 0.13853 | 0. | 0. | 1. | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13172.7 | 3 | 8.0 | 219.85 | 0.149 | 0.015 | 8.00 | 0.12876 | 0. | 0. | 1. | 0 |
|  | 13172.7 | 3 | 8.0 | 216.82 | 0.149 | 0.015 | 8.00 | 0.12888 | 0. | 0. | 1. | 0 |
|  | 13172.7 | 3 | 8.0 | 213.45 | 0.151 | 0.015 | 8.00 | 0.12209 | 0. | 0. | 1. | 0 |
|  | 13172.7 | 3 | 8.0 | 209.43 | 0.152 | 0.015 | 8.00 | 0.11510 | 0. | 0. | 1. | 0 |
|  | 13172.7 | 3 | 8.0 | 206.55 | 0.154 | 0.015 | 8.00 | 0.10988 | 0. | 0. | 1. | 0 |
|  | 13172.7 | 3 | 8.0 | 204.54 | 0.155 | 0.016 | 8.00 | 0.10358 | 0. | 0. | 1. | 0 |
|  | 14143.8 | 2 | 8.0 | 184.08 | 0.163 | 0.016 | 8.00 | 0.09332 | 0. | 0. | 1. | 0 |
|  | 14143.8 | 2 | 8.0 | 181.22 | 0.167 | 0.017 | 8.00 | 0.08125 | 0. | 0. | 1. | 0 |
|  | 14143.8 | 2 | 8.0 | 179.73 | 0.173 | 0.017 | 8.00 | 0.06683 | 0. | 0. | 1. | 0 |
|  | 14143.8 | 2 | 8.0 | 178.58 | 0.179 | 0.018 | 8.00 | 0.05662 | 0. | 0. | 1. | 0 |
|  | 14143.8 | 2 | 8.0 | 177.30 | 0.183 | 0.018 | 8.00 | 0.04929 | 0. | 0. | 1. | 0 |
|  | 14143.8 | 2 | 8.0 | 176.25 | 0.187 | 0.019 | 8.00 | 0.04460 | 0. | 0. | 1. | 0 |
|  | 14143.8 | 2 | 8.0 | 175.45 | 0.193 | 0.019 | 8.00 | 0.03757 | 0. | 0. | 1. | 0 |
|  | 14143.8 | 2 | 8.0 | 174.85 | 0.195 | 0.020 | 8.00 | 0.03553 | 0. | 0. | 1. | 0 |
|  | 15038.5 | 3 | 8.0 | 201.87 | 0.196 | 0.020 | 8.00 | 0.03893 | 0. | 0. | 1. | 0 |

| . . . . . . . . . . . .

The first line holds reference coordinates, followed by exactly 4 lines of comments. The entries of each subsequent line hold the following data:

| ib | INT | number of node, negative values mark nodes for which a hydrograph is dumped to the output file |
| --- | --- | --- |
| ibnext | INT | next downstream node number |
| ib_ord | INT | Strahler order |
| inhang | INT | number of hillslope connected to node |
| inarea | INT | sealed surface connected (1) / not connected (0) |
| xba | REAL | relative x-coordinate [m] |
| yba | REAL | relative y-coordinate [m] |
| b_vorl | REAL | DEM-elevation at node [m+NN] |
| b_dx | REAL | distance to next downstream node [m] |
| dz | REAL | difference in elevation to next downstream node [m] |
| slope | INT | DEM-slope to next downstream node [-] |
| aahang | REAL | hillslope surface [sqm] |
| b_ezg | REAL | local catchment area [sqm] |
| ib_reg | INT | number of rainfall station associated to node |
| b_rauh | REAL | Strickler-roughness |
| b_yso | REAL | elevation of channel bottom [m+NN] |
| y_vorl | REAL | channel depth [m] |
| brso | REAL | bottom width of trapezoidial channel bottom [m] |
| b_m | INT | slope of channel banks [-] |
| b_gef | REAL | bottom slope to next downstream node [-] |
| weg_br | REAL | width of sealed surface parallel to channel [m] |
| qbas | REAL | lateral baseflow per unit channel length [cbm/s/m] |

In principle channel flow may be simulated in two different ways, with or without interaction between subsurface and channel water.

#### No interaction between subsurface and channel water

If no interaction between subsurface and channel water is simulated, i.e. q_meth = noicat, No initial state of the drainage network has to be specified. The lateral baseflow qbas servers as a constant lateral inflow from the hill slope into the drainage system. Moreover, surface runoff from hillslopes feeds the water flow in the channel network. Subsurface inflow from the hill slopes into the drainage network may be accounted for, if the right hill slope boundary condition is set to irbtyp = 10 (leakage boundary, see section “Assigning boundary conditions and sinks/sources). In case of saturation, subsurface water may exfiltrate via the channel walls into the drainage network. However, this way no infiltration of channel water via the channel walls into the hillslope may be accounted for.

#### Interaction between subsurface and channel water

If interaction between subsurface and channel water is accounted for, i.e. q_meth = simact, it is obligatory to specify an initial state of the drainage network as well as an input rime series of water flux (section “Boundary conditions of the drainage network”) for those nodes marked as boundary nodes (iqsour=1). On runtime this nodes will be referred by an integer number according to their position in the list.

Figure 3.1: When interaction between subsurface and channel water is simulated, the height of the channel water table causes a time dependent pressure boundary condition at the right hill slope boundary.

##### Initial state of the drainage network

If interaction between subsurface and channel water is simulated (q_meth =simact), the initial height of the water table has to be specified for each node of the drainage network as shown in the example below.

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| 1 0.1 : node number, height of the water table in m

| 2 0.1

| 3 0.1

| 4 0.1

| 5 0.1

|…

##### Assigning time series of input fluxes to boundary nodes of the drainage channel

The time series of input water fluxes into the drainage network listed in the specification of times series (see section “Specification of time series” and “Boundary conditions of the drainage network”) have to be related to the boundary nodes of the drainage network specified in the drainage network geometry. The first line holds an integer which determines the number of the following lines to be read. Please note that this number has to equal to the number of boundary nodes specified in the drainage network geometry. An example is given below:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| 2: two lines to be read

| 2: assigns time series number two to the first boundary node

| 1: assigns time series number one to the second boundary node 0.1

|

## Hillslope specific files

### Hillslope geometry and -discretisation

The hillslope is represented by a longitudinal vertical cross section along a line of steepest decent (streamline). The upper soil layer beneath this line has to be discretised in curvilinear coordinates prior to calculation by an external program. Fig. 3.2 shows an example of the discretisation of a hypothetical hillslope.

Figure 3.2: Discretisation in orthogonal curvilinear coordinates, the lateral direction is named as , the vertical is named as -direction.

Below, the file format is described by means of FORTRAN read statements; comments start with #.

| Indices declaration: |  |
| --- | --- |
| # iihg: unit number | # iv: counter in vertical (eta) direction |
| # ih: counter for hillslopes | # il: counter in lateral (xsi) direction |
| read(iihg,*) iacnv(ih), iacnl(ih), w_fix, hangnr(ih) | # iacnv(ih): vertical number of points |
|  | # iacnl(ih): lateral number of points |
|  | # w_fix: additive global angle of anisotropy against horizontal |
|  | # hangnr(ih): hillslope number (serves as label) |
| read(iihg,*) xkobez(ih), ykobez(ih), hkobez(ih) | # xkobez(ih): x-reference in global coordinates |
|  | # ykobez(ih): y-reference in global coordinates |
|  | # hkobez(ih): z-reference in global coordinates |
| read(iihg,*) hgobfl(ih), hgbreit(ih), hglang(ih) | # hgobfl(ih): area covered by hillslope |
|  | # hgbreit(ih): average width of hillslope |
|  | # hglang(ih): length of hillslope |
| read(iihg,*) (eta(iv,ih),iv = 1,iacnv(ih)) | # i.e. iacnv(ih) lines containing 1 value each: |
|  | # eta(iv,ih): discrete curvilinear coordinates in vertical direction |
| do 10 il = 1,iacnl(ih) | # i.e. iacnl(ih) lines containing 4 values each: |
| read(iihg,*) xsi(il,ih), | # xsi(il,ih): discrete curvilinear coordinates in vertical direction |
| & xko(iacnv(ih),il,ih), yko(iacnv(ih),il,ih), | # local real world x,y-coordinates (relative to reference) |
| & varbr(il,ih) | # varbr(il,ih): relative width of hillslope |
| 10 continue |  |
| do 30 il = 1,iacnl(ih) | # i.e. iacnv(ih)*iacnl(ih) lines containing 7 values each: |
| do 20 iv = 1,iacnv(ih) | # hko(iv,il,ih): local real world z-coordinate |
| read(iihg,*) hko(iv,il,ih), sko(iv,il,ih), | # sko(iv,il,ih): local real world s-coordinate (horizontal along hillslope) |
| & f_eta(iv,il,ih), f_xsi(iv,il,ih), | # f_eta, f_xsi: metric coefficients of curvilinear coordinates in vertical and lateral direction, respectively |
| & w_xsho(iv,il), w_hohr(iv,il), | # w_xsho(iv,il): angle between lateral coordinate and horizontal (positive counterclockwise) |
| & iboden(iv,il,ih) | # w_hohr(iv,il): angle between horizontal and main direction of anisotropy (positive counterclockwise) |
| 20 continue | # iboden(iv,il,ih): associated soil number of point |
| 30 continue |  |

This is an example file describing the discretisation (truncated):

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| 11 17 0.0 1  % (vertical nodes; lateral nodes; additive global angle of anisotropy against horizontal; hillslope number)

| 3480100. 5445400. 202.0 	% (x,y,z coordinates)

| 4. 10. 40.   		% (area, width, length)

| 0.00		% (discrete curvilinear coordinates in vertical direction; relative)

| 0.10		%…

| .

| 0.90		%…

| 1.00		% (discrete curvilinear coordinates in vertical direction; relative)

| 0.00 10.00000000 10.00000000 1.    % (xsi (curvilin. coord), xko (real world), yko (real world), width of hillslope)

| 0.25 10.17677670 10.17677670 1.    % (4 columns)

| . . . .

| 3.75 12.65165043 12.65165043 1.

| 4.00 12.82842712 12.82842712 1.    % (4 columns)

| 0.0000 0.0000 2.0000 1.0000 0.0000 0.0000 1	% (7 columns:

| 0.2000 0.0000 2.0000 1.0000 0.0000 0.0000 1	% hko, sko (real world vertical and horizontal),

| 0.4000 0.0000 2.0000 1.0000 0.0000 0.0000 1	% f_eta, f_xsi (metric coefficients for curvilin. coordinates),

| 0.6000 0.0000 2.0000 1.0000 0.0000 0.0000 1	% w_xsho (angle of lateral coord. and horizontal)

| 0.8000 0.0000 2.0000 1.0000 0.0000 0.0000 1	% w_hohr (angle of horizontal and main dir. of anisotropy),

| 1.0000 0.0000 2.0000 1.0000 0.0000 0.0000 1	% iboden (asscociated soil number of point).)

| . . . . . .

### Assigning soil types

The following sketch gives an example of a discrete representation of a hillslope, where nodes are assigned soil type numbers:

| 1.0 | 5 | 2------- ¦ ¦ | 2------- ¦ ¦ | 2------- ¦ ¦ | 2------- ¦ ¦ | 2------- ¦ ¦ | 2------- ¦ ¦ | 2 ¦ ¦ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| rev=0.7 | 4 | 1------- ¦ ¦ | 1------- ¦ ¦ | 1------- ¦ ¦ | 1------- ¦ ¦ | 2------- ¦ ¦ | 2------- ¦ ¦ | 2 ¦ ¦ |
| rav=0.4 | 3 | 2------- ¦ ¦ | 1------- ¦ ¦ | 1------- ¦ ¦ | 1------- ¦ ¦ | 2------- ¦ ¦ | 2------- ¦ ¦ | 2 ¦ ¦ |
| 0.2 | 2 | 2------- ¦ ¦ | 2------- ¦ ¦ | 2------- ¦ ¦ | 2------- ¦ ¦ | 2------- ¦ ¦ | 2------- ¦ ¦ | 2 ¦ ¦ |
| 0. | 1 | 2------- | 2------- | 2------- | 2------- | 2------- | 2------- | 2 |
|  |  | 1 | 2=al | 3 | 4 | 5 | 6 | 7 |
|  |  | 0.0 | 0.2 | 0.3 | 0.5 | 0.7 | 0.9 | 1.0 |

Figure 3.3: Assigning soil types to grid nodes

Soil type data can be assigned to the discretisation in Fig. 3.3, either by absolute node numbers and/or by relative values (example 1 below), or node wise by soil declaration in matrix form (example 2 below). These specifications overwrite the specifications given in the hillslope geometry file.

Example 1: input file for soil declaration by absolute node numbers and relative values

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| 2 0.: number of lines to be read

| 1. 5. 1. 7	2 : soil type 2 for nodes 1 to 5 in the vertical and 1 to 7 in the horizontal

| 3. 4. 0.2 0.5 	1 : soil type 1 for nodes 1 to 4 in the vertical and between 20 and 50% in the horizontal

|

Below is a general description by means of FORTRAN read statements.

read(iist,*) ianz, iart

# iist: unit number

# ianz: number of lines to read. Each line defines a window, and isoil is assigned to all nodes within.

# Each subsequent dataset may overlap and overwrite previous inputs!!!

# iart: always 0, kept for compatibility reasons

read(iist,*) rav, rev, ral, rel, isoil

# if ra? and re? are in the range (0.0...1.0) they are interpreted as relative eta,xsi values and transformed

# to absolute numbers av, ev, al, el,

# otherwise, if in the range of (1...iacnv,1...iacnl), they are regarded as absolute node numbers directly

Example 2: node wise soil declaration as matrix

|BODEN  46   48    1    % BODEN (indicates that soils are assigned nodewise), nv, nl, hillslope no.

|14	14	14	14	14	14	14	14	14	14	14	14	14

|14	14	14	14	14	14	14	14	14	14	14	14	14

|14	14	14	14	14	14	14	14	14	14	14	14	14

|…

|17	17	15	18	18	17	18	15	17	15	15	15	17

|

The hillslope number is only a description and not used in the program.

### Assigning factors to the Ksat field of the hillslope

This file allows for a simple implementation of areas of preferential flow by assigning factors to be multiplied with the Ksat values of each node. These factors are assigned nodewise in matrix form. If you do not wish to implement preferential flow this way, assign the value “1” to each node of the hillslope.

Example file:

|-1001  22  27 		% -(1000+ihg) (hillslope no.) nv, nl (no. of vertical and lateral nodes)

|1	1	1	1	1	1	1	1	100	1	1	1	…	1

|1	1	1	1	1	1	1	1	100	1	1	1	…	1

|1	1	1	1	1	1	1	1	100	1	1	1	…	1

|…

|1	1	1	1	1	1	1	1	1	1	1	1	…	1|

### Assigning factors to the theta field of the hillslope

Assign the value “1” to each node of the hillslope. So far not stable for values other than 1.

Example file:

|-1001  22  27 		% -(1000+ihg) (hillslope no.) nv, nl (no. of vertical and lateral nodes)

|1	1	1	1	1	1	1	1	100	1	1	1	…	1

|1	1	1	1	1	1	1	1	100	1	1	1	…	1

|1	1	1	1	1	1	1	1	100	1	1	1	…	1

|…

|1	1	1	1	1	1	1	1	1	1	1	1	…	1|

### Specification of control volumes

This is done by files of the same structure then those for assigning soil types, except the fact, that instead of a soil type no value has to be specified.

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| 2 0.: number of lines to be read

| 1 5 1 7	control volume 1 from nodes 1 to 5 in the vertical and 1 to 7 in the horizontal

| 1 4 0.2 0.5 	control volume 2 from nodes 1 to 4 in the vertical and between 20 and 50% in the horizontal

### Assigning macropore parameters

This is done by files of the same structure then those for assigning soil types. The first line holds 3 integers, the first indicates the number of subsequent lines to be read, the second specifies whether the assignment is done in relative coordinates (0) or node wise. The third variable indicates whether the saturated conductivity is increased only in the vertical direction (m_aniso = 1), only in lateral direction (m_aniso = 2), or in both directions (m_aniso  1 or 2). Instead of a soil type the macroporosity factor fmac (for an increasing hydraulic conductivity in case of preferential flow) and the macroporous cross section amac has to be specified as a REAL number. The macroporous cross section amac is used for determination of effective transport velocities v for solute transport, with v being either the arithmetic (ari) or geometric (geo) mean of matrix velocity vS = q / θ  and macropore velocity vM = q / (θ amac).

for S0 ≤ S ≤ 1,          otherwise            KSB = KS

KSB = conductivity of both macropore system and matrix

KS = conductivity of matrix

S0 = threshold of relative saturation for macropore flow (defined in soil type specification file)

S = relative saturation

β = exponent to steepen the gradient of KSB increase with saturation

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| 2 1 2: number of lines to be read; type of assignment 0 relative coordinates or 1 node wise; m_aniso

|ari: mean calculation for transport velocities (ari= arithmetic, geo= geometric)

| 1 5 1 7	2.  1.E-3  0.1	: fmac = 2 and amac= 1. E-3, β=0.1 for nodes 1 to 5 in the vertical and 1 to 7 in the horizontal,

| 1 4 0.2 0.5 	1. 0. 1:	 fmac = 1 and amac= 0, β=1 for nodes 1 to 4 in the vertical and between 20 and 50% in the horizontal

Or node wise assignement of macropores.

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

DIRECT amac, , m_aniso

ari

DIRECT 1.000000 1.000000 1

ari

1.000000 1.000000 65.312296 1.000000 65.285366 …

65.312296 1.000000 65.319032 1.000000 1.000000…

….

### Assigning initial conditions

#### Initial state for soil water dynamics

There are three different basic input formats.

##### Blockwise assignment of either absolute suction head or degree of saturation

This is done by files of a similar structure then those for assigning soil types. Here is a general description by means of FORTRAN read statements.

read(iist,*) ianz, ischal, iart

# iist: unit number

# ianz: number of lines to read. Each line defines a window,

# and isoil is assigned to all nodes within this window.

# Each subsequent dataset may overlap and overwrite previous inputs!!!

# ischal: controls the interpretation of rval

# ischal=0 degree of saturation (fraction of theta_sat)

# ischal=1 suction head [m]

# iart: always 0, kept for compatibility reasons

read(iist,*) rav,rev,ral,rel,rval

# if ra? and re? are in the range (0.0...1.0) they

# are interpreted as relative eta,xsi values and transformed

# to absolute numbers av, ev, al, el,

# otherwise, if in the range of (1...iacnv,1...iacnl)

# regarded as absolute node numbers directly

Two example files show, how initial condition data can be assigned to the discretisation, either by absolute node numbers and/or by relative values, please note that suction head in the model ist defined as follows  = 0 saturation,  > 0 normal case, i.e. soil moisture is less than the saturation value,  < 0 is reserved to saturated states, when the water pressure in the soil is higher than the atmospheric pressure:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| 2 0 0 % 2 lines to be read, degree of saturation (0)

| 1. 5. 1. 7. 0.6 % from node 1 to 5 in the vertical and from node 1 to 7 in the horizontal S=0.6

| 3. 4. 0.2 0.5 0.9 % from node 3 to 4 in the vertical and between 20 to 50% in the horizontal S=0.9

|

or

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| 2 1 0 % two lines to be read, suction head/matric potential (1)

| 1. 5. 1. 7. 1.40 % from node 1 to 5 in the vertical and from node 1 to 7 in the horizontal  = -1.4m

| 3. 4. 0.2 0.5 0.09 % from node 3 to 4 in the vertical and between 20 to 50% in the horizontal  = -0.09m

|

##### Assigning individual absolute values at each point

This file type specifies each value separately. In fact it is the dump format of the final state file.

The header line holds 6 variables: kenn, tzeit, ihg, nv, nl and the integer 1.

| kenn | CHAR*5 | either PSI or THETA |
| --- | --- | --- |
| tzeit | REAL | time offset [s] from a reference time |
| ihg | INT | Hillslope number |
| nv | INT | Vertical number of nodes |
| nl | INT | lateral number of nodes |
| 1 | INT | always one |

Depending on the keyword (kenn) either suction head (PSI) or water content (THETA) may be specified. The header is followed by iv = 1  nv lines of either - or -values, each line with il = 1  nl columns. Thus, the first line represents the upper boundary.

##### Uniform potential in entire domain

The header line is exactly the same as above. The keyword for potential initial condition is PHI. Only a second line follows, holding two values, phi_s and iart.

| Phi_s | REAL | potential in the entire domain |
| --- | --- | --- |
| iart | INT | switch which controls reference potential |
|  |  | iart=0: relative to heights specified in the geometry input file |
|  |  | iart=1: relative to lowest point of discrete geometry |

#### Initial solute concentration

There are two different basic input formats.

##### Blockwise assignment of residual total concentrations

This is done by files of a similar structure then those for assigning the initial state for soil water dynamics.

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| 2 1 2	% two lines to be read for each solute type, relative values or nodes, istac number of solute types

|1 	% first solute

| 1. 5. 1. 7. 1. E-3 	% from node 1 to 5 in the vertical and from node 1 to 7 in the horizontal C=1. E-3 g/kg

| 3. 4. 0.2 0.5 0.09	 % from node 3 to 4 in the vertical and between 20 to 50% in the horizontal C=0.09 g/kg

|2	%second solute

| 1. 5. 1. 7. 5. E-2 	% from node 1 to 5 in the vertical and from node 1 to 7 in the horizontal C=5. E-2  g/kg

| 3. 4. 0.2 0.5 0.5	 % from node 3 to 4 in the vertical and between 20 to 50% in the horizontal C=0.5 g/kg

##### Assigning individual concentration values at each point

This file type specifies each value separately. In fact it is the dump format of the final state file. The header line holds 6 variables: kenn, tzeit, ihg, nv, nl and istact.

| kenn | CHAR*5 | Konz |
| --- | --- | --- |
| tzeit | REAL | time offset [s] from a reference time |
| ihg | INT | hillslope number |
| nv | INT | vertical number of nodes |
| nl | INT | lateral number of nodes |
| istact | INT | number of different solutes |

The header is followed by iv=1  nv lines of C values [g/kg], each line for il=1  nl. Thus, the first line represents the upper boundary.

### Specification of printout times

This file format is almost the same as for boundary conditions. An example:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|31.12.1990 18:30:00.00 18000.

|# reference time [5h] -> [s]

| 0. 0

| 2. 0

| 10. 1

| 22. 0

| …

The first line holds two variables, namely a string (CHAR*22) which has to start from the first column in a format dd.mm.yyyy hh:mm:ss.hs+ and a conversion factor for the time unit used subsequently (to seconds). There might be comment lines indicated by.

After this header information, a list of time offsets from the reference date follows in the first column, the print times. The second column holds either 0 or 1. In case of 1, values from the entire hillslope domain are dumped, in case of 0 only those of surface points are dumped.

### Assigning surface attributes

For each discrete point of a hillslopes' surface this file assigns some identifiers and factors, in order to connect surface points to their correct time series of boundary conditions and land use. Line 1 holds the number of attributes to consider for 3 classes of attributes (polygon-IDs, wind direction identifiers, horizon elevations). Line 2 is merely descriptive. Subsequent lines hold attributes of land-use-unit numbers, precipitation file ID, climate file ID and as many factors as there are wind direction identifiers and horizon elevations.

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|3 4 36

| ¦1. node | land-use ID | precip.ID | Climat ID | wdirfac D1 | .... | wdirfacID4 | hor1 | hor2 | .... | hor36 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ¦2. node | 5 | 2 | 1 | 0.81 | .... | 0.93 | 12 | 16 | .... | 9 |
| ¦3. node | 5 | 2 | 1 | 0.76 | .... | 0.93 | 13 | 17 | .... | 10 |
| ¦4. node | 5 | 4 | 1 | 0.73 | .... | 0.92 | 13 | 17 | .... | 10 |
| ¦5. node | 12 | 4 | 1 | 0.72 | .... | 0.91 | 13 | 17 | .... | 11 |
| ¦6. node | 12 | 4 | 1 | 0.70 | .... | 0.89 | 14 | 19 | .... | 12 |
| ¦ |  |  |  |  |  |  |  |  |  |  |
| ¦ |  |  |  |  |  |  |  |  |  |  |
| ¦last node | 12 | 4 | 1 | 0.70 | .... | 0.89 | 14 | 19 | .... | 12 |

### Assigning boundary conditions and sinks/sources

Each boundary node has to be assigned a boundary condition for all times of a simulation (mandatory). Optionally, each element can be assigned a sink/source term. Either constant or time varying conditions can be assigned. Again assignment is done by key-letters. There are up to 5 input-blocks, which are identified by key-letters in the first column:

| R | right boundary (mandatory) |
| --- | --- |
| L | left boundary (mandatory) |
| T | top boundary (mandatory) |
| B | bottom boundary (mandatory) |
| S | source/sink (optional) |
| M | mass input (in case of solute transport) |

Please note that if input mass file number n is related to a hill slope under M, the corresponding boundary condition file has also to be related to that hill slope. After the key letter has been identified, input is read in the following manner (analogous the assignment of soil type numbers):

read(iirb,*) ianz, iart

# iirb: unit number

# ianz: number of lines to read. Each line defines an interval

# and isoil is assigned to all nodes within this interval

# each subsequent dataset may overlap and overwrite previous inputs!!!

# iart: always 0, kept for compatibility reasons

#

# Depending on the key-letter, one of the following statements is executed:

# key-letter T or B:

read(iirb,*) ral, rel, irb_?

# key-letter R or L:

read(iirb,*) rav,rev, irb_?

# key-letter S:

read(iirb,*) rav,rev,ral,rel,irb_?

# if ra? and re? are in the range (0.0...1.0) they are interpreted as relative eta,xsi values and transformed to

# absolute numbers av, ev, al, el,

# otherwise, if in the range of (1...iacnv,1...iacnl), they are regarded as absolute node numbers directly

# irb_? specifies the boundary condition at each single node:

# irb_? > 0 indicates a time-series file-number to be assigned to specified nodes.

# The filename associated with the number is read from the boundary time series file.

# irb_? = 0 zero flux boundary condition

# irb_? = -3 gravitation flux boundary condition

# irb_? = -4 fixed constant potential

# irb_? = -5 flux divergence = zero: free flow across boundary (implemented for right and lower boundary)

# irb_? = -10 seepage boundary condition IN NEWEST VERSION: WATER LEAVES THE SYSTEM ABOVE               FIELD CAPACITY (NOT ONLY WHEN SATURATED)

# irb_? = -99 atmospheric boundary condition NEEDS TO BE IMPLEMENTED IN SINK/SOURCE FOR TRANSPIRATION TO BECOME EFFECTIVE

# Interpretation of atmospheric boundary condition (-99):

# In this case boundary conditions are determined from climate, # rainfall and land-use data. Internally the following # BC apply:

## irbtyp rbpar1 rbpar2 description

# -------------------------------------------------------------------------------------------------------------------------

| # | top: | +/-11 | -y_w | [m] | -i_net | [m/s] | rainfall |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | bottom: | 3 | - | - | free | drainage |  |  |
| # | left: | 0 | - | - | zero | flux |  |  |
| # | right: | +/-10 | - | - | seepage |  |  |  |
| # | or | 2 | k/d | [1/s] | Phi_a | [m] | leakage | (mixed) |
| # | sink/source: | according | to | evapotranspiration |  |  |  |  |

This is a simple example file describing assignment:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|LEFT

|1 0

| 0. 1. 0		% should almost always be 0, unless input time series is defined.

|

|RIGHT 		%right boundary

|1 0 		% 1 line to be read and dummy (always zero)

| 0. 1. 0		% zero flux is valid from 0 to 100% of the length of the right boundary

|

|TOP		% upper boundary

|2 0		% 2 lines to be read and dummy (always zero)

|0. 1. –99		% atmospheric boundary conditions valid from 0 to 100% of the length of the upper boundary

|7 16 1		% boundary conditions specified in file number 1 valid from node 7 to node 16 at the upper boundary

|

|BOTTOM		% lower boundary

|1 0		% 1 line to be read and dummy

| 0. 1. –3		% free drainage valid from 0 to 100% of the length of the lower boundary

|

|SINK/SOURCE

|1 0		% 1 line to be read and dummy

| 0. 1. 0. 1. –99	% atmospheric sinks and sources valid for whole domain (TRANSPIRATION!)

|

|MASS 		% either 0 (then no massinput is associated) or n > 1 (integer) relating the mass input file

| 1	% number n to the hillslope

# Preparing input files

## Prolog

As can be seen from the input description given in the previous chapter, the model's data requirements are enormous. Furthermore, the 4-dimensional nature of the problem at hand, together with the interaction of processes on different scales, and the high interdependence of model variables and parameters, is reflected in a model and data structure of considerable complexity. A range of questions has to be answered:

- At which scale (support, resolution, extend) data are available?

- At which scale data are required?

- How to bridge gaps due to differences between 1. and 2.?

- How to homogenise data of different sources?

- How to store data?

- How to retrieve data?

- How to visualise data?

- How to organise data for use in a model?

- How to flexibly incorporate new modules (and thus data) in a model?

- How to handle the even bigger amounts of data that are produced during simulation?

An attentive reading of recent publications on these issues reveals a growing awareness of open problems related to this questions and, on the other hand, the enormous development times spent to achieve thorough environmental software systems at state of the art, which easily sum up to dozens of man-years.

Exemplary, a quotation from a product information of PCRaster, a dynamic modelling language for two-dimensional map objects (http://www.frw.ruu.nl/pcraster/pcraster.html), developed in the Netherlands is given here:

Developing a computer model is not an easy task. The first difficulty is that environmental models are relatively complex because they must be both temporal and spatial due to the spatio-temporality of natural phenomena. Related to this is the problem that they are fed by huge amounts of environmental data, often coming from different kinds of sources such as manual field measurements, automated data loggers or remotely sensed images.

Further on it is pointed out that their software product is the result of eight years of research by a team of scientists with diverse backgrounds such as geographers, hydrologists, mathematicians and software engineers. Only recently, in recognition of the non-triviality of this issues, conferences and workshops on environmental software systems have been organized, e.g. the. “International Symposium on Environmental Software Systems (ISESS), 1995” at Penn State University, Malvern, PA, USA and 1997 at Whistler, British Columbia, Canada or the “Third International Conference/Workshop on Integrating GIS and Environmental Modelling”, January 1996 in Santa Fe, New Mexico, USA and also the HydroGIS conferences “Application of Geographic Information Systems in Hydrology and Water Resources Management” in Vienna, Austria, April 1993 and April 1996.

The announcement paper of a workshop called “Tools for Environmental Informatics” (Whistler, British Columbia, Canada, http://cfc.crle.uoguelph.ca/isess97/courfin.htm) points out gaps in knowledge and lack of awareness of problems associated with this issues: The concept of environmental informatics has not yet satisfactorily penetrated North America. There is an assumption that environmental science is orchestrated by planners and conciliators on one hand, and civil and environmental engineers/scientists on the other. The experiences of several groups in Canada, the U.S., Australia and Europe (most notably Germany and Austria) would suggest otherwise. Simultaneously the “stakeholders” in the academic structure and the existing environmental regulation and policy management structure are only recently wakening to the need for sophisticated information systems. The data are incomplete, the concepts of data management are slow to adapt and the money at all levels (including education and training) is shrinking.

As a consequence of this insights a workshop is proposed which theme is summarise as follows:

Environmental data management and environmental information systems have to bridge gaps in time and space in data, information and knowledge. Elements of modelling, statistical computing, scientific database, visualisation, environmental statistics (including risk assessment), uncertainty estimation and management, integration of heterogeneous and legacy systems and knowledge engineering all contribute to a better understanding of environmental problems.

It is the authors’ conviction that for a more effective use of resources much work has to be done in order to facilitate data handling and visualisation. Probably a more generalised method for administrating data has to be developed to ensure an open and easily extendable software architecture of long term continuance. This task encloses the integration of software tools such as databases and GIS as well as models for the various types of interpolation and extrapolation in the space-time domain (including physically-based, distributed models like CATFLOW). This challenge seems to be critical to prevent modellers from reinventing the wheel again and again, thus marking time and wasting resources urgently needed to advance science.

## General

CATFLOW works on a specified discretisation in space and time. This has implications on how variability is treated. The smallest variability which (at least theoretically) can be taken into account explicitly is of the order of the discretisation. A possible variability below discretisation scale has to be treated in an implicit, lumped way (Bloeschl, 1996c). Several approaches exist to quantify this so-called subgrid-variability and roughly can be grouped into two classes:

- effective parameters: the parameters of a process describing equation are modified, e.g. parameters are distributed with some distribution function.

- effective models: the process equation itself is changed, e.g. the process equation is rewritten using perturbated state variables which yield additional terms, so-called closure terms, which then have to be parameterised.

Not all information is available at the scale of CATFLOW's discretisation. Therefore, scaling laws have to be applied (or are applied implicitly) when assigning data to the discrete representation of the domain under consideration. It is important to keep in mind the possible effect of the underlying scaling assumptions when judging about the quality of a simulation and the confidence which can be set into a result.

The following sections give a brief overview of CATFLOW's concepts of data management and how the preparation of data for the model CATFLOW currently is conducted, using tools provided by different software environments and exchanging data mostly in ASCII-file formats. This procedure, together with data structure already embedded in the existing code, can be regarded as the minimum effort to be invested to enable model handling.

However, it is the authors conviction, that the methodology has to be developed further towards more generality and flexibility to facilitate model handling and to more easily allow for model extensions, thus improving performance and widen the range of applicability.

## Data organisation

The key to all data are their location in space and time. All data are attributed to this key. However, this location is not a point in a mathematical sense, i.e. associated with this key is an understanding of properties characterising the quality and validity of the data (method, volume/support, spacing, extend etc.). This demands for a universal definition of a data-descriptor and -qualifier. Physically based process descriptions are time marching, i.e. they describe the transition of some spatially distributed state variables with time, taking into account their interactions. In theory, there are two straightforward possibilities to provide required input data and parameters (and to store output data of computed state variables):

- For each point in space time series of every input data are given, i.e. one spatial distribution of time series (SDTS)

- For each point in time spatial distributions of every required input data (one value at each computational node) are given, i.e. one time series of spatial distributions (TSSD).

Both approaches keep the same information, but are structured in different ways. Practically there are two limitations to prevent this approach:

- For larger models (number of nodes) both methods lead to non-operational amounts of data and to a high degree of inflexibility when it comes to variation of inputs, especially due to high portions of redundant data.

- The scale of available data usually differs significantly from the scale of modelling, i.e. scaling assumptions need to be made (and, if need be, external models have to be used) in order to distribute possibly sparse data into the entire space-time domain under consideration. Often, this involves the assignment of one value to several computational nodes.

In this context, the term ``validity-volume'' is introduced, i.e. the volume in time and space for which some given data are regarded to be representative. The validity-volume is referenced as a combination of

- a polygon in the horizontal x,y-plane

- an interval in the vertical z-direction

- an interval in the time domain t

Any data are an attribute to some validity-volume.

Analogously to point values, validity-volumes can be assigned either by SDTS or TSSD. A single data value has to be assigned to all computational nodes within the validity-volume.

Which assignment approach is suited best, i.e. requires minimum data management efforts? The answer to this question depends on whether temporal or spatial variations prevail for some given data or parameter.

- If temporal variations prevail, the SDTS approach is suited better; e.g. few time series of rainfall are assigned to few Thiessen polygons of the associated rainfall stations.

- If spatial variations prevail, the TSSD approach is suited better; e.g. few spatial distributions of land-use IDs are assigned to land-use units at only few times within a year.

## Required information

### Spatial information

Prior to the preparation of input files spatial data have to be provided, possibly in a geographic information system (GIS) environment, as far as areal (2D) data are concerned. This can be either

- raster data or

- vector data, including the following data types

- points

- lines (series of connected points)

- polygons (closed lines)

Data included are:

- topography (raster and polygons); in order to be able to treat raster data in a similar methodology as data attributed to polygons each raster element is stored as a polygon)

- land-use units (polygons)

- soil type polygons and vertical subdivision for each polygon

- precipitation stations polygons (e.g. Thiessen-polygons)

- climate station polygons (e.g. Thiessen-polygons)

### Temporal information

Prior to the preparation of input files temporal data (time series) has to be provided, possibly in a relational database management system (RDBMS):

- precipitation

- climatic data, such as net radiation, temperature, humidity, windspeed, wind direction

- changes in land use patterns

- groundwater levels

- vegetation related transient parameters, such as surface roughness, macroporousity, leaf area index, degree of soil coverage, rooting depth, plant height, diverse parameters of the evaporation model

### Constant information

By this term, time and space independent data are addressed, such as:

- soil hydraulic functions, respectively parameters of a functional model

- land use parameters (not really time independent, but periodic)

### Domain decomposition in hillslopes and drainage network

Prior to all computation, the control volume under consideration (i.e. the upper soil layer of a catchment) has to be decomposed in hillslopes. This task is done in a semi-automated process making use of digital elevation model (DEM) analysis software such as DEDNM (Martz & Garbrecht, 1992; Martz& Garbrecht, 1993; Garbrecht & Martz ,1993) or T_4HIM (Wiesmann et al., 1994; Streit & Wiesmann, 1996; Streit & Kleeberg, 1996).

#### T_4HIM

Here, T_4HIM (Tools for Hydrological Information Management'') an ARC/INFO based interactive shell-program is used.

Given the digital elevation model in a raster format the application yields subcatchments at user-defined outlets and a detailed drainage network as raster information which subsequently is transformed to vector data, depicted in Fig. 4.1 b .

a) contourlines					b) subcatchments and drainage network

Figure 4.1: Topographic analysis of the 3.5 km2 Weiherbach catchment

After this, hillslopes are determined manually under consideration of geometries of the drainage network, subcatchment boundaries, land-use units and lines of steepest descend (normal to topographic contours). This task again is supported by T_4HIM allowing on-screen digitalisation on arbitrary zooms of the map while before-mentioned secondary information is underlayed. Hillslopes (Fig. 4.2a) are abstracted by their longitudinal section along mean slopelines, assumed to be of variable width. The result (as shown in Fig. 4.2b) is a graph of lateral flow paths made up from slopelines and drainage network. It is up to the modeller to choose the degree of abstraction by selecting the number of hillslopes.

a) hillslopes					b) slope- and drainage lines

Figure 4.2.: Domain decomposition of the 3.5 km2 research catchment “Weiherbach” up to gauge “Menzingen”.

At this point of the analysis we have coverages of the following objects:

- hillslopes (type: polygons)

- slopelines (type: lines)

- drainage network (type: lines)

#### ARC/INFO

The coordinates of points making up the slopelines are dumped to an ASCII file keeping the associated hillslope number as an attribute, as well as the coordinates of the drainage network marked by a distinguished identifier (File A, hangfall.out). Unfortunately this pieces of line information are not oriented (direction of flow is unknown). In addition a list of the hillslope numbers and associated hillslope areas is dumped (File B hngpol_a.out).

In order to derive the variable width of the hillslopes, an area associated to each individual point of a slopeline has to be extracted by the following procedure implemented in an AML-script (ARC Macro Language): Each hillslope is isolated as well as its associated slopeline (Fig.4.3a). The slopeline is converted in a coverage of type “points” (Fig. 4.3b). These points keep the number of their associated slopeline as an attribute. Unfortunately the connectivity (i.e. order) of points is lost in this step. Within the boundaries of the hillslope Thiessen polygons (Voronoi diagrams) are derived for each point (Fig. 4.3c). The coordinates of the points together with the slopeline number and the area of the Thiessen polygons are dumped to two ASCII-files (File C xyzpkthg.out and File D pkthgfl.out).

a) slopeline			b) points				c) Thiessen polygons

Figure 4.3: Steps towards the determination of hillslope's variable width

#### MATLAB

Within a MATLAB environment files A to D are read together with the DEM in order to combine and synthesize all information and derive a graph of the flow network. First, data from file C and D is read and combined by program BREIT2 to yield File E. Then data from file A and B is read and for each point's coordinate the elevation is extracted from the DEM by interpolation. Comparing the coordinates from file A and E allows to recover the alignment of points read from E. Then, the variable hillslope width can be calculated by division of polygon areas from file E and distances between slopeline points. By comparison of the endpoint elevations of line objects slopelines as well as drainage network line objects are sorted in a descending order (assuming flow to travel from higher to lower elevations). The combined data are dumped to a MAT-file (in MATLAB format, fully geo-referenced). This program is called PREPARE1.

Occasionally, the upstream point's elevation of a short line object might be lower than the downstream elevation. In such cases, subsequent analysis will fail and the flow network's connectivity has to be corrected. Here the program KORREKT is used.

### Ordered graph of drainage network

By comparison of the endpoint coordinates of line objects an ordered graph of the drainage network is derived by a MATLAB program called PREPARE2. Each single point is numbered in a way, that upstream point numbers always are lower then downstream point numbers. Point numbers of confluences with slopelines are associated to slopeline numbers. Finally the Strahler order (Strahler, 1956) is determined and stored.

The graph of the drainage network is made up from reaches, each of which is assigned a trapezoidial cross-section. Optionally, the properties of the drainage network are determined by aid of geomorphological reasoning. Geometric properties derived from these profiles are passed to the flood routing schemes.

### Discretisation of hillslopes

The irregularly shaped domain of a hillslope's longitudinal section is discretised using curvilinear orthogonal coordinates to (a) flexibly model the irregular boundaries and (b) more easily conduct numerical solutions of Richards equation for matrix flow on a mathematical rectangular grid. An algorithm described by Kiefer et al. (1990) is used to generate the curvilinear orthogonal coordinates from a mathematical description of the four bounding curves.

This algorithm is implemented in the MATLAB file h_spl.m making use of boundary descriptions by cubic splines. It allows passing nothing than an arbitrary number of boundary points (in cartesian coordinates) to the algorithm instead of a mathematical description. Fig. 4.5 shows the result for two exemplary hillslopes. The density of discretisation is highest in areas expected to be hydrologically most active. Fig. 4.6 shows the resulting surface discretisation of a catchment.

The program is called H_SPL.M and reads the output file from PREPARE1 (respectively KORREKT). It only relies on the slopelines geometry. The user has to specify a desired soil depth; then, the boundary curves are generated automatically. Additional input is the desired resolution of the discretisation. Final the program dumps all geometry files for use with CATFLOW.

Figure 4.5: Longitudinal-vertical section of two exemplary hillslopes and curvilinear orthogonal discretisation (computational nodes)

Figure 4.6: Hillslope discretisation: Plane view of the 3.5 km2 research catchment “Weiherbach”

## Assigning attributs/indices

### Surface (2D)

The generation of several input files requires the assignment of surface attributes to the surface nodes of the computational grid. This is performed by routines which read the discretisation file as well as various polygon files (exported from GIS) and apply a “find-point-in-polygon”' algorithm. Results are dumped to an assignment file which relates computational nodes to indicators and factors like:

- land-use units

- soil column type

- precipitation stations

- climate stations

- wind reduction factors for various directions

### Depth (3D)

The generation of several input files requires the assignment of attributes to all nodes of the computational grid. This is performed by routines which read the discretisation file as well as surface attributes assignment files which assign attributes to some depth intervals for different surface attributes.

A “find-point-in-interval” algorithm is applied to assign:

- soil type numbers

- initial conditions

- factors for macroporosity

- to all computational nodes

## Soil types

The file assigning hydraulic functions to soil type numbers is generated manually, (section “Specification of soil types”).

## Wind directions

The file assigning sectors to wind direction identifies is generated manually (section “Assigning sectors .to a wind direction identifier”).

## Land use parameters

The three file types specifying land-use parameters are generated manually (a favourable approach would be retrieving data from an experts database), (section ”Land use IDs and subsections”).

## Time series

Time series hold data of arbitrary spacing (breakpoint-format), thus reducing storage size. Furthermore, this storing scheme ensures the choice of a maximum possible time step at simulation time, because each breakpoint might restrict a possible time step. At one moment of simulation only the two values which directly enclose the actual simulation time are kept in memory. Currently, these files are created by custom-made programs, suited to reformat data formats of various origins.

- precipitation

- climate

- land use

- groundwater levels (pressure heads)

## Initial conditions

Initial conditions can be assigned by various methods (section “Initial conditions”). Ideally, each computational node is assigned a measured and/or scaled value, as can be obtained e.g. by specialised kriging-routines (Lehmann, 1995). The requires various drift- or indicator-variables at all computational nodes and measurement point to be provided, in order to use them with e.g. external drift kriging. Finally, 3d-assignment routines have to be applied.

## Control files

Various control files have to be prepared manually.

- main input file

- project control file

- global time series file name specification

- file specifying printout times

- file specifying control volumes

# Simulation output

After preparation of all input files CATFLOW can be started. Information is read from the main input file CATFLOW.IN and subsequent files. Summarising information about each calculated time step is reported to the screen and the log-file. State variables are dumped to output files at specified times (section “Specification of printout times”). For each hillslope a time series of spatial distributions is stored (TSSD). A detailed time series of rainfall and overland flow at the lower end of all hillslopes is recorded.

## Log-file

This file logs program inputs and lists the following information of each calculated time step:

| no | hillslope number |
| --- | --- |
| izaehl | timestep counter |
| t_act [h] | actual time since start in [h] |
| dt [s] | actual time step in [s] |
| cumbil [vol] | hillslope integrated and standardised cumulative balance |
| stpbil [rfl] | hillslope step balance, standardised with boundary fluxes |
| opt/max | actual time step control variable |
| n_it cg | actual number of conjugate-gradient-iterations |
| n_it pic | actual number of Picard –iterations |
| dPhi max | L norm of the residuum |

## Tables of soil-hydraulic functions

All soil type parameters are transferred to a table of the soil hydraulic functions by their associated model. These tables are dumped to a file of similar format then the file explained in section “Specification of soil types”; the header line holds a title string, two anisotropy factors, the soil model identifier (e.g. Van Genuchten = 1), the number of tabulated values and the specific storage coefficient.

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|title string (CHAR*30) 	1.000	 1.000	1	500	0.1000E-03

|%  		k	C

|0.656065E-01	0.100000E+06	0.513940E-21	-0.156982E-07

|0.656867E-01	0.950508E+05	0.589429E-21	-0.167519E-07

|0.657681E-01	0.903466E+05	0.676006E-21	-0.178765E-07

|0.658507E-01	0.858752E+05	0.775299E-21	-0.190765E-07

|0.659344E-01	0.816251E+05	0.889178E-21	-0.203570E-07

|0.660194E-01	0.775853E+05	0.101978E-20	-0.217236E-07

|0.661056E-01	0.737455E+05	0.116957E-20	-0.231818E-07

| . . . .

## Water balance

This file reports cumulative hillslope water balances for each specified output time.

| ih | hillslope number |
| --- | --- |
| izaehl | time step number |
| t_act | computation time [s] |
| biltot | cumulative total water balance (absolute error of water balance) bilin –bilrd- bsenk [m³] |
| bilin | cumulative total increase of soil water content [m3] |
| bsenk | cumulative sink/source input |
| bilrd | cumulative balance of boundary fluxes |
| -brfl_o | cumulative flux across upper boundary (perpendicular in the eta xsi coord.system) |
| -bflr_r | cumulative flux across right boundary |
| brfl_u | cumulative flux across lower boundary |
| brfl_l | cumulative flux across left boundary |
| qosum | cumulative amount of surface runoff |
| rcoeff | runoff coefficient |
| bnied | cumulative amount of precipitation perpendicular in eta xsi coordinates |
| bnied2 | Cumulative amount of precipitation perpendicular in Cartesian coordinates |
| bintz | cumulative amount of interception perpendicular in eta xsi coordinates |
| bevapo | cumulative amount of evaporation perpendicular in eta xsi coordinates |
| btrans | cumulative amount of transpiration |
| bm_l, bm_u, bm_r (istp,ih) | Cumulative loss of solute 1 at left/ lower/ right boundary of hillslope ih, cumulative loss of solute 2 at left/ lower/ right boundary, cumulative loss of solute 3 at left/ lower/ right boundary |

At the end of simulation a more detailed cumulative hillslope water balance is reported for subregions as specified in the control volume file. Fluxes that point inward are counted positive, fluxes that point outward are counted negative.

## State variables

The state variables dumped to files are soil moisture, suction head, flux and sink/source terms. All this files hold time series of spatial distributions within vertical-longitudinal hillslope sections. For each hillslope and printout time a block of the following structure is written:

A header line holds 5 variables: tzeit, -(1000+ihg), nv, nl.

| tzeit | REAL | time offset [s] from reference time |
| --- | --- | --- |
| -(1000+ihg) | INT | hillslope number |
| nv | INT | vertical number of nodes |
| nl | INT | lateral number of nodes |

The header is followed by iv=1  nv lines of state variables, each line for il=1  nl. Thus, the first line represents the upper boundary. The state variables are

soil moisture

suction head

water fluxes in vertical and lateral direction

final water state

sink/source terms

solute concentrations

## Overland flow

A time series of precipitation and overland flow at the lower end of a hillslope is dumped to a file, which first line keeps the reference date. The second line holds conversion factors to SI-units for columns 2-5. Subsequent lines hold hillslope number ihg, time t since reference date, overland flow (volume per time), and cumulative runoff volume. An example:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|28.07.1996 16:15:00.00

| 1.157407407e-05 3600000.0000 0.4006172928779019E+02 0.1112825813549728E-01

|	 ihg	 time	neff [m/s]	q [m3/s]		cumulative q [m3]

|	1	0.00	0.0000E+00	0.0000E+00	0.0000E+00

|	1	1.50	0.8240E-08	0.0000E+00	0.0000E+00

|	1	3.75	0.8240E-08	0.0000E+00	0.0000E+00

|	1	7.13	0.8240E-08	0.0000E+00	0.0000E+00

|	1	12.19	0.8240E-08	0.0000E+00	0.0000E+00

| . . . . .

## Evapo-Transpiration

The evapo-transpiration file holds for a specified node at each hill slope time series of variables important for the SVAT-module:

| ich | hillslope number |
| --- | --- |
| il | number of surface node |
| time | computation time [d] |
| LAI | leaf area index |
| BBG | plant coverage |
| twu | root depth [m] |
| pfh | plant height [m] |
| rGlo | global radiation [W/m2] |
| rBil | radiation budget [W/m2] |
| rBilm | observed radiation budget [W/m2] |
| shf | soil heat flux [W/m2] |
| temp | air temperature [°C] |
| Rhum | relative humidity [-] |
| Uref | wind speed in reference height [m/s] |
| Al | albedo [-] |
| Ustar | friction velocity [m/s] |
| u100 | wind speed in reference 10m above surface[m/s] |
| Thpfvw | permanent wilting point |
| Intz | interception [mm] |
| Nied | precipitation [mm/d] |
| Neff | effective precipitation [mm/d] |
| Catmp | turbulent conductivity for water vapour of the atmosphere [m/s] |
| Ccanop | turbulent conductivity for water vapour of the canopy layer [m/s] |
| Catms | turbulent conductivity for water vapour of the atmosphere [m/s] |
| Csoil | turbulent conductivity for water vapour of the soil [m/s] |
| Ecanop | plant transpiration [mm/d] |
| Eintz | evaporation of intercepted water [mm/d] |
| Esoil | evaporation from the soil [mm/d] |

## Channel flow

For each of the specified nodes of the drainage network, i.e. the nodes with a negative node number (see section “Drainage network geometry”) a hydrograph is stored:

increasing node numbers

| time [s] | icour | qpegel | totvol | regen | qb | yb | vb | qb | yb | vb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 360. | 16 | 0.0 | 2.5 | 0.00 | 0.000 | 0.00 | 0.00 | 0.004 | 0.05 | 0.34 |
| 720. | 16 | 0.8 | 3.8 | 0.00 | 0.002 | 0.04 | 0.31 | 0.008 | 0.05 | 0.35 |
| 3600. | 256 | 126.8 | 15.3 | 0.00 | 0.044 | 0.10 | 0.53 | 0.048 | 0.10 | 0.54 |
| 7200. | 320 | 303.2 | 15.4 | 0.00 | 0.049 | 0.10 | 0.54 | 0.050 | 0.10 | 0.54 |
| 10800. | 320 | 481.5 | 15.5 | 0.00 | 0.050 | 0.10 | 0.54 | 0.050 | 0.11 | 0.54 |
| 14400. | 320 | 661.1 | 15.5 | 0.00 | 0.050 | 0.11 | 0.54 | 0.050 | 0.11 | 0.54 |
| 18000. | 320 | 841.8 | 15.6 | 0.00 | 0.050 | 0.11 | 0.54 | 0.050 | 0.11 | 0.54 |

with

| icour | Part of Courant criteria |
| --- | --- |
| qpegel | cumulative outflow from the catchment [m3] |
| totvol | cumulative wetted channel volume [m3] |
| regen | precipitation [mm/h] |
| qb | channel flow [m3/s] |
| yb | height of water level [m] |
| vb | wetted channel volume [m3] |

## Co-ordinate files

The z- and x-coordinates of the grid of each hillslope are stored into two output files. The first line of each file contains the hillslope number -(ih+1000) followed by iv=1  nv lines of coordinate values [m], each line for il=1  nl. Thus, the first line represents the upper boundary of the hillslope.

# Visualisation

## General

Visualisation is most important to judge about input and output data. Furthermore, in case of large amounts of data comfortable data handling is an unevitable prerequisite to effectively use a model. However, developing nice post-processors is a task of high manpower consumption, requiring considerable knowledge of modern software capabilities in order to be efficient. Recently emerging interests in environmental software systems honour these findings. For the time being, an “ad hoc” solution based on Matlab-macros is presented here.

## Longitudinal sections using Matlab

For individual hillslopes the distribution of soil moisture, suction head as well as solute concentrations may be plotted in as 2-d vertical cross sections in the z-x- plane.

### 2-d cross sections of state variables without water fluxes

In this case the MATLAB-macro pstate.m (plot state variables) and the macro calnp (calculate number of print out times) are used. As an example the following lines explain the visualisation of the soil moisture field stored in a file named theta.out for hillslope 2 at the 3. printout time (This output may be produced by running the demo project, Fig. 6.1):

copy the macros into your output directory e.g. into demo/out

run calnp.m with calnp(‘theta.out’) to find out the number of printout times

Running the macro pstate.m produces the plot in Fig.6.1 and requires the control files pstate.in, which looks as follows:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

|theta.out; hko.out; sko.out; soil moisture; [-]; hillslope; 3; 2; 3600.

with:

theta.out	file holding state variable moisture

hko.out; sko.out	files holding coordinates of hillslope geometry (elevation and lateral position),

soil moisture; [-]	name and measurement unit of variable to be plotted

hillslope	 headline comment

3	 number of printout time to be plotted

2	 number of hill slope to be plotted

3600.	 conversion factor for computation time (optionally into sec, hours or days)

Please note that ; has to be used as delimiter!

Fig.6.1: Plot of the soil moisture distribution in hillslope 2 at the third printout time, produce with the macro pstate.m

### 2-d cross sections of state variables and water fluxes

In this case the MATLAB-macro pstate_fl.m (plot state variables & fluxes) and the macro calnp (calculate number of print out times) are used. As an example the following lines explain the visualisation of the soil moisture field stored in a file named theta.out for hillslope 2 at the 1. printout time (This output may be produced by running the demo project, Fig. 6.2):

copy the macros into your output directory e.g. into demo/out

run calnp.m with calnp(‘theta.out’) to find out the number of printout times (if not done yet)

Running the macro pstate_fl.m produces the plot in Fig.6.2 and requires the control files pstatefl.in, which looks as follows:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|

| theta.out;fl_xsi.out;fl_eta.out;hko.out;sko.out; soil moisture; [-]; hillslope; 1; 2; 3600.

with: 	theta.out	file holding state variable moisture

fl_xsi.out	file holding water flux densities in xsi direction

fl_eta.out	file holding water flux densities in eta direction

….	(same as above, Section 6.2.1)

Fig.6.2: Plot of the soil moisture distribution and water fluxes in hillslope 2 at the first printout time, produce with the macro pstate.m

## Areal maps using ARC/VIEW

2D areal plots can be produced using ARC/VIEW. First, a polygon coverage has to be produced, one polygon associated to each computational node. From the state variables' dump file, a table has to be produced which relates each state variable to one polygon. These files can be readily input by ARC/VIEWs interface to different database formats and be displayed.

## Time series

### Hydrographs for specified channel nodes

For the nodes specified in the drainage network geometry files, i.e. those with a negative channel node number, the hydrographs may be plotted using the MATLAB macro plotq_ch.m. The plotted hydrographs, which may be produced with the demo project, are shown in Fig. 6.3. The macro plotq_ch.m requires the control files plotq_ch.in, which looks as follows:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----

| gang.out;gauge;3600.

with: 	gang.out	file holding channel output

gauge	headline comment

3600.	 conversion factor for computation time (optionally into sec, hours or days)

Please note that ; has to be used as delimiter!

Fig.6.3: Hydrographs for specified channel nodes

### Surface runoff for individual hillslopes

Surface runoff may be visualised using the MATLAB macro plotq_s.m. The macro plotq_s.m requires the control files plotq_s.in, which looks as follows:

|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----|----.----

| qoben.out;hillslope;q_s;2;3600.

with:	qoben.out	file holding channel output

hillslope	headline comment

q_s	headline comment

2	number of hillslope to be plotted

3600.	 conversion factor for computation time (optionally into sec, hours or days)

Part II

Program Documentation

# Program structure

The sequence of operations carried out in the process modules implemented in CATFLOW is illustrated in the diagram below:

The program structure and the job which is done by individual subroutines are listed below:

-----------------------------------------------------------------------

>----, >=====	denotes computation loops,

<----, <=====	ih: indicates loop over all hillslopes

time: indicates time loop

-----------------------------------------------------------------------

catflow

relsec seconds from reference date

openi open input file

openo open output file

maxtst test parameter of dimensioning

rdsoil read parameters of soil-hydraulic-function-model

bodtab compute tables of soil-hydraulic functions

vg_tab van Genuchten (1980) model

ts_tab Tang & Skaggs (1977) model

filtab direct table input

rdrbf read filenames of boundary conditions, open them and wind up to start time

rdzrbf read subsequent point in time from boundary condition time series i

rdzsnk read subsequent point in time from source/sink time series i

rdzkli read subsequent point in time from climate time series i

rdznie read subsequent point in time from rainfall time series i

rdnutz read subsequent point in time from land use time series

gettn determine new time horizon (nearest time in any boundary condition file)

rdlpar read assignment of land use and related parameters from table

rdwind read wind-direction sectors

>------------ih start of input loop over all hillslopes

rdhang read geometry of hillslopes

rdstyp read soil types

rdmak read macropore data

rdcv read assignment of control volumes for watwer balance

calbna compute hillslope geometry and dump in BNA-format

calgeo compute diverse geometrical quantities of a hillslope

inicon read initial conditions for soil water dynamics

inipart read initial conditions for solute transport

rdprt read printout times

rdpob read assignment of surface-indices (land-use unit, rainfall gauge,  climatic data gauge)

rdrbs read assignment of boundary condition files to individual computational nodes;  initialize boundary-condition type  (B-C type: positive: Neumann, negative: Dirichlet)

getln assigning land-use units to land-use IDs

<------------ih end of input loop over all hillslopes

rdbach read drainage-network parameter

getbpt assigns nodes of the drainage network to hillslopes, optional if q_meth= simact

getrrb update of right boundary condition, optional if q_meth= simact

rdbini read initial state of drainage network, optional if q_meth= simact

calflek weight hydraulic conductivity of nodes a the right hillslope boundary with leakage factor, optional if q_meth= simact

rdrbb reads assignment of time series input fluxes to boundary nodes of the drainage network

rdbrbf reads next value from time series of input fluxes

upvorz update B-C type (sign) after reading new record

upvors update sink type (sign) after reading new record

>============Zeit start of time loop

gettn determine new time horizon (smallest of all B-C time series)

>------------ih start of computation loop over all hillslopes

hg (meth=pic) simulation of one hillslope up to the time horizon

etintz evapotranspiration

etidir climate data typ 2 (Penman-Monteith)

kolle climate data typ 1 (direct input of fluxes)

lookup table lookup of land-use parameters

strahl interpolation of climate data

sunpos computation of solar position

wicht1 subdivision of net energy between soil and canopy

druck computation of air pressure

uschub friction velocity

nstab class of atmospheric stability

ratmos atmospheric resistance

rblp atmospheric boundary layer resistance above plants

penmon vaporation of interception

s_e_sat gradient of saturation vapor pressure

rho_a specific mass of air

e_sat saturation vapor pressure

e_sat saturation vapor pressure

lam_v latent heat of vaporization of water

psychro psychrometric constant

lam_v latent heat of vaporization of water

rcanop Pflanzenwiderstand

wicht1 empirical function

wicht2 empirical function

e_sat saturation vapor pressure

penmon transpiration

rbls atmospheric boundary layer resistance above soil

rsoil soil resistance

wicht1 empirical function

penmon evaporation

updyo surface runoff computation

piccg single time step by Picard-Iterat. and CG-method

koeff computation of coefficients of numerical scheme

calfak computation of coefficients of numerical scheme

kinnen -at inner nodes

koeffrb -considering boundary conditions

ksenken -considering sinks/sources

pic_it preparation of itertation scheme

cg_solv conjungated-gradient (CG)-method

linbcg according to Numerical Recipes, 2.edn

asolve approximate solution

atimes multiplication...

sprsax ...of a matrix by a vector

sprstx ...of a transposed matrix by a vector

rand_fl computation of boundary fluxes at Dirichlet boundaries

chko_rb check of switching of boundary conditions (sign) and sinks

kc_phi updating of K and C

cal_q computation of fluxes inside the domain

stpdif change within time step

stpbil water balance after time step

mtheta mean soil moisture

hgcopy saving phineu->phialt, theta->th_alt

obcopy saving yoben->yo_alt, interz->iz_alt

ptkinj2 injection of solute particles in case of a flux Boundary condition for solute transport

v_strb computation of generalised pore water velocities

p_stepb computation of a particle tracking step

v_intb interpolation of pore water velocities

ran2 generation of random numbers for particle step

ran1 generation of random numbers for particle step

pmass computation of actual particle mass due to degradation of solutes

c_ipob computation of residual solute concentration

savevz saving of BC-type (sign)

totbil total balance

wrres output of results of current time step

<------------ih start of computation loop over all hillslopes

koeffbr update of computation of input flux into boundary nodes of the drainage network

bach comutation of drainage network until a steady discharge is reached

getnrb update of boundary conditions from time series

rdzrbf read next value from B-C time series i

upvorz update B-C type (sign) after reading new record

rdzsnk read next value from sink time series i

upvors update sink type (sign) after reading new record

rdzkli read next value from climate time series i

rdznie read next value from rainfall time series i

rdnutz read next value from land-use time series

getln assigning land-use units to land-use IDs

rdbrbf read next value form time series of input fluxes into the drainage network

getrrb update right hill slope boundary as a function of the height of the water table in the channels

<============time end of time loop

wrres output of results of current time step

# INCLUDE-files and COMMON-blocks

## Dim.inc

c-----------------------------------------------------------------------

c  DIM.INC           Dimensions of arrays

c-----------------------------------------------------------------------

integer*4 maxprt, maxin, maxout

integer*4 maxnv,maxnl,maxnh

integer*4 maxtyp, maxeig, maxpbm, maxtab

integer*4 maxfix, maxwrf, maxuse, maxpft, maxpfp

integer*4 maxnie, maxkli

integer*4 maxrbf, maxsnk

integer*4 maxrbp, maxskp, maxkld

integer*4 maxgr, maxfil

integer*4 maxcv

integer*4 maxnb, maxit, maxgng, maxhor, maxnum, maxero

integer*4 maxpkt

c----- neue Parameter fuers pt

integer*4 maxstt, maxmif

integer*4 npmax

integer*4 maxbrf

c-----------------------------------------------------------------------

c  control files

parameter (maxfil = 220)

c  input files

parameter (maxin  =   8)

c  output files geaendert

parameter (maxout =  17)

c  max. number of print-out times

parameter (maxprt = 3500)

c-----------------------------------------------------------------------

c  max. number of soil layers (eta)

parameter (maxnv = 25)

c       parameter (maxnv = 11)

c       parameter (maxnv =  8)

c       parameter (maxnv =  13)

c  max. number of soil columns (xsi)

c        parameter (maxnl = 100)

parameter (maxnl = 34)

c       parameter (maxnl = 14)

c       parameter (maxnl = 60)

c  max. number of hillslopes

parameter (maxnh = 20)

c       parameter (maxnh = 169)

c  highest hillslope number (used as a name)

parameter (maxnum = 200)

c-----------------------------------------------------------------------

c  max. number of soil types (as assigned in soil-parameter file)

parameter (maxtyp = 35)

c  max. number of soil properties  (columns in soil table)

parameter (maxeig = 5)

c  max. number of soil specific erosion parameters

parameter (maxero = 3)

c  max. number of soil parameters of a model of soil hydraulic behaviour

parameter (maxpbm = 8)

c  max. length of soil table (rows in soil table)

parameter (maxtab = 1200)

c-----------------------------------------------------------------------

c  max. number of fixed integers to be assigned to a surface node

parameter (maxfix =  3)

c  max. number of wind direction dependent factors

parameter (maxwrf =  4)

c  max. number of angle to horizon for a surface node

parameter (maxhor =  36)

c  max. number of land uses

parameter (maxuse = 20)

c  max. length of land-use time series (rows in land-use table)

parameter (maxpft = 25)

c  max. number of land-use parameters  (columns in land-use table)

parameter (maxpfp = 15)

c  max. number of rainfall time series

parameter (maxnie = 6)

c  max. number of climate time series

parameter (maxkli = 4)

c-----------------------------------------------------------------------

c  max. number of boundary condition time series

parameter (maxrbf = 5)

c  max. number of sink/source time series

parameter (maxsnk = 2)

c-----------------------------------------------------------------------

c  max. number of boundary condition parameters	geaendert 13.7.

parameter (maxrbp = 5)

c  max. number of sink/source parameters

parameter (maxskp = 2)

c  max. number of climatic data in climate time series (min. number = 6)

parameter (maxkld = 6)

c-----------------------------------------------------------------------

c  max. number of classes for time step statistics

parameter (maxgr = 15)

c-----------------------------------------------------------------------

c  max. number of control volumes for hillslope water balance

parameter (maxcv = 10)

c-----------------------------------------------------------------------

c  max. number of discrete drainage network reaches

parameter (maxnb = 1000)

c  max. number of inflows from upstream (confluences)

parameter (maxit =    3)

c  max. number of output hydrographs for output

parameter (maxgng=   10)

c  max. number of soil moisture time series for output

parameter (maxpkt=   70)

c  max. number of particles for particle tracking, global

parameter (npmax= 10000)

c  max. number of different solute types,

c  must match the maximal number of boundary parameters minus one

parameter (maxstt =3)

c  max number of mass inputs, must match the max. number of

c  parameter files maxrbf

parameter (maxmif =5)

c  max number of boundary files for the drainage network

parameter(maxbrf =3)

c  counter

integer*4 icf

c  global hillslope surface roughness for one project (can be overriden susequently)

real*8 kstall

dimension kstall(maxfil)

common /ckst/  kstall, icf

c  iteration criteria

real*8 piceps, cgeps

c  max. number of Picard iterations

c  number of Picard iterations, from which subsequent time step is reduced

integer*4 it_max, n_gr

common /cepsi/ it_max, n_gr

common /cepsr/ piceps, cgeps

c  switch is true, when surface attributes are read

c  switch is true, when land use is considered

logical lpob, lland

dimension lpob(maxnh)

common /catm/ lpob, lland

c  several input/output units

integer*4 ilok, ipar

integer*4 io, iin, io_log

dimension io(maxout), io_log(maxout)

dimension iin(maxin)

integer*4 io_act, iin_act

integer*4 iikli, iinie, iisnk, iirbf, inutz

dimension iikli(maxkli)

dimension iinie(maxnie)

dimension iisnk(maxsnk)

dimension iirbf(maxrbf)

common /cio/ io, iin, iikli, iinie, iisnk, iirbf,

&             ilok, ipar, inutz,

&             io_act, iin_act, io_log

c  max. and min. soil moisture within a hillslope

real*8 th_max, th_min

c  max. and min. suction head within a hillslope

real*8 psi_max, psi_min

c  max. and min. total potential within a hillslope

real*8 phi_max, phi_min

c  max. flux within a hillslope

c  max. sink/source flux within a hillslope

real*8 fl_max, sk_max

dimension th_max(maxnh), th_min(maxnh)

dimension psi_max(maxnh), psi_min(maxnh)

dimension phi_max(maxnh), phi_min(maxnh)

dimension fl_max(maxnh), sk_max(maxnh)

common /cmima/ th_max, th_min, phi_max, phi_min,

&               psi_max, psi_min, fl_max, sk_max

## Zeit.inc

c-----------------------------------------------------------------------

c  ZEIT.INC           times

c-----------------------------------------------------------------------

c... t_start     global starting point in time [s]

c... t_end       global end point in time [s]

c... t_alt       local starting point in time [s]

c... t_neu       local end point in time [s]

c... t_act       actual simulation point in time [s]

c... dt_exp      max. explicit time step [s]

c... dt_min      global min. time step [s]

c... dt_max      global max. time step [s]

c... dtbach      max. timestep for active drainage network [s]

c... itag        number of day in year [s]

c... stunde      hour of day [s]

c.....dt_mak     max. timestep for active macropore flow

real*8 t_start, t_end, t_alt, t_neu, t_act

real*8 dt_exp, dt_min, dt_max, dtbach

real*8 itag, stunde, dt_mak

c... counter for time steps

integer*4  izaehl

dimension  izaehl(maxnh)

c... date strings: start time, end time

character*22 dstrs, dstre, dstrs2

c... print times

real*8     t_p

c... counter

integer*4    ip

c... switch for output control

integer*4    it_p

dimension  t_p(maxprt,maxnh), ip(maxnh)

dimension  it_p(maxprt,maxnh)

c...statistics and histogram of time steps

c... boundary between classes of time step length

real*8     clgr

c... cumulative time within one time step class

real*8     cltim

c... cumulative number of timesteps within one time step class

integer*4  class

c... actual number of time step class boundaries

integer*4    iacgr

dimension  clgr(maxgr), cltim(maxgr+1), class(maxgr+1)

common /czeitr/ t_start, t_end, t_alt, t_neu, t_act,

&               dt_exp, dt_min, dt_max, dtbach, itag, stunde,

&               t_p, clgr, cltim,  dt_mak

common /czeiti/ class, iacgr,it_p, ip, izaehl

common /czeitl/ dstrs, dstre, dstrs2

## Soil.inc

c-----------------------------------------------------------------------

c  SOIL.INC           soil properties etc.

c-----------------------------------------------------------------------

c  boundary between arithmetric and geometric averaging for

c  Van Genuchtens n (after Zurmuehl)

real*8 vg_ngr

parameter (vg_ngr = 2.)

c  storage coefficient for saturated soil

real*8 snull

c  soil-albedo-parameter: dry conditions:            balmax

c                         from satkni * saturation:  balmin

real*8 satkni,balmax,balmin

c  soil resistance parameter (Kolle-model)

real*8 f_rs,wp_rs,zd_max

c  soil erosion parameter (Schramm-Model, Scherer-Model ?)

real*8 eross

c  soil hydraulic table

real*8 s_tab

c  soil parameters

real*8 bodpar

c  soil anisotropy

real*8 anisot

c  permanent wilting point [Vol.-%]

real*8 th_pwp

c  integer*4 for method to convert soil parameters to soil table

integer*4 imod

c  soil describing string

character*30 boden

c  name of a input file holding a table of soil hydraulic properties

character*30 tabfil

c  actual dimensions of soil table: soiltypes, rows, columns

integer*4 iactyp, iactab, iaceig

dimension s_tab(maxtab,maxeig,maxtyp)

dimension bodpar(maxpbm,maxtyp)

dimension anisot(2,maxtyp)

dimension imod(maxtyp)

dimension boden(maxtyp)

dimension th_pwp(maxtyp)

dimension snull(maxtyp)

dimension satkni(maxtyp), balmax(maxtyp), balmin(maxtyp)

dimension f_rs(maxtyp), wp_rs(maxtyp), zd_max(maxtyp)

dimension eross(maxero,maxtyp)

dimension iactab(maxtyp)

dimension tabfil(maxtyp)

common /soilr/ s_tab, bodpar, snull, anisot, th_pwp,

&       satkni,balmax,balmin,eross,f_rs,wp_rs,zd_max

common /soili/ iactyp, iactab,iaceig,imod

common /soill/ boden, tabfil

## Hgfest.inc

c-----------------------------------------------------------------------

c  HGFEST.INC       time independent variables hillslope

c-----------------------------------------------------------------------

c  rlongi reference longitude of time zone (CET: 15 degree east of Greenwich)

c  longi  longitude of catchment

c  lati   latitude of catchment

real*8 rlongi, longi, lati

real*8 varbr, fbrup, fbrlow

real*8 f_xsi, f_eta, fmf, fdf

real*8 xsi, eta, x_p1m0, x_p1m1, e_p1m0, e_p1m1

real*8 xko, yko, hko, sko, xkobez, ykobez, hkobez

real*8 hgobfl, hgbreit, hglang, lambda

real*8 macro, depth, area, xyxpy, alla, vola, hkomin, amak

real*8 slopeo, slopeu, sloper, slopel, gefall, azimut

real*8 av_fak, al_fak

real*8 dr_u, dr_o, dr_l, dr_r, drxsi, dreta

real*8 lrd_u, lrd_o, lrd_l, lrd_r

real*8 kxx, kee, kxe, w_hohr, w_xshr, w_xsho, w_xshol

real*8 d_Th_opt, d_Phi_opt

real*8 kxxf, kxef, keef

real*8 fcabf(maxnh), fabf(maxnh), flaeche

integer*4 iboden, mm_xsi, mm_eta

integer*4 ivm, igewkn

integer*4 icvo, icvu, icvl, icvr

integer*4 hangnr, hgih

integer*4 ihgr

integer*4 iacnh, m_aniso

integer*4 iacnv(maxnh), iacnl(maxnh)

integer*4 iaccv(maxnh)

logical lmak

c  metric coefficients (conversion to curvelinear coordinates) xsi-direction

dimension  f_xsi(maxnv,maxnl,maxnh)

c  metric coefficients (conversion to curvelinear coordinates) eta-direction

dimension  f_eta(maxnv,maxnl,maxnh)

c  tensor of anisotropy (normed with saturated conductivity k_s)

dimension  kxx(maxnv,maxnl,maxnh)

dimension  kee(maxnv,maxnl,maxnh)

dimension  kxe(maxnv,maxnl,maxnh)

dimension  kxxf(maxnv,maxnl,maxnh)

dimension  keef(maxnv,maxnl,maxnh)

dimension  kxef(maxnv,maxnl,maxnh)

c angle between xsi-direction and main direction of anisotropy, positiv counterclockwise

dimension  w_xshr(maxnv,maxnl)

c angle between xsi-direction and horizontal line, positiv counterclockwise,

c depends only on coordinate system

dimension  w_xsho(maxnv,maxnl), w_xshol(maxnv,maxnl,maxnh)

c angle between horizontal line and main direction of anisotropy, positiv counterclockwise

dimension  w_hohr(maxnv,maxnl)

c xsi-coordinate

dimension  xsi(maxnl,maxnh), flaeche(maxnv,maxnl,maxnh)

c

c    1                    il-1       il                   il+1

c    o------------o--....--o---------o----------------------o-------o---....

c    |      |                   |                |

c    |<---->|                   |<--dr_o(il,?)-->|

c    dr_o(1,?)                     drxsi(?,il,?)

c

c real length in [m] (related interval to a point)

c  dr_o: along surface boundary

c  dr_u: along bottom  boundary

c  drxsi: all lateral distances

dimension  dr_o(maxnl,maxnh)

dimension  dr_u(maxnl,maxnh)

dimension  drxsi(maxnv,maxnl,maxnh)

c xsi(il+1)-xsi(il)

dimension  x_p1m0(maxnl-1,maxnh)

c xsi(il+1)-xsi(il-1)

dimension  x_p1m1(maxnl-2,maxnh)

c eta-coordinate

dimension  eta(maxnv,maxnh)

c

c    1                    iv-1       iv                   iv+1

c    o------------o--....--o---------o----------------------o-------o---....

c    |      |                   |                |

c    |<---->|                   |<--dr_r(iv,?)-->|

c    dr_r(1,?)                     drxsi(?,iv,?)

c

c real length in [m] (related interval to a point)

c  dr_r: along right boundary

c  dr_l: along left  boundary

c  dreta: all vertical distances

dimension  dr_l(maxnv,maxnh)

dimension  dr_r(maxnv,maxnh)

dimension  dreta(maxnv,maxnl,maxnh)

c eta(iv+1)-eta(iv)

dimension  e_p1m0(maxnv-1,maxnh)

c eta(iv+1)-eta(iv-1)

dimension  e_p1m1(maxnv-2,maxnh)

c lateral hillslope coordinates of computational nodes (grid points) in [m]

dimension  sko(maxnv,maxnl,maxnh)

c horizontal global coordinates ("Rechtswert") in [m]

dimension  xko(maxnl)

c reference coordinate ("Rechtswert") in [m]

dimension  xkobez(maxnh)

c horizontal global coordinates ("Hochwert") in [m]

dimension  yko(maxnl)

c reference coordinates ("Hochwert") in [m]

dimension  ykobez(maxnh)

c vertical hillslope coordinates of grid points in [m+above reference level]

dimension  hko(maxnv,maxnl,maxnh)

c vertical hillslope coordinates reference level in [m]

dimension  hkobez(maxnh)

c depth under surface

dimension  depth(maxnv,maxnl,maxnh)

c true, if macropore information is read from file

dimension  lmak(maxnh)

c macropore factor

dimension  macro(maxnv,maxnl,maxnh), amak(maxnv,maxnl,maxnh)

c vertical index of deepest macroporous element

dimension  ivm(maxnl,maxnh)

c tangens of angle between surface and horizontal

dimension  gefall(maxnl,maxnh)

c azimut (orientation) of hillslope line (clockwise from north)

dimension  azimut(maxnl,maxnh)

c cosine of angle between surface and horizontal

c   slopeo: top

c   slopeu: bottom

c   sloper: right

c   slopel: left

dimension  slopeo(maxnl,maxnh)

dimension  slopeu(maxnl,maxnh)

dimension  sloper(maxnv,maxnh)

dimension  slopel(maxnv,maxnh)

c index of soil type

dimension  iboden(maxnv,maxnl,maxnh)

c method of averaging conductivity between two elements

dimension  mm_xsi(maxnv,maxnl-1,maxnh)

dimension  mm_eta(maxnv-1,maxnl,maxnh)

c related element area to a computational node

dimension  area(maxnv,maxnl,maxnh)

c weighting factor, used when computing mean moisture of two elements

dimension  al_fak(maxnv,maxnl-1,maxnh)

dimension  av_fak(maxnv-1,maxnl,maxnh)

c geometrical quantity used for computation of diffusion number (DIFFCAL, not activ anymore)

c      dimension  xyxpy(maxnv,maxnl,maxnh)

c area of longitudinal section of entire hillslope and (sub)-control-volumes

dimension  alla(maxcv,maxnh)

c volume of entire hillslope and (sub)-control-volumes (variable width)

dimension  vola(maxcv,maxnh)

c total length of boundaries along hillslope

c   lrd_o: top

c   lrd_u: bottom

c   lrd_r: right

c   lrd_l: left

dimension  lrd_o(maxnh)

dimension  lrd_u(maxnh)

dimension  lrd_l(maxnh)

dimension  lrd_r(maxnh)

c total surface area of hillslope

dimension  hgobfl(maxnh)

c variable width of hillslope

dimension  varbr(maxnl,maxnh)

c factors for use in numerical scheme, build from variable width of hillslope

dimension  fbrup(maxnl,maxnh)

c factors for use in numerical scheme, build from variable width of hillslope

dimension  fbrlow(maxnl,maxnh)

c mean width of hillslope

dimension  hgbreit(maxnh)

c length of hillslope

dimension  hglang(maxnh)

c reference level of internal used vertical coordinates

dimension  hkomin(maxnh)

c number of hillslopes (like a name)

dimension hangnr(maxnh)

c position of hillslope number

dimension hgih(maxnum)

c leakage-parameter (k'/d') of channel at hillfoot

dimension lambda(maxnh)

c number of channel reach, where hillslope runoff (1) and sealed surface (2)

c contribute

dimension igewkn(2,maxnh)

c indices of corners of control volumes for water balance

c   icv_o: top

c   icv_u: bottom

c   icv_r: right

c   icv_l: left

dimension  icvo(maxcv,maxnh), icvu(maxcv,maxnh)

dimension  icvl(maxcv,maxnh), icvr(maxcv,maxnh)

c      common /hgfest/

c     &  f_xsi,f_eta,fmf,fdf,varbr,fbrup,fbrlow,vola,

c     &  xsi,eta,x_p1m0,x_p1m1,e_p1m0,e_p1m1,

c     &  sko,hko,iboden,area,xyxpy,alla,hkomin,iacnv,iacnl,iacnh,ihgr,

c     &  kxx,kee,kxe,w_hohr,w_xshr,w_xsho,w_xshol,mm_xsi,mm_eta,

c     &  av_fak, al_fak, fcabf, fabf,

c     &  d_Th_opt, d_Phi_opt, hangnr, hgih, lmak,

c     &  macro, ivm, depth, xko, yko, xkobez, ykobez, hkobez,

c     &  slopeo, slopeu, sloper, slopel, gefall, azimut,

c     &  hgobfl, hgbreit, hglang, lambda, igewkn,

c     &  dr_u, dr_o, dr_l, dr_r, drxsi, dreta,

c     &  lrd_u, lrd_o, lrd_l, lrd_r,

c     &  iaccv, icvo, icvu, icvl, icvr,

c     &  rlongi, longi, lati, kxxf, keef, kxef, amak

common /hfestr/

& rlongi, longi, lati,

& varbr, fbrup, fbrlow,

& f_xsi, f_eta, fmf, fdf,

& xsi, eta, x_p1m0, x_p1m1, e_p1m0, e_p1m1,

& xko, yko, hko, sko, xkobez, ykobez, hkobez,

& hgobfl, hgbreit, hglang, lambda,

& macro, depth, area, xyxpy, alla, vola, hkomin, amak,

& slopeo, slopeu, sloper, slopel, gefall, azimut,

& av_fak, al_fak,

& dr_u, dr_o, dr_l, dr_r, drxsi, dreta,

& lrd_u, lrd_o, lrd_l, lrd_r,

& kxx, kee, kxe, w_hohr, w_xshr, w_xsho, w_xshol,

& d_Th_opt, d_Phi_opt,

& kxxf, kxef, keef, fcabf, fabf, flaeche

common /hfesti/

& iboden, mm_xsi, mm_eta,

& ivm, igewkn, iaccv,

& icvo, icvu, icvl, icvr,

& hangnr, hgih,

& ihgr, iacnh, iacnv, iacnl, m_aniso

common /hfestl/ lmak

## Hgvari.inc

c-----------------------------------------------------------------------

c  HGVARI.INC       time dependent hillslope variables

c-----------------------------------------------------------------------

real*8 Fx_p1,Fx_00,Fx_m1,Fe_p1,Fe_00,Fe_m1

real*8 RS,vorfak,A_x,A_e,psi,durchl,wasska,diffus

real*8 A2x,A2e

real*8 rfl_u, rfl_o, rfl_r, rfl_l

real*8 ueb_u, ueb_o, ueb_r, ueb_l

real*8 phiini, phialt, phineu

real*8 theta, th_ini, th_alt, senk, sueb

real*8 hwsp, yoben, yo_alt

real*8 qomax, qosum, qssum, q

real*8 th_mit, yo_mit

real*8 q_eta, q_xsi, qxalt, qealt

integer*4 tabpos,ih1st

c... lrill     true, if rillflow is computed, currently fixed (false)

logical lrill, mak_an

c... lwriten   true, if rainfall has to be dumped to surface runoff output file

c... lwriteq   true, if runoff has to be dumped to surface runoff output file

c... lwrites   true, if runoff has to be dumped, ensures one more value to be dumped

c              after event ended

c... lw1st     true, as long as no surface runoff output has been dumped

c... lw11st    true, as long header line has not been dumped to surface runoff file

c              (factors fabf, fcabf)

c... ih1st     number of that hillslope, which first produces surface runoff

logical lwriten,lwriteq,lwrites,lw1st,lw11st

c... total incremental water balance within a time step, should be zero

real*8 bilanz

c... incremental flux terms across individual boundaries within a time step

real*8 vrfl_u, vrfl_o, vrfl_r, vrfl_l, vsenk

real*8 vueb_u, vueb_o, vueb_r, vueb_l, vsueb

real*8 vnied, vintz, vevapo, vtrans

c... cumulative flux terms across individual boundaries

c    since start of computation

real*8 brfl_u, brfl_o, brfl_r, brfl_l

real*8 bueb_u, bueb_o, bueb_r, bueb_l

real*8 bnied, bintz, bevapo, btrans

c... incremental flux terms across all boundaries within a time step

real*8 volrd

c... cumulative flux terms across all boundaries since start of computation

real*8 bilrd

c... cumulative flux terms from sinks/sources since start of computation

real*8 bsenk, bsueb

c... incremental change in total water content within a time step

real*8 volin

c... cumulative change in total water content since start of computation

real*8 bilin

c... total cumulativ balance (bilin-bilrd+bsenk), should be zero

real*8 biltot

c... coefficients of numerical scheme

dimension  Fx_p1(maxnv,maxnl)

dimension  Fx_00(maxnv,maxnl)

dimension  Fx_m1(maxnv,maxnl)

dimension  Fe_p1(maxnv,maxnl)

dimension  Fe_00(maxnv,maxnl)

dimension  Fe_m1(maxnv,maxnl)

dimension  RS(maxnv,maxnl)

dimension  vorfak(maxnv,maxnl)

dimension  A_x(maxnv,maxnl-1)

dimension  A_e(maxnv-1,maxnl)

dimension  A2x(maxnv,maxnl-1)

dimension  A2e(maxnv-1,maxnl)

c... fluxes in eta-direction

dimension  q_eta(maxnv,maxnl), qealt(maxnv,maxnl)

c... fluxes in xsi-direction

dimension  q_xsi(maxnv,maxnl), qxalt(maxnv,maxnl)

c... old state of the system (potential), to be used e.g. if iteration

c    fails and time step has to be repeated

dimension  phialt(maxnv,maxnl,maxnh)

c... new state of the system (potential)

dimension  phineu(maxnv,maxnl)

c... initial state of the system (potential)

dimension  phiini(maxnv,maxnl)

c... suction head

dimension  psi(maxnv,maxnl)

c... current position in soil hydraulic function table

dimension  tabpos(maxnv,maxnl,maxnh)

c... initial soil moisture

dimension  th_ini(maxnv,maxnl)

c... actual soil moisture

dimension  theta(maxnv,maxnl)

c... old soil moisture

dimension  th_alt(maxnv,maxnl)

c... actual conductivity

dimension  durchl(maxnv,maxnl)

c... actual water capacity

dimension  wasska(maxnv,maxnl)

c... actual diffusion number

dimension  diffus(maxnv,maxnl)

c... true, if macropores are active

dimension  mak_an(maxnv,maxnl)

c... sink terms  [1/sec]

dimension  senk(maxnv,maxnl)

dimension  sueb(maxnv,maxnl)

c... elevation of water level at hillfoot

dimension  hwsp(maxnh)

c... depth of surface runoff (vertical, while local variable yo in UPDYO

c    is perpendicular to surface)

dimension  yoben(maxnl)

c... old depth of surface runoff

dimension  yo_alt(maxnl,maxnh)

c... true, if rillflow is computed, currently fixed (false)

dimension  lrill(maxnl,maxnh)

c... max. runoff at hillfoot since start of simulation

dimension  qomax(maxnh)

c... cumulative surace runoff and interflow at hillfoot since start of simulation

dimension  qosum(maxnh), qssum(maxnh)

c... actual runoff at each surface node

dimension  q(maxnl,maxnh)

c... true, as long as no output has been dumped

dimension  lw1st(maxnh)

c... boundary fluxes

dimension  rfl_u(maxnl)

dimension  rfl_o(maxnl)

dimension  rfl_r(maxnv)

dimension  rfl_l(maxnv)

c... boundary flux infiltration excess (potentially)

dimension  ueb_u(maxnl)

dimension  ueb_o(maxnl)

dimension  ueb_r(maxnv)

dimension  ueb_l(maxnv)

c old and new mean soil moisture

dimension  th_mit(maxcv,2,maxnh)

c old and new mean depth of surface runoff on hillslope

dimension  yo_mit(2,maxnh)

c... total cumulativ balance (bilin-bilrd+bsenk), should be zero

dimension  biltot(maxcv,maxnh)

c... cumulative change in total water content since start of computation

dimension  bilin(maxcv,maxnh)

c... cumulative flux terms across all boundaries since start of computation

dimension  bilrd(maxcv,maxnh)

c... cumulative flux terms from sinks/sources since start of computation

dimension  bsenk(maxcv,maxnh)

dimension  bsueb(maxcv,maxnh)

c... cumulative flux terms across individual boundaries

c    since start of computation

dimension  brfl_u(maxcv,maxnh)

dimension  brfl_o(maxcv,maxnh)

dimension  brfl_r(maxcv,maxnh)

dimension  brfl_l(maxcv,maxnh)

dimension  bueb_u(maxnh)

dimension  bueb_o(maxnh)

dimension  bueb_r(maxnh)

dimension  bueb_l(maxnh)

dimension  bnied(maxcv,maxnh)

dimension  bintz(maxcv,maxnh)

dimension  bevapo(maxcv,maxnh)

dimension  btrans(maxcv,maxnh)

c... incremental flux terms across individual boundaries within a time step

dimension  vrfl_u(maxcv), vrfl_o(maxcv)

dimension  vrfl_r(maxcv), vrfl_l(maxcv)

dimension  vsenk(maxcv),  vsueb(maxcv)

dimension  vnied(maxcv),  vintz(maxcv)

dimension  vevapo(maxcv), vtrans(maxcv)

c... incremental flux terms across all boundaries within a time step

dimension  volrd(maxcv)

c... incremental change in total water content within a time step

dimension  volin(maxcv)

c... total incremental water balance within a time step, should be zero

dimension  bilanz(maxcv)

common /hgvarr/

&  Fx_p1,Fx_00,Fx_m1,Fe_p1,Fe_00,Fe_m1,

&  RS,vorfak,A_x,A_e,psi,durchl,wasska,diffus,

&  A2x,A2e,phiini,phialt,phineu,

&  theta,th_ini,th_alt,senk,sueb,

&  hwsp, yoben, yo_alt, rfl_u, rfl_o, rfl_r, rfl_l,

&  th_mit, yo_mit, ueb_u, ueb_o, ueb_r, ueb_l,

&  vrfl_u, vrfl_o, vrfl_r, vrfl_l, vsenk, bilanz,

&  vueb_u, vueb_o, vueb_r, vueb_l, vsueb,

&  vnied, vintz, vevapo, vtrans,

&  brfl_u, brfl_o, brfl_r, brfl_l, bsenk,

&  bueb_u, bueb_o, bueb_r, bueb_l, bsueb,

&  bnied, bintz, bevapo, btrans,

&  volrd, bilrd, volin, bilin, biltot,

&  q_eta, q_xsi, qxalt, qealt, qomax, qosum, q,

&  qssum

common /hgvari/ tabpos,ih1st

common /hgvarl/

& lwriten,lwriteq,lwrites,lw1st,lw11st, lrill, mak_an

## Hgbdry.inc

c-----------------------------------------------------------------------

c  HGBDRY.INC      boundary conditions

c-----------------------------------------------------------------------

integer*4    irbtyp, isktyp, ktyp, iackld

integer*4    irb_u, irb_o, irb_r, irb_l, isnk

integer*4    vorz_u, vorz_o, vorz_r, vorz_l, vorz_s

integer*4    vsav_u, vsav_o, vsav_r, vsav_l, vsav_s

integer*4    hor

integer*4    ifixob, ivroot

integer*4    iacuse, iacpft, iacwrf, iacfix, iacpfp, iachor

integer*4    iusenr, luseid

integer*4    iacnie, iackli, iacsnk, iacrbf, rBilart

real*8       rbpar, skpar, klima, klimas, zref, nied

real*8       qu_pot, qo_pot, qr_pot, ql_pot, qs_pot

real*8       pu_pot, po_pot, pr_pot, pl_pot, ps_pot

real*8       wrf, wru, wro, bezfak

real*8       neff, interz, iz_alt

real*8       Esoil, Eintz, Ecanop, plrw

real*8       t_ln, pflpar

real*8       zrbf, z2srbf, szrbf

real*8       zsnk, z2ssnk, szsnk

real*8       zkli, z2skli, szkli

real*8       znie, z2snie, sznie, niefak, klifak

real*8       sw0,sw1,sw2,trueb,truebf, sattgr

character*30 lnfile

character*30 n_fil, k_fil

character*30 rbfil, snkfil

character*30 cluse

logical      lueb_u, lueb_o, lueb_r, lueb_l, lueb_s

logical      lrintp,lsintp,lnintp,lklima,lnied,etilog

logical      klfehlt

c  actual type of boundary condition  (mathematical)

dimension  irbtyp(2,maxrbf)

c  actual type of sink/source boundary condition  (mathematical)

dimension  isktyp(2,maxsnk)

c  associated actual B-C parameters

dimension  rbpar(2,maxrbp,maxrbf)

c  associated actual sink/source B-C parameters

dimension  skpar(2,maxskp,maxsnk)

c  actual climate data interval

dimension  klima(2,maxkld,maxkli)

c  actual climate data (saved)

dimension  klimas(maxkld,maxkli)

c  true, if climate data is missing (-999)

dimension  klfehlt(maxkli)

c  reference height of wind velocity measurement

dimension  zref(maxkli)

c  type of climatic data (1 or 2)

dimension  ktyp(maxkli)

c  parameter of regression between rBil and shortwave-Bil (ETINTZ, Kolle-model)

dimension  sw0(maxkli),sw1(maxkli),sw2(maxkli)

c  parameter for atmosperic dimming (ETINTZ, Kolle-model)

dimension  trueb(maxkli),truebf(maxkli)

c  actual read number of different climate data per record (time)

dimension  iackld(maxkli)

c  actual rainfall data interval

dimension  nied(2,maxnie)

c  boundary condition file index (bottom boundary)

dimension  irb_u(maxnl,maxnh)

c  boundary condition file index (surface boundary)

dimension  irb_o(maxnl,maxnh)

c  boundary condition file index (right boundary)

dimension  irb_r(maxnv,maxnh)

c  boundary condition file index (left boundary)

dimension  irb_l(maxnv,maxnh)

c  sink/source boundary condition file index (internal boundary)

dimension  isnk(maxnv,maxnl,maxnh)

c  sign of boundary condition (-1: potential,  1: flux) and saved values

dimension  vorz_u(maxnl,maxnh)      ,  vsav_u(maxnl)

dimension  vorz_o(maxnl,maxnh)      ,  vsav_o(maxnl)

dimension  vorz_r(maxnv,maxnh)      ,  vsav_r(maxnv)

dimension  vorz_l(maxnv,maxnh)      ,  vsav_l(maxnv)

dimension  vorz_s(maxnv,maxnl,maxnh),  vsav_s(maxnv,maxnl)

c  true, if boundary flux cannot infiltrate totally (in case of unequality condition)

dimension  lueb_u(maxnl)

dimension  lueb_o(maxnl)

dimension  lueb_r(maxnv)

dimension  lueb_l(maxnv)

dimension  lueb_s(maxnv,maxnl)

c  potential fluxes/potentials (in case of unequality condition)

dimension  qu_pot(maxnl)      ,  pu_pot(maxnl)

dimension  qo_pot(maxnl)      ,  po_pot(maxnl)

dimension  qr_pot(maxnv)      ,  pr_pot(maxnv)

dimension  ql_pot(maxnv)      ,  pl_pot(maxnv)

dimension  qs_pot(maxnv,maxnl),  ps_pot(maxnv,maxnl)

c  effective (net) rainfall

dimension  neff(maxnl)

c  share of total plant reachable water within current element (0...1)

dimension  plrw(maxnv,maxnl)

c  vaporation of interception [m/s]

dimension  Eintz(maxnl)

c  evaporation [m/s]

dimension  Esoil(maxnl)

c  transpiration [m/s]

dimension  Ecanop(maxnl)

c  interception [m] and saved value of state variable

dimension  interz(maxnl)

dimension  iz_alt(maxnl,maxnh)

c  boundary condition filenames

dimension  rbfil(maxrbf), snkfil(maxsnk)

c  true, if interpolation between subsequent boundary condition values is desired

dimension  lrintp(maxrbf), lsintp(maxsnk)

c  number of vertical element at rooting depth

dimension  ivroot(maxnl)

c  array of surface indices

dimension  ifixob(maxnl,maxfix,maxnh)

c  array of wind reduction factors assigned to surface nodes

dimension  wrf(maxnl,maxwrf,maxnh)

c  array of angle on horizon assigned to surface nodes

dimension  hor(maxnl,maxhor,maxnh)

c  reference wind factors at climate stations

dimension  bezfak(maxwrf,maxkli)

c  boundaries of wind direction sectors (lower and upper value, in degree)

dimension  wru(maxwrf), wro(maxwrf)

c  land-use number of a surface node

dimension  iusenr(maxnl,maxnh)

c  land-use descriptive string

dimension  cluse(maxuse)

c  land-use parameter table

dimension  pflpar(maxpft,maxpfp+1,maxuse)

c  actual length of time series of landuse parameters

dimension  iacpft(maxuse)

c  IDs of land uses

dimension  luseid(maxuse)

c  rainfall and climate filenames

dimension  n_fil(maxnie), k_fil(maxkli)

c  true, if interpolation between subsequent rainfall values is desired

dimension  lnintp(maxnie)

c  time series of land use assignment files

dimension  t_ln(2)

c  time series of boundary conditions

c    z___    time since t_start in [s]

c    z2s___  conversion factor file time unit -> seconds

c    sz___   offset between computation time and time in input file

dimension  zrbf(2,maxrbf)

dimension  z2srbf(maxrbf), szrbf(maxrbf)

dimension  zsnk(2,maxsnk)

dimension  z2ssnk(maxsnk), szsnk(maxsnk)

dimension  zkli(2,maxkli)

dimension  z2skli(maxkli), szkli(maxkli), klifak(maxkli)

dimension  znie(2,maxnie)

dimension  z2snie(maxnie), sznie(maxnie), niefak(maxnie)

c rBilart specifies whether the measured net radiation rBilm is used (2) or rBil is computed (1)

dimension rBilart(maxkli)

c      common /hgbdry/

c     &     irbtyp, isktyp, klfehlt,

c     &     rbpar, skpar, klima, klimas, zref, ktyp, iackld, nied,

c     &     wrf, wru, wro, bezfak, ifixob, ivroot,

c     &     t_ln, lnfile,

c     &     iacuse, iacpft, iacwrf, iacfix, iacpfp,

c     &     iusenr, luseid, cluse, pflpar,

c     &     n_fil, k_fil, iacnie, iackli,

c     &     irb_u, irb_o, irb_r, irb_l, isnk,

c     &     vorz_u, vorz_o, vorz_r, vorz_l, vorz_s,

c     &     vsav_u, vsav_o, vsav_r, vsav_l, vsav_s,

c     &     lueb_u, lueb_o, lueb_r, lueb_l, lueb_s,

c     &     qu_pot, qo_pot, qr_pot, ql_pot, qs_pot,

c     &     pu_pot, po_pot, pr_pot, pl_pot, ps_pot,

c     &     neff, interz, iz_alt, sattgr,

c     &     Esoil, Eintz, Ecanop, plrw,

c     &     rbfil, snkfil, iacrbf, iacsnk,

c     &     lrintp, lsintp, lnintp, lklima, lnied, etilog

c      common /hgrb_t/

c     &      zrbf,  zsnk,  zkli,  znie,

c     &      z2srbf,z2ssnk,z2skli,z2snie,

c     &      szrbf, szsnk, szkli, sznie, niefak, klifak,

c     &      sw0,sw1,sw2,trueb,truebf,hor,iachor

common /hgrbi/

&    irbtyp, isktyp, ktyp, iackld,

&    irb_u, irb_o, irb_r, irb_l, isnk,

&    vorz_u, vorz_o, vorz_r, vorz_l, vorz_s,

&    vsav_u, vsav_o, vsav_r, vsav_l, vsav_s,

&    hor,ifixob, ivroot, rbilart

&    iacuse, iacpft, iacwrf, iacfix, iacpfp, iachor,

&    iusenr, luseid, iacnie, iackli, iacsnk, iacrbf

common /hgrbr/

&       rbpar, skpar, klima, klimas, zref, nied,

&       qu_pot, qo_pot, qr_pot, ql_pot, qs_pot,

&       pu_pot, po_pot, pr_pot, pl_pot, ps_pot,

&       wrf, wru, wro, bezfak,

&       neff, interz, iz_alt,

&       Esoil, Eintz, Ecanop, plrw,

&       t_ln, pflpar, zrbf, z2srbf, szrbf,

&       zsnk, z2ssnk, szsnk, zkli, z2skli, szkli,

&       znie, z2snie, sznie, niefak, klifak,

&       sw0,sw1,sw2,trueb,truebf, sattgr

common /hgrbc/

& lnfile, n_fil, k_fil, rbfil, snkfil, cluse

common /hgrbl/

& lueb_u, lueb_o, lueb_r, lueb_l, lueb_s,

& lrintp,lsintp,lnintp,lklima,lnied,etilog,

& klfehlt

## Pfest.inc

c--------------------------------------------------------------------

c  pfest.inc time independent parameters for solute transport

c---------------------------------------------------------------------

c retardation faktor and degradation constant for different solute types,

c partikel mass, dispersion coefficient, global parameters

real*8  k_sorp,m_pt, abbau, d_koef

dimension k_sorp(maxstt,maxtyp)

dimension m_pt(maxstt,maxrbf,maxnh)

dimension abbau(maxstt,maxtyp)

dimension d_koef(maxstt,maxtyp)

common /pfest/

&  k_sorp, abbau, m_pt, d_koef

## Pvari.inc

c-----------------------------------------------------------

c time dependent variables for particle tracking

c-----------------------------------------------------------

real*8 hkpact, skpact, hkpalt, skpalt, xpact, epact

real*8 xpalt, epalt

real*8 vxact, veact, vxalt, vealt

real*8 vhact,vsact

real*8 vx_st, ve_st, vx_sta, ve_sta

real*8 gtxact, gteact, gtxalt, gtealt

real*8 r_ret

real*8 vxipo, veipo

real*8 c_pw, c_ptr, c_wact, c_tact

real*8 mpact, mpkon

real*8 cg_q, sig_cg, skew_cg, chs_q, sig_hs, skew_hs

real*8 kovc, kovcg, mkorrf,  cgz

integer*4 ixalt, ixact, ieact, iealt

integer*4 iplosr, iplosl, iplosu

integer*4 nplosl, nplosu, nplosr

integer*4 npkon

c----actual and old position of particles in the cartesian grid

dimension hkpact(maxstt,npmax,maxnh),skpact(maxstt,npmax,maxnh)

dimension hkpalt(maxstt,npmax,maxnh),skpalt(maxstt,npmax,maxnh)

c----- actual and old generalised pore water velocites in the cartesian grid---------

dimension vhact(maxnv, maxnl), vsact(maxnv,maxnl)

c-----actual and old position of particles in curivlinear coordiantes

dimension xpact(maxstt,npmax,maxnh), epact(maxstt,npmax,maxnh)

dimension xpalt(maxstt,npmax,maxnh), epalt(maxstt,npmax,maxnh)

c-----actual and old index locating the number of the grid cell associated to the particle position

c     Konvention ixact, ieact bezeichen iv bzw. il des linken, oberen Knotens

dimension ixact(maxstt,npmax,maxnh),ieact(maxstt,npmax,maxnh)

dimension ixalt(maxstt,npmax,maxnh),iealt(maxstt,npmax,maxnh)

c---- actual and old generalised pore water velocites in the curivlinear grid

dimension vxact(maxnv,maxnl), veact(maxnv,maxnl)

dimension vealt(maxnv,maxnl), vxalt(maxnv,maxnl)

c---- actual and old moisture gradient

dimension gtxact(maxnv,maxnl), gteact(maxnv,maxnl)

dimension gtxalt(maxnv,maxnl), gtealt(maxnv,maxnl)

c---- actual and old  v'=v+D grad ln(theta)

dimension vx_st(maxnv,maxnl,maxnh),ve_st(maxnv,maxnl,maxnh)

dimension vx_sta(maxnv,maxnl,maxnh),ve_sta(maxnv,maxnl,maxnh)

c----- actual and old pore water velocities at the particle position

c      dimension vxipo(maxstt,npmax), veipo(maxstt,npmax)

c      dimension vxipa(maxstt,npmax), veipa(maxstt,npmax)

c----- number of lost particle at right, left and lower boundary

dimension nplosl(maxstt, maxnh), nplosr(maxstt,maxnh)

dimension nplosu(maxstt,maxnh)

c-----index of particles lost at right, left and lower boundary

dimension iplosr(maxstt,npmax), iplosl(maxstt,npmax)

dimension iplosu(maxstt,npmax)

c------ number of particles in a grid cell for concentration computation

dimension npkon(maxstt,maxnv,maxnl)

c------residual concentration in the water phase and dry phase without degradation

dimension c_pw(maxstt,maxnv-1,maxnl-1,maxnh)

dimension c_ptr(maxstt,maxnv-1,maxnl-1,maxnh)

c------residual concentration in the water phase and dry phase without degradation

dimension c_wact(maxstt,maxnv-1,maxnl-1,maxnh)

dimension c_tact(maxstt,maxnv-1,maxnl-1,maxnh)

dimension mkorrf(maxstt,maxnh)

c------ retardation factor--------------------------------------------

dimension r_ret(maxstt,maxnv,maxnl)

c------- actual parctile mass and actual mass of all particles in a grid cell --------------

dimension mpact(maxstt,npmax,maxnh), mpkon(maxstt,maxnv,maxnl)

c------ mean, standard deviation and skewnes of the solute center of mass in the soil

dimension cg_q(maxstt,maxnh), sig_cg(maxstt,maxnh)

dimension skew_cg(maxstt,maxnh)

c------ mean, standard deviation and skewnes of the solute center of mass in the soil of transport distance pdf

dimension chs_q(maxstt,maxnh),sig_hs(maxstt,maxnh)

dimension skew_hs(maxstt,maxnh)

c--------covarianz of solute concentrations

c        und der Schwerpunktstiefen

dimension kovc(maxstt, maxnv,maxnl,maxnh)

dimension kovcg(maxstt,maxnl,maxnh)

c-------- horizontal dsitribution of the solute center of mass in the grid columns

dimension cgz(maxstt,maxnl-1, maxnh)

common /pvari1/ hkpact, skpact, xpact, epact,

& xpalt, epalt, vxact, veact, vxalt, vealt, r_ret,

& skpalt, hkpalt, vhact, vsact, gtxact, gteact,

& gtxalt, gtealt, mpact,vx_st, ve_st, vx_sta, ve_sta,

& vxipo, veipo, c_pw, c_ptr, mpkon, c_wact, cg_q,

& sig_cg, skew_cg, chs_q, sig_hs,skew_hs, kovcg, kovc,

& c_tact, mkorrf, cgz

common /pvari2/ ixalt, ixact,ieact, iealt, nplosl,

& nplosr, nplosu, iplosl, iplosr, iplosu, npkon

## pbdry.inc

c---------------------------------------------------------------

c pbdry.inc boundary conditions for solute transport

c--------------------------------------------------------------

real*8 mp_inj, c_inj, cil_o, t_inj, t_injk

real*8 mp_ini, m_ges, cil_r, cil_u, cil_l

integer*4 npact,npalt, ninpot, ninact, np_ofl

integer*4  nposum, iinj, iinj1

integer*4 iseed

integer*4 iimif, iacmif,istact

character*30 minfil

character*8 strand

c input mass and concentration

dimension mp_inj(maxstt,maxrbf), c_inj(2,maxstt,maxrbf)

c  initial solute mass in a hillslope

dimension mp_ini(maxstt,maxnh)

c injected number of particles , actual and old number of particles in a hillslope

dimension npact(maxstt,maxnh), npalt(maxstt,maxnh)

c number of particles locally injected at a grid node

dimension ninact(maxstt,maxnl), ninpot(maxstt,maxnl)

c cumulative solute loss with surface runoff

dimension np_ofl(maxstt,maxnh), nposum(maxstt,maxnh)

c counter of  inputmass files

dimension iimif(maxmif)

c name of inputmass files

dimension minfil(maxmif)

c input concentration locally injected at a grid node at the upper boundary

dimension cil_o(maxstt,maxnl,maxnh)

c input concentration locally injected at a grid node at the lower boundary

dimension cil_u(maxstt,maxnl,maxnh)

c input concentration locally injected at a grid node at the left boundary

dimension cil_l(maxstt,maxnv,maxnh)

c input concentration locally injected at a grid node at the right boundary

dimension cil_r(maxstt,maxnv,maxnh)

c --- time increment for solute injection

dimension t_inj(0:30000)

c total solute mass in a hillslope

dimension m_ges(maxstt,maxnh)

common /pbo1/ mp_inj, c_inj, cil_o, t_inj, t_injk, mp_ini,

&         m_ges, cil_r, cil_l, cil_u

common /pbo2/ npact, npalt, istact, iimif, iacmif,

&        np_ofl, ninpot, ninact, nposum,iinj,iinj1, iseed

common /pbo3/ minfil, strand

## Bach.inc

c-----------------------------------------------------------------------

c  BACH.INC              drainage network

c-----------------------------------------------------------------------

integer*4  iacnb, ib_nr, igang, iacgng, ihgb

integer*4  ibnext, inhang, inarea, ib_ord, ib_reg, itmax, ibtop

integer*4   iacqbr, iqrbb,iqsour, iacbrf, ibrbf, irb_b

real*8   b_dx, b_gef, b_ezg, b_yso, b_vorl, y_vorl, y_bach

real*8   b_rauh, b_m, qhang, qarea, qbas, brso, v_bach, weg_br

real*8   qpegel, qpmax, dtsur, aahang, xba, yba

real*8   bapsi, stpsi(3), lapsi(3)

real*8   qtol, qshg, qsour, flek, szbach, zfbach, bwq, zbach

real*8   qshang

character*30 brbfil

c  tolerance for drainage network runoff computation in [cbm/s per s].

c  If acceleration of runoff is lower, runoff process is tempral neglected.

c  swich is true when drainage network runoff is active

logical  lqactiv

logical  exhg, inter, auint, rbbneu

c  number of actual node of drainage network

dimension ib_nr(maxnb)

c  node in the river system assocoiated with a hill slope

dimension ihgb(maxnh)

c  number of subsequent downstream node

dimension ibnext(maxnb)

c  number of incoming hillslope

dimension inhang(maxnb)

c  if > 0, contributing sealed surface is present

dimension inarea(maxnb)

c  STRAHLER-order

dimension ib_ord(maxnb)

c  rainfall-gauge number which contributes to runoff

c  (according to the numbering in input file for rainfall time series)

dimension ib_reg(maxnb)

c  length to next downstream node in [m]

dimension b_dx(maxnb)

c  width of sealed surface along channel

dimension weg_br(maxnb)

c  slope of drainage network reach

dimension b_gef(maxnb)

c  local catchment area of current node

dimension b_ezg(maxnb)

c  elevation of channel bottom

dimension b_yso(maxnb)

c  elevation of channel top

dimension b_vorl(maxnb)

c  horizontal coordinates

dimension xba(maxnb)

dimension yba(maxnb)

c  channel depth (bottom to top)

dimension y_vorl(maxnb)

c  actual runoff depth

dimension y_bach(maxnb)

c  bottom width of trapeziodial channel

dimension brso(maxnb)

c  wetted channel volume

dimension v_bach(maxnb)

c  max. number of inflowing upstream nodes ...

dimension itmax(maxnb)

c  ... and related channel reach numbers

dimension ibtop(maxnb,maxit)

c  surface area of linked hillslope

dimension aahang(maxnb)

c  leakage coefficient for interchange of subsurface and drainage water

c  iqsour indices whether a node is a boundary node (value 1) or not (value 0)

c  iqrbb pointer to the nodes specified as boundary nodes

c   ibrbf pointer to the boundary condition files

dimension flek(maxnb),iqrbb(maxnb),iqsour(maxnb), ibrbf(maxbrf)

dimension szbach(maxbrf), ibrart(maxbrf), zfbach(maxbrf)

dimension irb_b(maxnb), zbach(2,maxbrf)

c   bwq Boundary value for boundary (source) knotes of the drainage network

dimension bwq(2,maxbrf)

c  channel node numbers, where hydrographs will be output

dimension igang(maxgng)

c  channel roughness (Strickler)

dimension b_rauh(maxnb)

c  slope of wall of trapeziodial crossection (1:b_m vertical/horizontal)

dimension b_m(maxnb)

c  mean inflow from hillslope during time up to actual time horizon (in CATFLOW.FOR)

dimension qhang(maxnh), qshg(maxnh), qshang(maxnh)

c  true, after hillslope number (not counter ih, but hangnr(ih)) has been computed

dimension exhg(maxnum)

c  mean inflow from sealed surfaces during time up to actual time horizon (in CATFLOW.FOR)

dimension qarea(maxnb)

c  baseflow qbas per unit channel length, which has to be given as a boundary condition

c  if no interaction between brook and soil water is simulated

c   (e.g. due to springs from outside the control volume of the hillslopes),

c  flow from sources qsour into the drainage system [cbm/s], which has to be given as boundary condtion

c  if interaction between brook and soil water is simulated,

dimension qbas(maxnb), qsour(maxnb)

c  logical variable to indicate whether interation at a node is allowed or not

dimension inter(maxnb)

c   Inputfile names

dimension brbfil(maxbrf)

common /cbachr/

&   b_dx, b_gef, b_ezg, b_yso, b_vorl, y_vorl, y_bach, weg_br,

&   b_rauh, b_m, qhang, qarea, qbas, brso, v_bach, aahang,

&   qpegel, qpmax, xba, yba,dtsur, qtol, bapsi, stpsi, lapsi,

&   qshg, qsour, flek, szbach, zfbach, bwq, zbach, qshang

common /cbachi/

&   iacnb, ib_nr, igang, iacgng, ibnext, inhang, inarea,

&   ib_ord, ib_reg, itmax, ibtop, ihgb,

&   iacqbr, iqrbb,iqsour, iacbrf, ibrbf, ibrart, irb_b

common /cbachl/ lqactiv, exhg, inter, auint, rbbneu

common /cbachc/

& brbfil

## Erosion.inc

c-----------------------------------------------------------------------

c  EROSION.INC            erosion

c-----------------------------------------------------------------------

real*8 abtrag, ab_alt, absum, ab_mit

real*8 qe, qemax

c erosion mass [kg/m/m]

dimension  abtrag(maxnl)

c erosion mass [kg/m/m]  (saving old state variable)

dimension  ab_alt(maxnl,maxnh)

c cumulative erosion from hillslope [kg] (inflow to channel)

dimension  absum(maxnh)

c old and new mean erosion mass (mean of hillslope surface)

dimension  ab_mit(2,maxnh)

c erosion flux along hillslope [kg/s], integrated over wet cross-section

dimension  qe(maxnl,maxnh)

c max. erosion flux at hillfoot [kg/s]

dimension  qemax(maxnh)

common /erosion/

&        abtrag, ab_alt, absum, ab_mit,

&        qe, qemax

# Alphabetical list of all variables in COMMON-blocks

The following pages list all variables which can be found in COMMON-blocks introduced in the previous chapter in alphabetical order. Variables are based in general on SI-units (meters, seconds, etc.) if not stated otherwise.

This explains the header of the following tables:

| variable | variable in program code |
| --- | --- |
| dimensions | dimensions assigned in program code |
| type | type of variable in program code. If * is ommitted default values of the compiler are used |
| common | COMMON-block name, where variable is described |
| meaning | explanation of variable |

| variable | dimensions | type | common | meaning |
| --- | --- | --- | --- | --- |
| A_e | (maxnv-1,maxnl) | R*8 | HGVARI | coefficients of numerical scheme |
| A_x | (maxnv,maxnl-1) | R*8 | HGVARI | coefficients of numerical scheme |
| A2e | (maxnv-1,maxnl) | R*8 | HGVARI | coefficients of numerical scheme |
| A2x | (maxnv,maxnl-1) | R*8 | HGVARI | coefficients of numerical scheme |
| aahang | (maxnb) | R*8 | BACH | surface area of linked hillslope |
| ab_alt | (maxnl,maxnh) | R*8 | EROSION | erosion mass [kg/m/m](saving old state variable) |
| ab_mit | (2,maxnh) | R*8 | EROSION | old and new mean erosion mass (mean of hillslope surface) |
| abbau | (maxstt,maxtyp) | R*8 | PFEST.INC | degradation coefficient (1st order) |
| absum | (maxnh) | R*8 | EROSION | cumulative erosion from hillslope [kg] (inflow to channel) |
| abtrag | (maxnl) | R*8 | EROSION | erosion mass [kg/m/m] |
| al_fak | (maxnv,maxnl-1,maxnh) | R*8 | HGFEST | weighting factor, used when computing mean moisture of two elements |
| alla | (maxcv,maxnh) | R*8 | HGFEST | area of longitudinal section of entire hillslope and (sub)-control-volumes |
| anisot | (2,maxtyp) | R*8 | SOIL | soil anisotropy |
| area | (maxnv,maxnl,maxnh) | R*8 | HGFEST | related element area to a computational node |
| auint | (1) | L | BACH.INC | indicates globally whether interaction between subsurface &channel water is simulated or not |
| av_fak | (maxnv-1,maxnl,maxnh) | R*8 | HGFEST | weighting factor, used when computing mean moisture of two elements |
| azimut | (maxnl,maxnh) | R*8 | HGFEST | azimut (orientation) of hillslope line (clockwise from north) |
| b_dx | (maxnb) | R*8 | BACH | length to next downstream node in [m] |
| b_ezg | (maxnb) | R*8 | BACH | local catchment area of current node |
| b_gef | (maxnb) | R*8 | BACH | slope of drainage network reach |
| b_m | (maxnb) | R*8 | BACH | slope of wall of trapeziodial crossection (1:b_m vertical/horizontal) |
| b_rauh | (maxnb) | R*8 | BACH | channel roughness (Strickler) |
| b_vorl | (maxnb) | R*8 | BACH | elevation of channel top |
| b_yso | (maxnb) | R*8 | BACH | elevation of channel bottom |
| balmax | (maxtyp) | R*8 | SOIL | soil-albedo-parameter: dry conditions: balmax |
| balmin | (maxtyp) | R*8 | SOIL | soil-albedo-parameter: from satkni * saturation:balmin |
| bapsi | (1) | R*8 | BACH | runoff coefficient of channel surfaces |
| bevapo | (maxcv,maxnh) | R*8 | HGVARI | cumulative evaporation flux term across boundary since start of computation |
| bezfak | (maxwrf,maxkli) | R*8 | HGBDRY | reference wind factors at climate stations |
| bilanz | (maxcv) | R*8 | HGVARI | total incremental water balance within a time step, should be zero |
| bilin | (maxcv,maxnh) | R*8 | HGVARI | cumulative change in total water content since start of computation |
| bilrd | (maxcv,maxnh) | R*8 | HGVARI | cumulative flux terms across all boundaries since start of computation |
| biltot | (maxcv,maxnh) | R*8 | HGVARI | total cumulativ balance (bilin-bilrd+bsenk), should be zero |
| bintz | (maxcv,maxnh) | R*8 | HGVARI | cumulative interceptionflux term across boundary since start of computation |
| bnied | (maxcv,maxnh) | R*8 | HGVARI | cumulative net rainfallflux term across boundary since start of computation |
| boden | (maxtyp) | C*30 | SOIL | soil describing string |
| bodpar | (maxpbm,maxtyp) | R*8 | SOIL | soil parameters |
| brbfil | (maxbrf) | C | BACH.INC | names of input files into the drainage network |
| brfl_l | (maxcv,maxnh) | R*8 | HGVARI | cumulative flux term across left boundary since start of computation |
| brfl_o | (maxcv,maxnh) | R*8 | HGVARI | cumulative flux term across topboundary since start of computation |
| brfl_r | (maxcv,maxnh) | R*8 | HGVARI | cumulative flux term across rightboundary since start of computation |
| brfl_u | (maxcv,maxnh) | R*8 | HGVARI | cumulative flux term across bottom boundary since start of computation |
| brso | (maxnb) | R*8 | BACH | bottom width of trapeziodial channel |
| bsenk | (maxcv,maxnh) | R*8 | HGVARI | cumulative flux terms from sinks/sources since start of computation |
| bsueb | (maxcv,maxnh) | R*8 | HGVARI | remainder of cumulative flux terms from sinks/sources since start of computation |
| btrans | (maxcv,maxnh) | R*8 | HGVARI | cumulative transpiration flux term across boundary since start of computation |
| bueb_l | (maxnh) | R*8 | HGVARI | remainder of cumulative flux term across left boundary since start of computation |
| bueb_o | (maxnh) | R*8 | HGVARI | remainder of cumulative flux term across topboundary since start of computation |
| bueb_r | (maxnh) | R*8 | HGVARI | remainder of cumulative flux term across rightboundary since start of computation |
| bueb_u | (maxnh) | R*8 | HGVARI | remainder of cumulative flux term across bottom boundary since start of computation |
| bwq | (2, maxbrf) | R*8 | BACH.INC | input flux into the drainage network boundary nodes |
| c_inj | (2,maxstt,maxrbf) | R*8 | PBDRY.INC | input concentration of solutes |
| c_ptr | (maxstt,maxnv-1,maxnl-1,maxnh) | R*8 | PVARI.INC | potential solute concentration in the solid phase without degradation |
| c_pw | (maxstt,maxnv-1,maxnl-1,maxnh) | R*8 | PVARI.INC | potential solute concentration in the water phase without degradation |
| c_tact | (maxstt,maxnv-1,maxnl-1,maxnh) | R*8 | PVARI.INC | total actual solute concentration in the solid phase without degradation |
| c_wact | (maxstt,maxnv-1,maxnl-1,maxnh) | R*8 | PVARI.INC | actual solute concentration in the water phase without degradation |
| cgeps | (1) | R*8 | DIM | iteration criteria |
| cilok | (maxstt,maxnl,maxnh) | R*8 | PBDRY.INC | input concentration at each boundary node |
| class | (maxgr+1) | I*4 | ZEIT | cumulative time within one time step class |
| clgr | (maxgr) | R*8 | ZEIT | boundary between classes of time step length |
| cltim | (maxgr+1) | R*8 | ZEIT | cumulative time within one time step class |
| cluse | (maxuse) | C*30 | HGBDRY | land-use descriptive string |
| d_koeff | (maxstt,maxtyp) | R*8 | PFEST.INC | dispersion coefficient |
| d_Phi_opt | (1) | R*8 | HGFEST | time step control parameter |
| d_Th_opt | (1) | R*8 | HGFEST | time step control parameter |
| depth | (maxnv,maxnl,maxnh) | R*8 | HGFEST | depth under surface |
| diffus | (maxnv,maxnl) | R*8 | HGVARI | actual diffusion number |
| dr_l | (maxnv,maxnh) | R*8 | HGFEST | real length in [m] (related interval to a point): along leftboundary |
| dr_o | (maxnl,maxnh) | R*8 | HGFEST | real length in [m] (related interval to a point): along surface boundary |
| dr_r | (maxnv,maxnh) | R*8 | HGFEST | real length in [m] (related interval to a point): along right boundary |
| dr_u | (maxnl,maxnh) | R*8 | HGFEST | real length in [m] (related interval to a point): along bottomboundary |
| dreta | (maxnv,maxnl,maxnh) | R*8 | HGFEST | real length in [m] (related interval to a point): all vertical distances |
| drxsi | (maxnv,maxnl,maxnh) | R*8 | HGFEST | real length in [m] (related interval to a point): all lateral distances |
| dstre | (1) | C*22 | ZEIT | date string end time [dd.mm.yyyy hh:mm:ss.hs] |
| dstrs | (1) | C*22 | ZEIT | date string start time [dd.mm.yyyy hh:mm:ss.hs] |
| dstrs2 | (1) | C*22 | ZEIT | date string[dd.mm.yyyy hh:mm:ss.hs] |
| dt_exp | (1) | R*8 | ZEIT | max. explicit time step [s] |
| dt_max | (1) | R*8 | ZEIT | global max. time step [s] |
| dt_min | (1) | R*8 | ZEIT | global min. time step [s] |
| dtbach | (1) | R*8 | ZEIT | max. timestep for active drainage network [s] |
| dtsur | (1) | R*8 | BACH | sub-time-step during computation of channel flow |
| durchl | (maxnv,maxnl) | R*8 | HGVARI | actual conductivity |
| e_p1m0 | (maxnv-1,maxnh) | R*8 | HGFEST | eta(iv+1)-eta(iv) |
| e_p1m1 | (maxnv-2,maxnh) | R*8 | HGFEST | eta(iv+1)-eta(iv-1) |
| Ecanop | (maxnl) | R*8 | HGBDRY | transpiration [m/s] |
| Eintz | (maxnl) | R*8 | HGBDRY | vaporation of interception [m/s] |
| epact | (maxstt,npmax,maxnh) | R*8 | PVARI.INC | actual particle position (eta) in curvilinear coordinates |
| epalt | (maxstt,npmax,maxnh) | R*8 | PVARI.INC | old particle position (eta) in curvilinear coordinates |
| eross | (maxero,maxtyp) | R*8 | SOIL | soil erosion parameter (Schramm-Model, Scherer-Model ?) |
| Esoil | (maxnl) | R*8 | HGBDRY | evaporation [m/s] |
| eta | (maxnv,maxnh) | R*8 | HGFEST | eta-coordinate |
| etilog | (1) | L | HGBDRY | true, if climate data is read, but yet no output of computed vaporation is dumped |
| exhg | (maxnum) | L | BACH | true, after hillslope number (not counter ih, but hangnr(ih)) has been computed |
| f_eta | (maxnv,maxnl,maxnh) | R*8 | HGFEST | metric coefficients (conversion to curvelinear coordinates) eta-direction |
| f_rs | (maxtyp) | R*8 | SOIL | soil resistance parameter (Kolle-model) |
| f_xsi | (maxnv,maxnl,maxnh) | R*8 | HGFEST | metric coefficients (conversion to curvelinear coordinates) xsi-direction |
| fabf | (maxnh) | R*8 | HGFEST | unit conversion factor for hillslope runoff [cbm/s] |
| fbrlow | (maxnl,maxnh) | R*8 | HGFEST | factors for use in numerical scheme, build from variable width of hillslope |
| fbrup | (maxnl,maxnh) | R*8 | HGFEST | factors for use in numerical scheme, build from variable width of hillslope |
| fcabf | (maxnh) | R*8 | HGFEST | unit conversion factor for cumulative hillslope runoff [cbm] |
| fdf | (1) | R*8 | HGFEST | intermediate factors |
| Fe_00 | (maxnv,maxnl) | R*8 | HGVARI | coefficients of numerical scheme |
| Fe_m1 | (maxnv,maxnl) | R*8 | HGVARI | coefficients of numerical scheme |
| Fe_p1 | (maxnv,maxnl) | R*8 | HGVARI | coefficients of numerical scheme |
| fl_max | (maxnh) | R*8 | DIM | max. flux within a hillslope |
| flaeche | (maxnv,maxnl,maxnh) | R*8 | PVARI.INC | area of grid cell |
| flek | (maxnb) | R*8 | BACH.INC | leakage coefficient |
| fmf | (1) | R*8 | HGFEST | intermediate factors |
| Fx_00 | (maxnv,maxnl) | R*8 | HGVARI | coefficients of numerical scheme |
| Fx_m1 | (maxnv,maxnl) | R*8 | HGVARI | coefficients of numerical scheme |
| Fx_p1 | (maxnv,maxnl) | R*8 | HGVARI | coefficients of numerical scheme |
| gefall | (maxnl,maxnh) | R*8 | HGFEST | tangens of angle between surface and horizontal |
| gteact | (maxnv,maxnl) | R*8 | PVARI.INC | actual moisture gradient in eta direction |
| gtealt | (maxnv,maxnl) | R*8 | PVARI.INC | old moisture gradient in eta direction |
| gtxact | (maxnv,maxnl) | R*8 | PVARI.INC | actual moisture gradient in xsi direction |
| gtxalt | (maxnv,maxnl) | R*8 | PVARI.INC | old moisture gradient in xsi direction |
| hangnr | (maxnh) | I | HGFEST | number of hillslopes (like a name) |
| hgbreit | (maxnh) | R*8 | HGFEST | mean width of hillslope |
| hgih | (maxnum) | I | HGFEST | position of hillslope number |
| hglang | (maxnh) | R*8 | HGFEST | length of hillslope |
| hgobfl | (maxnh) | R*8 | HGFEST | total surface area of hillslope |
| hko | (maxnv,maxnl,maxnh) | R*8 | HGFEST | vertical hillslope coordinates of grid points in [m+above reference level] |
| hkobez | (maxnh) | R*8 | HGFEST | vertical hillslope coordinates reference level in [m] |
| hkomin | (maxnh) | R*8 | HGFEST | reference level of internal used vertical coordinates |
| hkpact | (maxstt,npmax,maxnh) | R*8 | PVARI.INC | actual particle position (height) in cartesian coordinates |
| hkpalt | (maxstt,npmax,maxnh) | R*8 | PVARI.INC | old particle position (height) in cartesian coordinates |
| hor | (maxnl,maxhor,maxnh) | I*2 | HGBDRY | array of angle on horizon assigned to surface nodes |
| hwsp | (maxnh) | R*8 | HGVARI | elevation of water level at hillfoot |
| iacbrf | (1) | I*4 | BACH.INC | actual number of input time series for drainage network boundary nodes |
| iaccv | (maxnh) | I | HGFEST | actual number of control volumes |
| iaceig | (1) | I | SOIL | actual dimension of soil table: columns (properties) |
| iacfix | (1) | I*2 | HGBDRY | actual number of surface attributes |
| iacgng | (1) | I | BACH | actual number of output hydrographs |
| iacgr | (1) | I | ZEIT | actual number of time step class boundaries |
| iachor | (1) | I*2 | HGBDRY | actual number of directions where angle on horizon values are input |
| iackld | (maxkli) | I*2 | HGBDRY | actual read number of different climate data per record (time) |
| iackli | (1) | I*2 | HGBDRY | actual number of climate stations |
| iacnb | (1) | I | BACH | actual number of discrete drainage network reaches |
| iacnh | (1) | I | HGFEST | actual number of hillslope |
| iacnie | (1) | I*2 | HGBDRY | actual number of rainfall stations |
| iacnl | (maxnh) | I | HGFEST | actual number of lateralhillslope elements |
| iacnv | (maxnh) | I | HGFEST | actual number of vertical hillslope elements |
| iacpfp | (1) | I*2 | HGBDRY | actual number of different land use parameters in table pflpar |
| iacpft | (maxuse) | I*2 | HGBDRY | actual length of time series of landuse parameters |
| iacrbf | (1) | I*2 | HGBDRY | actual number of boundary condition files |
| iacsnk | (1) | I*2 | HGBDRY | actual number of sink/source files |
| iactab | (maxtyp) | I | SOIL | actual dimension of soil table: rows (discrete entries) |
| iactyp | (1) | I | SOIL | actual dimension of soil table: soiltypes |
| iacuse | (1) | I*2 | HGBDRY | actual number of land uses |
| iacwrf | (1) | I*2 | HGBDRY | actual number of wind direction classes |
| ib_nr | (maxnb) | I | BACH | number of node of drainage network |
| ib_ord | (maxnb) | I | BACH | STRAHLER-order |
| ib_reg | (maxnb) | I | BACH | rainfall-gauge number which contributes to runoff (according to the numbering in input file for rainfall time series) |
| ibnext | (maxnb) | I | BACH | number of subsequent downstream node |
| iboden | (maxnv,maxnl,maxnh) | I | HGFEST | index of soil type |
| ibrbf | (maxbrf) | I | BACH.INC | unit numbers of input files into the drainage network |
| ibtop | (maxnb,maxit) | I | BACH | related channel reach numbers of inflowing upstream nodes (itmax) |
| icf | (1) | I | DIM | project counter |
| icvl | (maxcv,maxnh) | I | HGFEST | index of left corner of control volumes for water balance |
| icvo | (maxcv,maxnh) | I | HGFEST | index of topcorner of control volumes for water balance |
| icvr | (maxcv,maxnh) | I | HGFEST | index of rightcorner of control volumes for water balance |
| icvu | (maxcv,maxnh) | I | HGFEST | index of bottom corner of control volumes for water balance |
| ieact | (maxstt,npmax,maxnh) | R*8 | PVARI.INC | actual eta index (nv) of grid cell where a particle is located |
| iealt | (maxstt,npmax,maxnh) | R*8 | PVARI.INC | old eta index (nv) of grid cell where a particle is located |
| ifixob | (maxnl,maxfix,maxnh) | I*2 | HGBDRY | array of surface indices |
| igang | (maxgng) | I | BACH | channel node numbers, where hydrographs will be output |
| igewkn | (2,maxnh) | I | HGFEST | number of channel reach, where hillslope runoff (1) and sealed surface (2) contribute |
| ih1st | (1) | I | HGVARI | number of that hillslope, which first produces surface runoff |
| ihgb | (maxnh) | R*8 | BACH.INC | hillslope number associated to drainage network nodes |
| ihgr | (1) | I | HGFEST | switch, controls output on log file |
| iikli | (maxkli) | I | DIM | input units climate |
| iimif | (maxmif) | I*4 | PBDRY.INC | assigns input mass files to boundary nodes |
| iin | (maxin) | I | DIM | input units |
| iin_act | (1) | I | DIM | actual number of global input files |
| iinie | (maxnie) | I | DIM | input units rainfall |
| iirbf | (maxrbf) | I | DIM | input units boundary conditions |
| iisnk | (maxsnk) | I | DIM | input units sink/source |
| ilok | (1) | I | DIM | input unit |
| imod | (maxtyp) | I | SOIL | integer for method to convert soil parameters to soil table |
| inarea | (maxnb) | I | BACH | if > 0, contributing sealed surface is present |
| inhang | (maxnb) | I | BACH | number of incoming hillslope |
| inter | (maxnb) | L | BACH.INC | indicates at each channel node whether interaction between subsurface &channel water is simulated or not |
| interz | (maxnl) | R*8 | HGBDRY | interception [m] |
| inutz | (1) | I | DIM | input unit |
| io | (maxout) | I | DIM | output units |
| io_act | (1) | I | DIM | actual number of global output files |
| io_log | (maxout) | I | DIM | switch, controls amount of output data |
| ip | (maxnh) | I | ZEIT | print times counter |
| ipar | (1) | I | DIM | input unit |
| iplosl | (maxstt,npmax) | R*8 | PVARI.INC | id of particle lost across lower boundary |
| iplosr | (maxstt,npmax) | R*8 | PVARI.INC | id of particle lost across left boundary |
| iplosu | (maxstt,npmax) | R*8 | PVARI.INC | id of particle lost across right boundary |
| iqrbb | (maxnb) | I | BACH.INC | points on the number of boundary nodes of the drainage network |
| iqsour | (maxnb) | I | BACH.INC | marks boundary nodes (1) of the drainage network |
| irb_b | (maxnb) | I | BACH.INC | assigns input time series to drainage network nodes |
| irb_l | (maxnv,maxnh) | I*2 | HGBDRY | boundary condition file index (left boundary) |
| irb_o | (maxnl,maxnh) | I*2 | HGBDRY | boundary condition file index (surface boundary) |
| irb_r | (maxnv,maxnh) | I*2 | HGBDRY | boundary condition file index (right boundary) |
| irb_u | (maxnl,maxnh) | I*2 | HGBDRY | boundary condition file index (bottom boundary) |
| irbtyp | (2,maxrbf) | I*2 | HGBDRY | actual type of boundary condition(mathematical) |
| iseed | (1) | R*8 | PVARI.INC | seed for random number generator |
| isktyp | (2,maxsnk) | I*2 | HGBDRY | actual type of sink/source boundary condition(mathematical) |
| isnk | (maxnv,maxnl,maxnh) | I*2 | HGBDRY | sink/source boundary condition file index (internal boundary) |
| it_max | (1) | I | DIM | max. number of Picard iterations |
| it_p | (maxprt,maxnh) | I | ZEIT | switch for output control |
| itag | (1) | R*8 | ZEIT | number of day in year [s] |
| itmax | (maxnb) | I | BACH | number of inflowing upstream nodes |
| iusenr | (maxnl,maxnh) | I*2 | HGBDRY | land-use number of a surface node |
| ivm | (maxnl,maxnh) | I | HGFEST | vertical index of deepest macroporous element |
| ivroot | (maxnl) | I*2 | HGBDRY | number of vertical element at rooting depth |
| ixact | (maxstt,npmax,maxnh | R*8 | PVARI.INC | actual xsi index (nl) of grid cell where a particle is located |
| ixalt | (maxstt,npmax,maxnh) | R*8 | PVARI.INC | old xsi index (nl) of grid cell where a particle is located |
| iz_alt | (maxnl,maxnh) | R*8 | HGBDRY | interception [m], saved value of state variable |
| izaehl | (maxnh) | I*4 | ZEIT | counter for time steps |
| k_fil | (maxkli) | C*30 | HGBDRY | climate filenames |
| k_sorp | (maxstt,maxtyp) | R*8 | PFEST.INC | adsorption coefficient (linear, equilibrium) |
| kee | (maxnv,maxnl,maxnh) | R*8 | HGFEST | element of tensor of anisotropy (normed with saturated conductivity k_s) |
| klfehlt | (maxkli) | L | HGBDRY | true, if climate data is missing (-999) |
| klifak | (maxnie) | R*8 | HGBDRY | conversion factor climatefile time unit [m/s] (only file typ 2) |
| klima | (2,maxkld,maxkli) | R*8 | HGBDRY | actual climate data interval |
| klimas | (maxkld,maxkli) | R*8 | HGBDRY | actual climate data (saved) |
| kstall | (maxfil) | R*8 | DIM | global hillslope surface roughness for one project (can be overriden susequently) |
| ktyp | (maxkli) | I*2 | HGBDRY | type of climatic data (1 or 2) |
| kxe | (maxnv,maxnl,maxnh) | R*8 | HGFEST | element of tensor of anisotropy (normed with saturated conductivity k_s) |
| kxx | (maxnv,maxnl,maxnh) | R*8 | HGFEST | element of tensor of anisotropy (normed with saturated conductivity k_s) |
| lambda | (maxnh) | R*8 | HGFEST | leakage-parameter (k'/d') of channel at hillfoot |
| lapsi | (3) | R*8 | BACH | runoff coefficient of land surfaces related to a STRAHLER-order (not in use anymore) |
| lati | (1) | R*8 | HGFEST | latitude of catchment |
| lklima | (1) | L | HGBDRY | true, if climate data is read |
| lland | (1) | L | DIM | switch is true, when land use is considered |
| lmak | (maxnh) | L | HGFEST | true, if macropore information is read from file |
| lnfile | (1) | C*30 | HGBDRY | name of assignment file land-use-unit ID and land-use ID |
| lnied | (1) | L | HGBDRY | true, if rainfall data is read |
| lnintp | (maxnie) | L | HGBDRY | true, if interpolation between subsequent rainfall values is desired |
| longi | (1) | R*8 | HGFEST | longitude of catchment |
| lpob | (maxnh) | L | DIM | switch is true, when surface attributes are read |
| lqactiv | (1) | L | BACH | swich, true when drainage network runoff is active |
| lrd_l | (maxnh) | R*8 | HGFEST | total length of left boundaries along hillslope |
| lrd_o | (maxnh) | R*8 | HGFEST | total length of topboundaries along hillslope |
| lrd_r | (maxnh) | R*8 | HGFEST | total length of rightboundaries along hillslope |
| lrd_u | (maxnh) | R*8 | HGFEST | total length of bottom boundaries along hillslope |
| lrill | (maxnl,maxnh) | L | HGVARI | true, if rillflow is computed, currently fixed (false) |
| lrintp | (maxrbf) | L | HGBDRY | true, if interpolation between subsequent boundary condition values is desired |
| lsintp | (maxsnk) | L | HGBDRY | true, if interpolation between subsequent boundary condition values is desired |
| lueb_l | (maxnv) | L | HGBDRY | true, if left boundary flux cannot infiltrate totally (in case of unequality condition) |
| lueb_o | (maxnl) | L | HGBDRY | true, if topboundary flux cannot infiltrate totally (in case of unequality condition) |
| lueb_r | (maxnv) | L | HGBDRY | true, if rightboundary flux cannot infiltrate totally (in case of unequality condition) |
| lueb_s | (maxnv,maxnl) | L | HGBDRY | true, if internal boundary flux cannot infiltrate totally (in case of unequality condition) |
| lueb_u | (maxnl) | L | HGBDRY | true, if bottom boundary flux cannot infiltrate totally (in case of unequality condition) |
| luseid | (maxuse) | I*2 | HGBDRY | IDs of land uses |
| lw11st | (1) | L | HGVARI | true, as long header line has not been dumped to surface runoff file (factors fabf, fcabf) |
| lw1st | (maxnh) | L | HGVARI | true, as long as no surface runoff output has been dumped |
| lwriten | (1) | L | HGVARI | true, if rainfall has to be dumped to surface runoff output file |
| lwriteq | (1) | L | HGVARI | true, if runoff has to be dumped to surface runoff output file |
| lwrites | (1) | L | HGVARI | true, if runoff has to be dumped, ensures one more value to be dumped after event ended |
| m_aniso | (1) (parameter) | I | HGFEST | indicates in case of macropore flow whether ks is increased exclusively in the vertical direction (=1) or in both directions (0) |
| m_pt | (maxstt,maxrbf) | R*8 | PFEST.INC | initial particle mass |
| macro | (maxnv,maxnl,maxnh) | R*8 | HGFEST | macropore factor |
| mak_an | (maxnv,maxnl) | L | HGVARI | true, if macropores are active |
| maxbrf | (1) | I*4 | DIM.INC | maximum number of input time series for drainage network boundary nodes |
| maxcv | (1) (parameter) | I | DIM | max. number of control volumes for hillslope water balance |
| maxeig | (1) (parameter) | I | DIM | max. number of soil properties(columns in soil table) |
| maxero | (1) (parameter) | I | DIM | max. number of soil specific erosion parameters |
| maxfil | (1) (parameter) | I | DIM | control files |
| maxfix | (1) (parameter) | I | DIM | max. number of fixed integers to be assigned to a surface node |
| maxgng | (1) (parameter) | I | DIM | max. number of output hydrographs for output |
| maxgr | (1) (parameter) | I | DIM | max. number of classes for time step statistics |
| maxhor | (1) (parameter) | I | DIM | max. number of angle to horizon for a surface node |
| maxin | (1) (parameter) | I | DIM | input files |
| maxit | (1) (parameter) | I | DIM | max. number of inflows from upstream (confluences) |
| maxkld | (1) (parameter) | I | DIM | max. number of climatic data in climate time series (min. number = 6) |
| maxkli | (1) (parameter) | I | DIM | max. number of climate time series |
| maxnb | (1) (parameter) | I | DIM | max. number of discrete drainage network reaches |
| maxnh | (1) (parameter) | I | DIM | max. number of hillslopes |
| maxnie | (1) (parameter) | I | DIM | max. number of rainfall time series |
| maxnl | (1) (parameter) | I | DIM | max. number of soil columns (xsi) |
| maxnum | (1) (parameter) | I | DIM | highest hillslope number (used as a name) |
| maxnv | (1) (parameter) | I | DIM | max. number of soil layers (eta) |
| maxout | (1) (parameter) | I | DIM | output files |
| maxpbm | (1) (parameter) | I | DIM | max. number of soil parameters of a model of soil hydraulic behaviour |
| maxpfp | (1) (parameter) | I | DIM | max. number of land-use parameters(columns in land-use table) |
| maxpft | (1) (parameter) | I | DIM | max. length of land-use time series (rows in land-use table) |
| maxpkt | (1) (parameter) | I | DIM | max. number of soil moisture time series for output |
| maxprt | (1) (parameter) | I | DIM | max. number of print-out times |
| maxrbf | (1) (parameter) | I | DIM | max. number of boundary condition time series |
| maxrbp | (1) (parameter) | I | DIM | max. number of boundary condition parameters |
| maxskp | (1) (parameter) | I | DIM | max. number of sink/source parameters |
| maxsnk | (1) (parameter) | I | DIM | max. number of sink/source time series |
| maxstt | (1) | I | DIM.INC | maximum of different solute types |
| maxtab | (1) (parameter) | I | DIM | max. length of soil table (rows in soil table) |
| maxtyp | (1) (parameter) | I | DIM | max. number of soil types (as assigned in soil-parameter file) |
| maxuse | (1) (parameter) | I | DIM | max. number of land uses |
| maxwrf | (1) (parameter) | I | DIM | max. number of wind direction dependent factors |
| minfil | (maxmif) | C*30 | PBDRY.INC | name of input mass files |
| mkorrf | (maxstt,maxnh) | R*8 | PVARI.INC | corrective factor for mass balance |
| mm_eta | (maxnv-1,maxnl,maxnh) | I | HGFEST | method of averaging conductivity between two elements |
| mm_xsi | (maxnv,maxnl-1,maxnh) | I | HGFEST | method of averaging conductivity between two elements |
| mp_inj | (maxstt,maxrbf) | R*8 | PBDRY.INC | solute input masses |
| mpact | (maxstt,npmax,maxnh) | R*8 | PVARI.INC | actual particle mass |
| mpkon | (maxstt,maxnv,maxnl) | R*8 | PVARI.INC | mass in a grid cell |
| n_fil | (maxnie) | C*30 | HGBDRY | rainfall filenames |
| n_gr | (1) | I | DIM | number of Picard iterations, from which subsequent time step is reduced |
| neff | (maxnl) | R*8 | HGBDRY | effective (net) rainfall |
| nied | (2,maxnie) | R*8 | HGBDRY | actual rainfall data interval |
| niefak | (maxkli) | R*8 | HGBDRY | conversion factor rainfall file time unit [m/s] |
| ninact | (maxstt,maxnl) | I*4 | PBDRY.INC | actual number of particles injected at a boundary node |
| ninpot | (maxstt,maxnl) | I*4 | PBDRY.INC | potential number of particles injected at a boundary node |
| np_ofl | (maxstt,maxnh) | I*4 | PBDRY.INC | number lost in surface runoff & return flow per time step |
| npact | (maxstt,maxnh) | I*4 | PBDRY.INC | actual number of particles in a hillslope |
| npalt | (maxstt,maxnh) | I*4 | PBDRY.INC | old number of particles in a hillslope |
| npkon | (maxstt,maxnv,maxnl) | R*8 | PVARI.INC | number of particles in a grid cell |
| nplosl | (maxstt, maxnh) | R*8 | PVARI.INC | particle loss across left boundary |
| nplosr | (maxstt,maxnh) | R*8 | PVARI.INC | particle loss across lower boundary |
| nplosu | (maxstt,maxnh) | R*8 | PVARI.INC | particle loss across right boundary |
| nposum | (maxstt,maxnh) | I*4 | PBDRY.INC | cumulative number lost in surface runoff & return flow |
| pflpar | (maxpft,maxpfp+1,maxuse) | R*8 | HGBDRY | land-use parameter table |
| phi_max | (maxnh) | R*8 | DIM | max. total potential within a hillslope |
| phi_min | (maxnh) | R*8 | DIM | min. total potential within a hillslope |
| phialt | (maxnv,maxnl,maxnh) | R*8 | HGVARI | old state of the system (potential), to be used e.g. if iteration fails and time step has to be repeated |
| phiini | (maxnv,maxnl) | R*8 | HGVARI | initial state of the system (potential) |
| phineu | (maxnv,maxnl) | R*8 | HGVARI | new state of the system (potential) |
| piceps | (1) | R*8 | DIM | iteration criteria |
| pl_pot | (maxnv) | R*8 | HGBDRY | potential left potentials (in case of unequality condition) |
| plrw | (maxnv,maxnl) | R*8 | HGBDRY | share of total plant reachable water within current element (0...1) |
| po_pot | (maxnl) | R*8 | HGBDRY | potential toppotentials (in case of unequality condition) |
| pr_pot | (maxnv) | R*8 | HGBDRY | potential rightpotentials (in case of unequality condition) |
| ps_pot | (maxnv,maxnl) | R*8 | HGBDRY | potential internal potentials (in case of unequality condition) |
| psi | (maxnv,maxnl) | R*8 | HGVARI | suction head |
| psi_max | (maxnh) | R*8 | DIM | max. suction head within a hillslope |
| psi_min | (maxnh) | R*8 | DIM | min. suction head within a hillslope |
| pu_pot | (maxnl) | R*8 | HGBDRY | potential bottom potentials (in case of unequality condition) |
| q | (maxnl,maxnh) | R*8 | HGVARI | actual runoff at each surface node |
| q_eta | (maxnv,maxnl) | R*8 | HGVARI | fluxes in eta-direction |
| q_xsi | (maxnv,maxnl) | R*8 | HGVARI | fluxes in xsi-direction |
| qarea | (maxnb) | R*8 | BACH | mean inflow from sealed surfaces during time up to actual time horizon (in CATFLOW.FOR) |
| qbas | (maxnb) | R*8 | BACH | baseflow per unit channel length, which has to be given as a boundary condition (e.g. due to springs from outside the control volume of the hillslopes) |
| qe | (maxnl,maxnh) | R*8 | EROSION | erosion flux along hillslope [kg/s], integrated over wet cross-section |
| qemax | (maxnh) | R*8 | EROSION | max. erosion flux at hillfoot [kg/s] |
| qhang | (maxnh) | R*8 | BACH | mean inflow from hillslope during time up to actual time horizon (in CATFLOW.FOR) |
| ql_pot | (maxnv) | R*8 | HGBDRY | potential left fluxes (in case of unequality condition) [m/s] |
| qo_pot | (maxnl) | R*8 | HGBDRY | potential topfluxes (in case of unequality condition) [m/s] |
| qomax | (maxnh) | R*8 | HGVARI | max. runoff at hillfoot since start of simulation |
| qosum | (maxnh) | R*8 | HGVARI | cumulative runoff at hillfoot since start of simulation |
| qpegel | (1) | R*8 | BACH | cumulative outflow from catchment [cbm/s] |
| qpmax | (1) | R*8 | BACH | max. outflow from catchment [cbm/s] |
| qr_pot | (maxnv) | R*8 | HGBDRY | potential rightfluxes (in case of unequality condition) [m/s] |
| qs_pot | (maxnv,maxnl) | R*8 | HGBDRY | potential internal fluxes (in case of unequality condition) [m/s] |
| qshg | (maxnh) | R*8 | BACH.INC | subsurface flow into the drainage network |
| qssum | (maxnh) | R*8 | BACH.INC | cumulative subsurface flow into the drainage network |
| qtol | (1) | R*8 | BACH | tolerance for drainage network runoff computation in [cbm/s per s]. If acceleration of runoff is lower, runoff process is temporal neglected. |
| qu_pot | (maxnl) | R*8 | HGBDRY | potential bottom fluxes (in case of unequality condition) |
| r_ret | (maxstt,maxnv,maxnl) | R*8 | PVARI.INC | retardation factor |
| rbfil | (maxrbf) | C*30 | HGBDRY | boundary condition filenames |
| rBilart | (maxkli) | I*4 | HGPDRY | indicates whether measured net radiation is used (2) or not (1) |
| rbpar | (2,maxrbp,maxrbf) | R*8 | HGBDRY | associated actual B-C parameters |
| rfl_l | (maxnv) | R*8 | HGVARI | left boundary flux |
| rfl_o | (maxnl) | R*8 | HGVARI | topboundary flux |
| rfl_r | (maxnv) | R*8 | HGVARI | rightboundary flux |
| rfl_u | (maxnl) | R*8 | HGVARI | bottom boundary flux |
| rlongi | (1) | R*8 | HGFEST | reference longitude of time zone (CET: 15 degree east of Greenwich) |
| RS | (maxnv,maxnl) | R*8 | HGVARI | coefficients of numerical scheme |
| s_tab | (maxtab,maxeig,maxtyp) | R*8 | SOIL | soil hydraulic table |
| satkni | (maxtyp) | R*8 | SOIL | soil-albedo-parameter |
| senk | (maxnv,maxnl) | R*8 | HGVARI | sink term[1/sec] |
| sk_max | (maxnh) | R*8 | DIM | max. sink/source flux within a hillslope |
| sko | (maxnv,maxnl,maxnh) | R*8 | HGFEST | lateral hillslope coordinates of computational nodes (grid points) in [m] |
| skpact | (maxstt,npmax,maxnh) | R*8 | PVARI.INC | actual particle position (lateral) in cartesian coordinates |
| skpalt | (maxstt,npmax,maxnh) | R*8 | PVARI.INC | old particle position (lateral) in cartesian coordinates |
| skpar | (2,maxskp,maxsnk) | R*8 | HGBDRY | associated actual sink/source B-C parameters |
| slopel | (maxnv,maxnh) | R*8 | HGFEST | cosine of angle between left boundary and horizontal |
| slopeo | (maxnl,maxnh) | R*8 | HGFEST | cosine of angle between topboundary and horizontal |
| sloper | (maxnv,maxnh) | R*8 | HGFEST | cosine of angle between rightboundary and horizontal |
| slopeu | (maxnl,maxnh) | R*8 | HGFEST | cosine of angle between bottom boundary and horizontal |
| snkfil | (maxsnk) | C*30 | HGBDRY | sink/source filenames |
| snull | (maxtyp) | R*8 | SOIL | storage coefficient for saturated soil |
| stpsi | (3) | R*8 | BACH | runoff coefficient of sealed surfaces related to a STRAHLER-order (not in use anymore) |
| stunde | (1) | R*8 | ZEIT | hour of day [s] |
| sueb | (maxnv,maxnl) | R*8 | HGVARI | remainder of sink term[1/sec] |
| sw0 | (maxkli) | R*8 | HGBDRY | parameter of regression between rBil and shortwave-Bil (ETINTZ, Kolle-model) |
| sw1 | (maxkli) | R*8 | HGBDRY | parameter of regression between rBil and shortwave-Bil (ETINTZ, Kolle-model) |
| sw2 | (maxkli) | R*8 | HGBDRY | parameter of regression between rBil and shortwave-Bil (ETINTZ, Kolle-model) |
| szbach | (maxbrf) | C*30 | BACH.INC | reference date of input time series to drainage network |
| szkli | (maxkli) | R*8 | HGBDRY | offset between computation time and time in climateinput file |
| sznie | (maxnie) | R*8 | HGBDRY | offset between computation time and time in rainfall input file |
| szrbf | (maxrbf) | R*8 | HGBDRY | offset between computation time and time in B-Cinput file |
| szsnk | (maxsnk) | R*8 | HGBDRY | offset between computation time and time in sink input file |
| t_act | (1) | R*8 | ZEIT | actual simulation point in time [s] |
| t_alt | (1) | R*8 | ZEIT | local starting point in time [s] |
| t_end | (1) | R*8 | ZEIT | global end point in time [s] |
| t_inj | (0 30000) | R*8 | PBDRY.INC | time increment for particle injection |
| t_ln | (2) | R*8 | HGBDRY | time series of land use assignment files |
| t_neu | (1) | R*8 | ZEIT | local end point in time [s] |
| t_p | (maxprt,maxnh) | R*8 | ZEIT | print times |
| t_start | (1) | R*8 | ZEIT | global starting point in time [s] |
| tabfil | (maxtyp) | C*30 | SOIL | name of a input file holding a table of soil hydraulic properties |
| tabpos | (maxnv,maxnl,maxnh) | I | HGVARI | current position in soil hydraulic function table |
| th_alt | (maxnv,maxnl) | R*8 | HGVARI | old soil moisture |
| th_ini | (maxnv,maxnl) | R*8 | HGVARI | initial soil moisture |
| th_max | (maxnh) | R*8 | DIM | max. soil moisture within a hillslope |
| th_min | (maxnh) | R*8 | DIM | min. soil moisture within a hillslope |
| th_mit | (maxcv,2,maxnh) | R*8 | HGVARI | old and new mean soil moisture |
| th_pwp | (maxtyp) | R*8 | SOIL | permanent wilting point [Vol.- %] |
| theta | (maxnv,maxnl) | R*8 | HGVARI | actual soil moisture |
| trueb | (maxkli) | R*8 | HGBDRY | parameter for atmosperic dimming (ETINTZ, Kolle-model) |
| truebf | (maxkli) | R*8 | HGBDRY | parameter for atmosperic dimming (ETINTZ, Kolle-model) |
| ueb_l | (maxnv) | R*8 | HGVARI | remainder of left boundary flux |
| ueb_o | (maxnl) | R*8 | HGVARI | remainder of topboundary flux |
| ueb_r | (maxnv) | R*8 | HGVARI | remainder of rightboundary flux |
| ueb_u | (maxnl) | R*8 | HGVARI | remainder of bottom boundary flux |
| v_bach | (maxnb) | R*8 | BACH | wetted channel volume |
| varbr | (maxnl,maxnh) | R*8 | HGFEST | variable width of hillslope |
| ve_st | (maxnv,maxnl,maxnh) | R*8 | PVARI.INC | actual pore water velocity in eta direction |
| ve_sta | (maxnv,maxnl,maxnh) | R*8 | PVARI.INC | old pore water velocity in eta direction |
| vevapo | (maxcv) | R*8 | HGVARI | incremental evaporation flux term across boundary within a time step |
| vg_ngr | (1) (parameter) | R*8 | SOIL | boundary between arithmetric and geometric averaging for Van Genuchtens n (after Zurmuehl) |
| vintz | (maxcv) | R*8 | HGVARI | incremental interceptionflux term across boundary within a time step |
| vnied | (maxcv) | R*8 | HGVARI | incremental net rainfallflux term across boundary within a time step |
| vola | (maxcv,maxnh) | R*8 | HGFEST | volume of entire hillslope and (sub)-control-volumes (variable width) |
| volin | (maxcv) | R*8 | HGVARI | incremental change in total water content within a time step |
| volrd | (maxcv) | R*8 | HGVARI | incremental flux terms across all boundaries within a time step |
| vorfak | (maxnv,maxnl) | R*8 | HGVARI | coefficients of numerical scheme |
| vorz_l | (maxnv,maxnh) | I*2 | HGBDRY | sign of left boundary condition (-1: potential,1: flux) |
| vorz_o | (maxnl,maxnh) | I*2 | HGBDRY | sign of topboundary condition (-1: potential,1: flux) |
| vorz_r | (maxnv,maxnh) | I*2 | HGBDRY | sign of rightboundary condition (-1: potential,1: flux) |
| vorz_s | (maxnv,maxnl,maxnh) | I*2 | HGBDRY | sign of internal boundary condition (-1: potential,1: flux) |
| vorz_u | (maxnl,maxnh) | I*2 | HGBDRY | sign of bottom boundary condition (-1: potential,1: flux) |
| vrfl_l | (maxcv) | R*8 | HGVARI | incremental flux term across left boundary within a time step |
| vrfl_o | (maxcv) | R*8 | HGVARI | incremental flux term across topboundary within a time step |
| vrfl_r | (maxcv) | R*8 | HGVARI | incremental flux term across rightboundary within a time step |
| vrfl_u | (maxcv) | R*8 | HGVARI | incremental flux term across bottom boundary within a time step |
| vsav_l | (maxnv) | I*2 | HGBDRY | sign of left boundary condition (-1: potential,1: flux), saved values |
| vsav_o | (maxnl) | I*2 | HGBDRY | sign of topboundary condition (-1: potential,1: flux), saved values |
| vsav_r | (maxnv) | I*2 | HGBDRY | sign of rightboundary condition (-1: potential,1: flux), saved values |
| vsav_s | (maxnv,maxnl) | I*2 | HGBDRY | sign of internal boundary condition (-1: potential,1: flux), saved values |
| vsav_u | (maxnl) | I*2 | HGBDRY | sign of bottom boundary condition (-1: potential,1: flux), saved values |
| vsenk | (maxcv) | R*8 | HGVARI | incremental flux term across internal boundary within a time step |
| vsueb | (maxcv) | R*8 | HGVARI | remainder of incremental flux term across internal boundary within a time step |
| vtrans | (maxcv) | R*8 | HGVARI | incremental transpiration flux term across boundary within a time step |
| vueb_l | (1) | R*8 | HGVARI | remainder of incremental flux term across left boundary within a time step |
| vueb_o | (1) | R*8 | HGVARI | remainder of incremental flux term across topboundary within a time step |
| vueb_r | (1) | R*8 | HGVARI | remainder of incremental flux term across rightboundary within a time step |
| vueb_u | (1) | R*8 | HGVARI | remainder of incremental flux term across bottom boundary within a time step |
| vx_st | (maxnv,maxnl,maxnh) | R*8 | PVARI.INC | actual pore water velocity in xsi direction |
| vx_sta | (maxnv,maxnl,maxnh) |  |  | old pore water velocity in xsi direction |
| w_hohr | (maxnv,maxnl) | R*8 | HGFEST | angle between horizontal line and main direction of anisotropy, positiv counterclockwise |
| w_xsho | (maxnv,maxnl) | R*8 | HGFEST | angle between xsi-direction and horizontal line, positiv counterclockwise, depends only on coordinate system |
| w_xshr | (maxnv,maxnl) | R*8 | HGFEST | angle between xsi-direction and main direction of anisotropy, positiv counterclockwise |
| wasska | (maxnv,maxnl) | R*8 | HGVARI | actual water capacity |
| weg_br | (maxnb) | R*8 | BACH | width of sealed surface along channel |
| wp_rs | (maxtyp) | R*8 | SOIL | soil resistance parameter (Kolle-model) |
| wrf | (maxnl,maxwrf,maxnh) | R*8 | HGBDRY | array of wind reduction factors assigned to surface nodes |
| wro | (maxwrf) | R*8 | HGBDRY | boundaries of wind direction sectors (lower value, in degree) |
| wru | (maxwrf) | R*8 | HGBDRY | boundaries of wind direction sectors (upper value, in degree) |
| x_p1m0 | (maxnl-1,maxnh) | R*8 | HGFEST | xsi(il+1)-xsi(il) |
| x_p1m1 | (maxnl-2,maxnh) | R*8 | HGFEST | xsi(il+1)-xsi(il-1) |
| xba | (maxnb) | R*8 | BACH | horizontal coordinate of drainage network node |
| xko | (maxnl) | R*8 | HGFEST | horizontal global coordinates ("Rechtswert") in [m] |
| xkobez | (maxnh) | R*8 | HGFEST | reference coordinate ("Rechtswert") in [m] |
| xpact | (maxstt,npmax,maxnh) | R*8 | PVARI.INC | actual particle position (height) in curvilinear coordinates |
| xpalt | (maxstt,npmax,maxnh) | R*8 | PVARI.INC | old particle position (xsi) in curvilinear coordinates |
| xsi | (maxnl,maxnh) | R*8 | HGFEST | xsi-coordinate |
| xyxpy | (maxnv,maxnl,maxnh) | R*8 | HGFEST | geometrical quantity used for computation of diffusion number (DIFFCAL, not activ anymore) |
| y_bach | (maxnb) | R*8 | BACH | actual runoff depth |
| y_vorl | (maxnb) | R*8 | BACH | channel depth (bottom to top) |
| yba | (maxnb) | R*8 | BACH | horizontal coordinate of drainage network node |
| yko | (maxnl) | R*8 | HGFEST | horizontal global coordinates ("Hochwert") in [m] |
| ykobez | (maxnh) | R*8 | HGFEST | reference coordinates ("Hochwert") in [m] |
| yo_alt | (maxnl,maxnh) | R*8 | HGVARI | old depth of surface runoff |
| yo_mit | (2,maxnh) | R*8 | HGVARI | old and new mean depth of surface runoff on hillslope |
| yoben | (maxnl) | R*8 | HGVARI | depth of surface runoff (vertical, while local variable yo in UPDYO is perpendicular to surface) |
| z2skli | (maxkli) | R*8 | HGBDRY | conversion factor climatefile time unit [seconds] |
| z2snie | (maxnie) | R*8 | HGBDRY | conversion factor rainfall file time unit [seconds] |
| z2srbf | (maxrbf) | R*8 | HGBDRY | conversion factor B-Cfile time unit [seconds] |
| z2ssnk | (maxsnk) | R*8 | HGBDRY | conversion factor sink file time unit [seconds] |
| zbach | (2, maxbrf) | R*8 | BACH.INC | time in the input time series to drainage network nodes |
| zd_max | (maxtyp) | R*8 | SOIL | soil resistance parameter (Kolle-model) |
| zfbach | (maxbrf) | R*8 | BACH.INC | conversion factor for zbach into seconds |
| zkli | (2,maxkli) | R*8 | HGBDRY | time since t_start in [s] climatefile |
| znie | (2,maxnie) | R*8 | HGBDRY | time since t_start in [s] rainfall file |
| zrbf | (2,maxrbf) | R*8 | HGBDRY | time since t_start in [s] B-Cfile |
| zref | (maxkli) | R*8 | HGBDRY | reference height of wind velocity measurement |
| zsnk | (2,maxsnk) | R*8 | HGBDRY | time since t_start in [s] sink file |

# Moving to other platforms

The code has been written in standard FORTRAN77 with few exceptions:

Intensive use is made of a FORTRAN extension, which allows reading data from strings unformatted.

- The program is compiled best with the short integer compiler option (INTEGER*2) active, thus saving memory space.

- This way, all unspecified INTEGER declarations are defined as two byte representations. However, occasionally long integer are required and declared explicitely in the program (INTEGER*4). Intrinsic functions to convert short in long integer are non-standard, though state-of-the-art compilers support this conversions. Here INTL is used (i4=INTL(i2)) as supplied by the SALFORD FTN77 compiler. These functions have to be changed when other compilers are being used. E.g., the HP-UX F77 compiler supplies the intrinsic function JIFIX for this purpose. % files: BACH, OBERFL, TCAL

- The measurement of computing time is performed by use of a small subroutine called REL_SEC. It implements the function to calculate seconds since some time in the past. System calls are inevitable for this task. Thus, this routine has to be rewritten for each platform.

Further characteristics of the code include:

- All variables used are declared explicitly.

- All subroutines used are stated as external functions.

- All intrinsic functions used are stated as such.

- A great portion of data is communictated throughout the program by COMMON blocks.

As a final example this is the command which produces the executable CF.EXE from the source code kept in file ALL.F on a HP-UX system:

f77 -L +e -I2 -o CF.EXE ALL.F 1> list.log

# References

Beven, K., Calver, A. Morris, E. M. (1987): The Institute of Hydrology distributed model. Inst. Hydrol. Rep. 98, Institute of Hydrology, Wallingford.

Biggar, J.W. and Nielsen, D.R. (1967): Miscible displacement and leaching phenomena. Argonomy 11: 254 – 274.

Blöschl, G. (1996): Scale and Scaling in Hydrology. Wiener Mitteilungen, Wasser-Abwasser-Gewässer, Band 132, Habilitationsschrift, TU Wien

Calfer,. a And Wood, W., L. (1995): The institue of hydrology distributed model. V.P. Singh (ed.) Computer Models for Watershed Hydrology, Water Resources Publications: 221 – 229, 606 – 607.

Carsel, R., F. and Parrish, R,. S. (1988): Development of joint probability distributions of soil water retention characteristics. Water Resour. Res. 24 (5): 755 – 769.

Celia, M. A. and Bouloutas, E. T. (1990): A general mass-conservative numerical solution for the unsaturated flow equation. Water Resources Res., Vol. 26, 7, 1483 - 1496

Garbrecht, J. (1988): Determination of the execution sequence of channels in a drainage network for cascade routing. Hydrosoft: Software for Hydraulics, Hydrology and Hydrodynamics. Comutational Mechanics Pub..

Hornung, U. und Messing W. (1984): Poröse Medien - Methoden und Simulation. Verlag Beiträge zur Hydrologie, Kirchzarten

Jayawardena, A., W. and White, J., K. (1977): A finite element distributed catchment model, I Analytical basis. J. Hydrol. 34: 269-249.

Jayawardena, A., W. and white, J., K. (1977): A finite element distributed catchment model, II Application to real catchments. J. Hydrol. 42: 231-249.

Kiefer, E.-M., Liedl, R., Schmitz, G.H. und Seus, G.J. (1990): Konservative Strömungsmodelle auf der Basis krummliniger Koordinaten unter besonderer Berücksichtigung von Wasserbewegungen im ungesättigt-gesättigten Boden. Berichte der Versuchsanstalt Obernach und des Lehrstuhls für Wasserbau und Wassermengenwirtschaft der Technischen Universität München, Nr. 64

Kinzelbach, W. and Uffink, G. (1991): The random walk method and extensions in groundwater modeling. In: Transport Processes in Porous Media, Bear J. et al. (eds.), Kluwer (NATO ASI series E 202)

Kühnel, V., Dooghe, J., Sander, G., O’Kane, J. (1990a): Duration of atmosphere-controled and soli controlled phases of infiltration for constant rainfall at a soil surface. Ann. Geophys. 8(1): 11-20.

Kühnel, V., Dooghe, J., Sander, G., O’Kane, J. (1990a): Duration of atmosphere-controled and soli controlled phases of constant potential evaporation at a soil surface. Ann. Geophys. 8(1): 21-28.

Lehmann, W. (1995): Anwendung geostatistischer Verfahren auf die Bodenfeucht ein ländlichen Einzugsgebieten. Mitteilungen des IHW, Uinversität Karlsruhe, Heft. 52.

Martz, L. and Garbrecht, J. (1992): Numerical definition of drainage network and subcatchment areas from digital elevation models. Computers & Geosiences 18(6): 747-761.

Martz, L. and Garbrecht, J. (1993): DEDNM: A software for automated extraction of channel network and watershed data from raster digital elevation models. Geographic Information Systems and Water Resources: 211 –230.

Maurer, T. (1997): Physikalisch begründete, zeitkontinuierliche Modellierung des Wassertransports in kleinen ländlichen Einzugsgebieten. Universität Karlsruhe, Mitteilungen des Instituts für Hydrologie und Wasserwirtschaft, Heft 61, Dissertation

Mualem, Y. (1976): A new model for predicting the hydraulic conductivity of unsaturated porous media. Water Resources Res., 12, 513 -522

Rawls, W., J. and Brakensiek, D., L. (1989): Estimation of soil water retention and hydraulic properties. In H. J. Morel Seytaux (eds.): Unsaturated Flow in Hydraulic Modelling. NATO ASI series, Kluwer Academic Pub., Dordrecht, Boston, London.

Roth, K. and Hammel, K. (1996): Transport of a conservative chemical through an unsaturated two-dimensional miller-similar medium with steady state flow. Water Resources Res., Vol. 32, 6, 1653 - 1663

Singh, V., P. (eds.) (1995): Computer Models for Watershed Hydrology. Water Resources Publications.

Strahler, A., N. (1956): Hypsometric (area-altitude) analysis or erosional topography. Geological Soc. of America Bulletin 63: 1117 – 1142.

Streit., U. and Kleeberg (1996): GIS-based regionalisation in hydrology: German priority programme on spatial transfer of hydrological information. In Kovar, K. & Nachtnebel, H. (eds.), Application of Geographic Informationsystems in Hydrology and water Resources Management, Proc. HydroGIS 96, Conf., Vienna, Austria 16 –19 April 1996. IAHS Pub. No. 235, IAHS Press, Wallingford, UK, 485 –491.

Tang, Y. and Skaggs, R. (1977): experimental evaluation of theorectical solutions for subsurface drainage and irrigation. Water Resour. Res. 13(6): 957-965.

Tompson, A. F. B. (1987): Numerical Simulation of Solute Transport in Randomly Heterogeneous Porous Media: Motivation, Model Development and Application, Report No. UCID-21281, Lawrence Livermore National Laboratoy, Livermore California

van Genuchten, M. T. (1980): A closed-form equation for predicting the hydraulic conductivity of unsaturated soils. Soil Sci. Soc. Am. Jour., Vol. 44, 892 - 898

Vauclin, M., Khanji, D., Vachaud, G. (1979): Experimental and numerical study of a transient, two dimensional unsaturated-saturated water table recharge problem. Water Resour. Res. 15(5): 1089 – 1101.

Vertessy, R., Hatton, T., O’Shaughnessy, J., Jayayuriya, M. (1993): Predicting water yield from a mountain ash forest catchment using a terrain analysis based catchment model. J. Hydrol. 105: 665 – 700.

Wang, Q. and Dooge, J. (1993): Limiting  cases of water fluxes at the land surface. J. Hydrol. 155(3-4): 429-440.

Wiesmann, K, Kirchhoff, C., Streit, U. (1994): T4HIM: GIS-applikation für Hydrologie und Wasserwirtschaft. Angewandte Geographische Informationstechnologie AGIT 1994, Salzburg, Austria: 721-730.

Zehe, E. (1999): Stofftransport in der ungesättigten Bodenzone auf verschienden Skalen. Mitteilungen Inst. f. Hydrologie u. Wasserwirtschaft H. 64, Universität Karlsruhe.